3GPP TSG-RAN WG2 Meeting #121	                                     R2-2300411

Athens, Greece, Feb 27 – Mar 03, 2023

Agenda item:			8.2.3

Source:			Intel Corporation

Title:		Integrity for RAT dependent positioning methods

Document for:	 	Discussion and decision

# Introduction

During SI phaseRAN2 discussed the support of integrity for RAT dependent positioning methods, and provided recommendation from RAN2 perspective. At RAN#98, RAN plenary agreed the new WI on expanded and improved NR positioning [1] with following objective related:

- Specify the error modelling parameters, signalling, and procedures to support UE-based and LMF-based integrity of RAT-dependent positioning methods [RAN2, RAN3].

In this contribution, we continue the discussion on  how to specify these objectives.

# Discussion

During study phase, RAN1 agreed the identified error sources for LMF-based and UE-based positioning integrity modes for different positioning methods.

Table 6.1.1-1: Error sources for LMF-based and UE-based positioning integrity modes (TR 38.859)

| Positioning Integrity Mode | DL TDOA | UL TDOA | Multi-RTT | UL AoA | DL AoD |
| --- | --- | --- | --- | --- | --- |
| LMF-based (as defined in Table 9.4.1.1.1 in TR 38.857 [2]) | - RSTD measurement - TRP location - Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.) | - RTOA measurement - TRP location - Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.) | - UE Rx-Tx time difference measurement - gNB Rx-Tx time difference measurement - TRP location | - Angle of arrival measurement - TRP location - ARP location (e.g., ARPLocationInformation in TS 38.455 [17]) | - TRP location - DL-PRS RSRPP of the first path or RSRP |
| UE-based (as defined in Table 9.4.1.1.1 in TR 38.857 [2]) | - TRP location (e.g., NR-TRP-LocationInfo in TS 37.355 [16]) - Inter-TRP synchronization (e.g., NR-RTD-Info in TS 37.355 [16]) |  |  |  | - TRP location (e.g., NR-TRP-LocationInfo in TS 37.355 [16]) |

### 2.1 LMF-based integrity

For LMF-based integrity, RAN2 agreed the signalling impact as:

-	UE sends capability info to LMF for LMF-based positioning integrity mode using LPP capability transfer procedure

-	LMF sends the request of results related to integrity for integrity error sources to UE for integrity of LMF-based mode

-	LMF sends the request of results related to integrity for integrity error sources to RAN for integrity of LMF-based mode

-	RAN sends results related to integrity to LMF using NRPPa message.

NOTE 1: The signaling to transmit integrity KPI and integrity results can be discussed during normative work.

NOTE 2: Whether UE sends results related to integrity to LMF using LPP message or not can be discussed during normative work.

Based on the Error sources in the table, error sources for UE related measurements should be provided by UE and error sources for gNB related measurement and assistance data error should be provided by the gNB.

For DL TDOA, Multi-RTT:

Error sources for UE related measurements: “RSTD measurement”, “UE Rx-Tx time difference measurement” and “DL-PRS RSRPP of the first path or RSRP” should be provided by UE to the LMF via LPP Provide location information message if requested by the LMF via LPP Request Location information message;

For DL TDOA, UL TDOA, Multi-RTT, UL AoA, and DL AoD:

Error sources for assistance information: “TRP location”, “Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.)” and “ARP location (e.g., ARPLocationInformation in TS 38.455 [17])”should be provided by the gNB via NRPPa message if requested by the LMF via NRPPa message. (details should be decided in RAN3);

Error sources for gNB related measurements: “gNB Rx-Tx time difference measurement”,  “Angle of arrival measurement” should be provided by the gNB via NRPPa message if requested by the LMF via NRPPa message. (details should be decided in RAN3);

Proposal 1: RAN2 agree following categories of error sources for LMF based integrity:

Error sources for UE related measurements: “RSTD measurement”, “UE Rx-Tx time difference measurement” and “DL-PRS RSRPP of the first path or RSRP”;

Error sources for assistance information: “TRP location”, “Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.)” and “ARP location (e.g., ARPLocationInformation in TS 38.455 [17])”

Error sources for gNB related measurements: “gNB Rx-Tx time difference measurement”,  “Angle of arrival measurement”

Proposal 2: For LMF based integrity, Error sources related to the UE measurements should be provided by UE via LPP ProvideLocationInformation message if requested by the LMF via LPP RequestLocationInformation message;

Proposal 3: For LMF based integrity, Error sources related the gNB should be provided by gNB  via NRPPa message if requested by the LMF (details to be discussed in RAN3).

### 2.2 UE-based integrity

For UE-based integrity, RAN2 agreed the signalling impact as:

-	UE sends capability info to LMF on integrity for UE-based mode using LPP capability transfer procedure.

-	LMF sends the assistance data for integrity calculation to UE. LMF provides, in assistance data, the information of error sources (e.g., originated from RAN node) to UE for integrity in UE-based mode.

-	LMF sends integrity requirement e.g., TIR to UE in LPP request location information message for integrity of UE-based mode.

-	UE sends integrity result to LMF using LPP location information Transfer message.

Only assistance data related error sources (for DL AoD and DL TDOA) are applicable for UE based integrity. The LMF should obtain these error sources from the gNB via NRPPa message, and then forward them to a UE via LPP message.

Proposal 4: RAN2 agree following categories of error sources for UE based integrity:

Error sources for assistance information: “TRP location”, “Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.)” ”

Proposal 5: For UE based integrity,  the LMF obtains the error sources for assistance information from a gNB  via NRPPa message (details to be discussed in RAN3).

Proposal 6: For UE based integrity,  same as GNSS integrity, the LMF forwards the error sources for assistance information obtained from gNB to a UE LPP message ProvideAssistanceData.

The error sources for RAT dependent positioning were discussed in RAN1. However, RAN1 is not listed as responsible WG for the objective. We expect RAN1 to provide the details of error sources as part of RAN1 parameters. Therefore, RAN2 should send an LS to RAN1, and ask them to provide the details of error sources as part of RAN1 parameters.

Proposal 7: Send LS to RAN1, ask them to provide the details of error sources as part of RAN1 parameters.

# Conclusion

Based on the discussion, we have following proposals:

Proposal 1: RAN2 agree following categories of error sources for LMF based integrity:

Error sources for UE related measurements: “RSTD measurement”, “UE Rx-Tx time difference measurement” and “DL-PRS RSRPP of the first path or RSRP”;

Error sources for assistance information: “TRP location”, “Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.)” and “ARP location (e.g., ARPLocationInformation in TS 38.455 [17])”

Error sources for gNB related measurements: “gNB Rx-Tx time difference measurement”,  “Angle of arrival measurement”

Proposal 2: For LMF based integrity, Error sources related to the UE measurements should be provided by UE via LPP ProvideLocationInformation message if requested by the LMF via LPP RequestLocationInformation message;

Proposal 3: For LMF based integrity,  Error sources related the gNB should be provided by gNB  via NRPPa message if requested by the LMF (details to be discussed in RAN3).

Proposal 4: RAN2 agree following categories of error sources for UE based integrity:

Error sources for assistance information: “TRP location”, “Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.)” ”

Proposal 5: For UE based integrity, the LMF obtains the error sources for assistance information from a gNB via NRPPa message (details to be discussed in RAN3).

Proposal 6: For UE based integrity, same as GNSS integrity, the LMF forwards the error sources for assistance information obtained from gNB to a UE LPP message ProvideAssistanceData.

Proposal 7: Send LS to RAN1, ask them to provide the details of error sources as part of RAN1 parameters.

# Reference

[1] RP-223549 New WID on Expanded and Improved NR Positioning

[2] R2-2211131 LS on LPHAP information delivery to RAN (S2-2209591; contact: Huawei)	SA2