3GPP TSG-RAN WG2 Meeting #121	                                     R2-2300413

Athens, Greece, Feb 27 – Mar 03, 2023

Agenda item:			8.2.5

Source:			Intel Corporation

Title:		Considerations on  RAN1 led items

Document for:	 	Discussion and decision

# Introduction

At RAN#98, RAN plenary agreed the new WI on expanded and improved NR positioning [1] with following objectives led by RAN1:

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs. - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4]. - Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference). - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4]. - Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4]. - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

In this contribution, we continue the discussion on  how to specify these objectives.

# Discussion

### 2.1 Support of positioning for UEs with Reduced Capabilities (RedCap UEs)

The objective is

- Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2].

During SI phase, RAN1 identified following potential impact:

From RAN1's perspective, the following have been identified for potential specification impact to support NR positioning for RedCap UEs: - Maximum tolerable phase error, timing gap, and timing error between hops. - Considerations for IIoT, commercial, Public Safety and V2X scenarios, and UE capabilities. - Details on the Tx or Rx hopping pattern(s), including frequency overlapping between hops, if supported.

From RAN2 perspective, the main work should be to capture RAN1 parameters and capabilities which can be done once there is reasonable progress in RAN1.

Proposal 1: Potential RAN2 impact is to capture RAN1 parameters and feature lists. Postpone the discussion on positioning for RedCap UE until there is reasonable progress in RAN1.

### 2.2 Bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers

The objective is

- Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference).

As captured in the TR [2],

From the perspective of Radio Resource Management (RRM), the following are assumed for PRS bandwidth aggregation: - A common numerology is required across all intra-band contiguous PFLs to be aggregated. - PRS resources to be aggregated from different PFLs can have different bandwidths (i.e., different number of PRS RBs). - PRS resources to be aggregated from different PFLs are transmitted in the same slot and in the same symbols. - PRS resources to be aggregated from different PFLs are transmitted by the same TRP and associated with a common Antenna Reference Point (ARP). From RRM perspective, the following are assumed for SRS bandwidth aggregation: - A common numerology is required across all intra-band contiguous carriers to be aggregated. - SRS resources to be aggregated from different carriers can have different bandwidths (i.e., different number of SRS RBs). - SRS resources to be aggregated from different carriers are transmitted in the same slot and in the same symbols.

It is unclear whether the bandwidth aggregation is similar to CA, i.e. based 3 intra-band contiguous carriers (3 cells). That is:

- For PRS, 3 cells (intra-band contiguous carriers ) send the PRS in the aggregated bandwidth;

- For SRS, UE sends the SRS in 3 cells  (intra-band contiguous carriers ) at the same time.

If yes, it will have significant impact on RAN2 work, e.g. RAN2 needs to consider:

- Do we need to introduce new assistance data structure to support PRS/SRS in aggregated bandwidth?

- The relationship with CA, should CA be the pre-condition for aggregated bandwidth? If yes, how to trigger the UE in intra-band contiguous carriers CA case? Or the LMF only use this feature when UE has been configured with intra-band contiguous carriers CA?

Considering the bandwidth aggregation may have significant RAN2 impact, it would be good to ask RAN4, RAN1 to provide information as early as possible.

Proposal 2: Send LS to RAN1 and RAN4, ask them to clarify the relationship between aggregated bandwidth and CA.

### 2.3 Support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning

The objective is:

- Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL.

Based on the objective, if existing DL PRS and UL SRS are reused, RAN2 does not need to introduce new assistance information. However, Carrier phase measurement is a new positioning method, the impact to RAN2 should be to introduce a new positioning methods, similar to other UE based/assisted RAT dependent positioning methods, .e.g. DL TDOA.

However it is difficult for RAN2 to start the discussion. Therefore we propose:

Proposal 3: Potential RAN2 impact is to capture RAN1 parameters, feature lists and may introduce new RAT dependent positioning method. Postpone the discussion on Carrier phase positioning until there is reasonable progress in RAN1.

# Conclusion

Based on the discussion, we have following proposals:

Proposal 1: Potential RAN2 impact is to capture RAN1 parameters and feature lists. Postpone the discussion on positioning for RedCap UE until there is reasonable progress in RAN1.

Proposal 2: Send LS to RAN1 and RAN4, ask them to clarify the relationship between aggregated bandwidth and CA.

Proposal 3: Potential RAN2 impact is to capture RAN1 parameters, feature lists and may introduce new RAT dependent positioning method. Postpone the discussion on Carrier phase positioning until there is reasonable progress in RAN1.

# Reference

[1] RP-223549 New WID on Expanded and Improved NR Positioning

[2] TR 38.859