3GPP TSG RAN WG2 Meeting #121          	      	                	                R2-2300418

Athens, GR, 27th Feb – 3rd Mar, 2023

Agenda Item:	8.16.2

Source:	Intel Corporation

Title:	Analysis on data collection framework for AI/ML air interface

Document for:	Discussion and Decision

# Introduction

Data collection aspects were discussed in the email discussion [054][AIML18] Data Collection. As proposed in the email discussion, RAN2 could first work on analysis of existing data collection framework considering a) the content of the data, b) the data size, c) latency and periodicity, d) signalling and configuration aspects, etc.

In this contribution, we provide a comparison and analysis on below existing data collection frameworks:

      - MDT

      - UE assistance information

      - Early idle/inactive measurements

      - RRM measurement reports

      - CSI reporting framework

      - LPP Provide location information

# Discussion

## General requirement

### Data type

During email discussion, what type of data needs to be collected was discussed. A common understanding among companies is that RAN2 will consider data collection for all concerning LCM procedures. In this section, we will focus on what data needs to be collected to support three AI/ML air interface use cases considered by RAN1.

Input data for model training/inference

To support AI/ML basic processing (i.e. training/inference), collecting inputs that used by one use case for model training and inference purpose is necessary. However, the input data needs to be decided based on enough performance evaluation from RAN1, for example, what to collect, how frequent to collect, amount of data to be collected, latency, etc. Those requirements are highly use case and performance dependent.

Observation 1: Input data used for model training/inference are highly use case and performance dependent, which require further progress in RAN1.

Model Monitoring/Selection/Update

Following agreements were reached during RAN1 #110bis-e meeting:

Agreement Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case: - Monitoring based on inference accuracy, including metrics related to intermediate KPIs - Monitoring based on system performance, including metrics related to system peformance KPIs - Other monitoring solutions, at least following 2 options. - Monitoring based on data distribution - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc. - Output-based: e.g., drift detection of output data - Monitoring based on applicable condition Note: Model monitoring metric calculation may be done at NW or UE Agreement Study performance monitoring approaches, considering the following model monitoring KPIs as general guidance - Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance) - Overhead (e.g., signaling overhead associated with model monitoring) - Complexity (e.g., computation and memory cost for model monitoring) - Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring) - FFS: Power consumption - Other KPIs are not precluded. Note: Relevant KPIs may vary across different model monitoring approaches. FFS: Discussion of KPIs for other LCM procedures

Additionally, for beam management and positioning optimization use case, following agreements on model monitoring are agreed:

Agreement Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the necessity and the potential specification impacts from the following aspects: - UE reporting of beam measurement(s) based on a set of beams indicated by gNB - Signaling, e.g., RRC-based, L1-based - Note: Performance and UE complexity, power consumption should be considered Agreement - Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on feasibility, potential benefits (if any) and potential specification impact at least for the following aspects - At least the following are identified for further study as potential data for calculating monitoring metric - If monitoring based on model output - E.g. , estimated UE location corresponding to model output for direct AI/ML positioning, estimated intermediate parameter(s) corresponding to model output for AI/ML assisted positioning, ground truth label corresponding to model inference output for both direct and AI/ML assisted positioning - If monitoring based on model input - E.g., measurement corresponding to model inference input - Note1: other type of potential data for model monitoring is not precluded - Note2: combination of one or more type of potential data for monitoring is not precluded - If a given type of data is necessary for calculating monitoring metric, study whether and if so - How an entity can be used to provide the given type of data for calculating monitoring metric - Companies are requested to report their assumption of the entity (or entities) used to provide the given type of data for calculating monitoring metric for each case - Potential signalling for provisioning of the given type of data for calculating associated monitoring metric - Potential assistance signaling and procedure to facilitate an entity providing data for calculating monitoring metric - Potential UE-network interaction - E.g., model monitoring decision indication between UE and network

It can be observed that the model monitoring can be summarized in following two aspects:

1) monitoring wireless related system performance or measurement: it can be defined per use case, considering UE throughput, packet delay, etc. For example, system performance KPIs, beam measurement, UE location in above RAN1 agreement.

2) monitoring model performance feedback: this includes inference accuracy, model bias, model variance, confidence level, drift detection of input/output data, etc. This information is generated by model inference function. For example, inference accuracy, AI/ML signalling overhead, computation and memory cost, power consumption, ground truth label, etc in above RAN1 agreement.

Moreover, since the feedback information used for model monitoring also tight to performance evaluation of one AI/ML model, hence we believe the detailed metric of model monitoring (including data to be collected, frequency, latency, etc) shall also first be discussed in RAN1.

Proposal 1: Model performance feedback (e.g. inference accuracy, model bias, etc) and wireless related system performance/measurement are considered as input for model monitoring. Wait further progress in RAN1 on the detailed metrics (e.g. data to be collected, frequency, latency, etc).

## Data collection framework

### RRM measurement reports

As defined in TS38.331 [1], the network may configure a measConfig to the UE together with measurement object and reporting configuration.

Below, we summarized the key characteristics of RRM measurement report in order to assess whether it is suitable for data collection framework:

    - Termination: gNB

    - Measurement duration:

      - Periodical: the report periodicity varies from 120ms to 30min

      - Event-based: Event A1, A2, A3, A4, A5, A6, D1, X1, X2, I1

    - Measurement/Reporting interval: same as measurement/reporting interval

    - Current data type: radio performance measurement, measurement gap

    - Maximum data size to be reported: maximum UL RRC message (16 segments, 144kB)

The measurement targets are mainly related to the radio performance of one UE, e.g. reference signal strength (RSRP, RSRQ, SINR, etc), which are real-time information and mainly triggered by radio related event of a UE. Additionally, one of the advantages of RRM measurement report is that it is able to report multiple meausrment repot for multiple cells.

Observation 2: RRM measurement report is suitable to collect real-time data (e.g. radio performance measurement). Multiple measurement reporting is also supported.

### SON/MDT

As captured in TS37.320 [2], there are two types of MDT data collection, i.e. logged MDT and immediate MDT.

Logged MDT

For logged MDT, the network initiates the procedure when UE is in RRC_CONNECTED state by sending LoggedMeasurementConfiguration message. Below, we summarized the key characteristics of MDT data:

    - Termination: OAM

    - Measurement duration: the UE measures/collects data in every {10,20,40,60,90,120} minutes.

    - Measurement/Reporting interval:

      - Periodical: the report periodicity varies from 320ms to infinity.

      - Event-based:

        - Out of coverage: the UE to perform logging of measurements when the UE enters any cell selection state

        - eventL1: similar as eventA2 for the camping cell in camped normally state.

    - Current data type: Downlink pilot strength measurement, MBSFN measurement, WLAN measurement, Bluetooth measurement, etc

    - Maximum data size to be reported: maximum UL RRC message (16 segments, 144kB)

Based on above observation, the logged MDT supports to collect data which is not needed by the entity immediately. On the other hand, it can support to collect data for a longer period and larger data size.

Observation 3: Logged MDT data collection framework is suitable to collect non-urgent data. It can also support large data size.

Immediate MDT

Both RAN measurement and UE measurement can be configured, where UE measurement is based on existing RRM measurement procedure for configuration and reporting. Additionally, location information is supported as an extension.

Since the same mechanism is used between RRM measurement reporting and immediate MDT, except the termination is different, as immediate MDT is using tracing mechanism and terminates at OAM.

Besides, as captured in TR37.817 [3], during RAN3 AI/ML NG-RAN SI discussion, it was agreed that existing framework (including MDT and RRM measurements) are considered as baseline and reused for existing UE measurement.

If existing UE measurements are needed by a gNB for AI/ML-based network energy saving/load balancing/mobility optimization, RAN3 shall reuse the existing framework (including MDT and RRM measurements).

Observation 4: Immediate MDT supports similar measurement interval and data size as in RRM measurement report. The data collection type of immediate MDT is the same as logged MDT i.e. non-urgent data. RAN3 AI/ML NG-RAN WI also assumes existing RRM and MDT measurement is considered as baseline for existing UE measurements.

### UE assistance information

Below, we summarized the key characteristics of UE assistance information for delay budget, overheating information, etc:

    - Termination: gNB

    - Reporting interval: Based on configuration or upon change of problem information

      - Prohibit Timer-based in sec-level, e.g. delayBudgetReportingProhibitTimer, overheatingIndicationProhibitTimer, etc

Below we show some examples of the granularity of prohibit timer:

      - Change of problem information

    - Measurement duration: same as reporting interval

    - Current data type: delay budget, overheating assistance information, preference on configuration, relaxation state for RLM/BFD measurement, etc

    - Maximum data size to be reported: maximum UL RRC message (16 segments, 144kB)

Observation 5: UE assistance information is suitable to transmit UE preferred configuration, assistance information, delay performance of a UE, and can be configured to transmit in seconds-level.

### Others

CSI reporting framework and LPP Provide location information are also proposed as candidate framework for data collection in email discussion [Post120][054][AIML18] Data Collection (Ericsson / vivo). Different from above three frameworks, these two frameworks are very use case specific. If new AI/ML related data is introduced, the data characteristic should be the same as existing parameters/data collected in current framework. Key characteristics of the two are summarized as below:

CSI reporting framework

Below, we summarized the key characteristics of CSI reporting:

    - Termination: gNB

    - Reporting interval:

      - Periodic: slot-level

      - Aperiodic (semi-persistent): slot-level

    - Measurement duration: same as reporting interval

    - Current data type: CSI related reporting, e.g. RI, PMI, CQI, LI, etc

    - Maximum data size to be reported: based on current codebook configuration

LPP Provide location information

Following the key characteristics of LPP provide location information are summarized:

    - Termination: LMF

    - Reporting interval: based on request message

    - Measurement duration: per LPP Request Location Information message

    - Current data type: location information

    - Maximum data size to be reported: via ULInformationTransfer message, maximum UL RRC message (16 segments, 144kB)

Observation 6: CSI reporting framework and LPP Provide location information are use case specific, where the collected data characteristic for AI/ML enabled use case should follow the same criteria as the scenario where AI/ML is not used.

## Summary

|  | Termination | Suitable data type | Max reporting data size | Measurement duration | Report interval |
| --- | --- | --- | --- | --- | --- |
| Logged MDT | OAM | Non-urgent data, including location info | If segmentation is supported, 16 UL RRC segments, 144kB; otherwise 9kB | Min: 10min Max: 120min | Min: 320ms Max: infinity |
| Immediate MDT | OAM |  |  | Min: 120ms Max: 30min | Min: 120ms Max: 30min |
| RRM measurement reports | gNB | Real-time radio performance |  |  |  |
| UE assistance information | gNB | Assistance information to show UE preference |  | Sec-level Varies from 0s to 600s depends on the reported data | Configuration or upon change of problem information |
| LPP Provide location information | LMF | Location information | maximum UL RRC message (16 segments, 144kB) | immediate | Per request |
| CSI reporting framework | gNB | CSI reporting | based on current codebook configuration | Slot-level Min: 4 slot Max: 320 slot | Slot-level Min: 4 slot Max: 320 slot |

Proposal 2: RAN2 to capture above data collection framework summary in the TR.

# Conclusion

In this contribution, we compared and analyzed pros/cons of different data collection framework, then summarized the suitable data type for each one, with following observations and proposals:

Observation 1: Input data used for model training/inference are highly use case and performance dependent, which require further progress in RAN1.

Proposal 1: Model performance feedback (e.g. inference accuracy, model bias, etc) and wireless related system performance/measurement are considered as input for model monitoring. Wait further progress in RAN1 on the detailed metrics (e.g. data to be collected, frequency, latency, etc).

Observation 2: RRM measurement report is suitable to collect real-time data (e.g. radio performance measurement). Multiple measurement reporting is also supported.

Observation 3: Logged MDT data collection framework is suitable to collect non-urgent data. It can also support large data size.

Observation 4: Immediate MDT supports similar measurement interval and data size as in RRM measurement report. The data collection type of immediate MDT is the same as logged MDT i.e. non-urgent data. RAN3 AI/ML NG-RAN WI also assumes existing RRM and MDT measurement is considered as baseline for existing UE measurements.

Observation 5: UE assistance information is suitable to transmit UE preferred configuration, assistance information, delay performance of a UE, and can be configured to transmit in seconds-level.

Observation 6: CSI reporting framework and LPP Provide location information are use case specific, where the collected data characteristic for AI/ML enabled use case should follow the same criteria as the scenario where AI/ML is not used.

Proposal 2: RAN2 to capture above data collection framework summary in the TR.

|  | Termination | Suitable data type | Max reporting data size | Measurement duration | Report interval |
| --- | --- | --- | --- | --- | --- |
| Logged MDT | OAM | Non-urgent data, including location info | If segmentation is supported, 16 UL RRC segments, 144kB; otherwise 9kB | Min: 10min Max: 120min | Min: 320ms Max: infinity |
| Immediate MDT | OAM |  |  | Min: 120ms Max: 30min | Min: 120ms Max: 30min |
| RRM measurement reports | gNB | Real-time radio performance |  |  |  |
| UE assistance information | gNB | Assistance information to show UE preference |  | Sec-level Varies from 0s to 600s depends on the reported data | Configuration or upon change of problem information |
| LPP Provide location information | LMF | Location information | maximum UL RRC message (16 segments, 144kB) | immediate | Per request |
| CSI reporting framework | gNB | CSI reporting | based on current codebook configuration | Slot-level Min: 4 slot Max: 320 slot | Slot-level Min: 4 slot Max: 320 slot |

# References

- TS38.331, Radio Resource Control (RRC) protocol specification

- TS37.320, Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2

- TR37.817, Study on enhancement for Data Collection for NR and EN-DC