3GPP TSG RAN WG2 Meeting #121                                                         R2-2300430

# Athens, Greece, 27th February - 3rd March 2023

Agenda item:	8.5.2.3

Source:	Intel Corporation

Title:	Criteria and Mechanism of PDU Discard for XR traffic

Document for:	Discussion

# Introduction

In the last RAN2 #120 meeting, following agreements related to PDU discard were made.

- For uplink, handling of discard FFS. - RAN2 to support timer-based discarding of UL transmit side of PDCP PDU/SDUs of a PDU set. FFS how this is modelled in PDCP specification, can be discussed in WI phase.

This document discusses RAN impacts and procedure for packet discard considering RAN2 agreements, SA2 conclusions in TR 23.700-60 [1] and SA2 inputs in LS [2].

# Discussion

## PDU Discard Criteria

### PDU Discard for Partial PDU Set Delivery

In R18 XR, packet discard is performed on a PDU set basis. A straightforward requirement for such PDU set level discard in uplink is for the UE to be able to identify PDUs that belong to a PDU set. Such information could be dynamic, and considering the agreements and conclusions made in RAN2 and SA2 thus far, a UE could know of this PDU to PDU set mapping either by upper layer or by implementation.

UE transmitter can perform discard on a per PDU-set basis based on UE’s knowledge of PDU to PDU-set mapping, either from upper layers or by implementation.

SA2 has introduced the QoS parameter of PDU Set Integrated Handling Indication (PSIHI) which indicates whether all PDUs are needed for the usage of the PDU Set by the application layer in the receiver side. With the introduction of PSIHI in 5G, packets may be discarded based on this QoS parameter, such that if any PDU for a PDU set is lost/corrupted/delayed, for which is it indicated by the CN that all PDUs are needed for the usage of PDU Set by application layer, then remaining PDUs in that PDU set are considered unnecessary to be transmitted and can be discarded by UE transmitter. Here it is for the UE to determine/consider a packet as lost/corrupted/delayed for example, based on maximum latency requirement, or after it has exhausted HARQ retransmissions etc. This scenario is shown in Figure 1.

Figure 1: PDU Discard Based on Integrated Handling Requirement

In R18 XR, a PDU set is only successfully delivered when all PDUs of that set have been successfully received, i.e partial delivery of PDU set is not considered (as PSIHI is always considered enabled for R18).

The QoS parameter of PDU Set Integrated Handling Indication (PSIHI) can be used as a discard criterion by UE transmitter if any PDU for a PDU set is lost/corrupted/delayed.

- RAN2 confirms SA2 conclusions that partial delivery of PDU sets is not considered in R18 XR since a PDU set is only successfully delivered when all PDUs of that set have been successfully received (as PSIHI is always considered enabled for R18).

### PDU Discard Under Congestion

For the case of XR traffic, PDU sets may carry different kind of content (e.g. I/B/P frames, or slices/tiles within an I/B/P frame etc.). An XR application may require correct decoding of the I-frame to enable the decoding of sub-sequent P-frames, but it may be able to sustain loss of some P or B frames.  In this case, under certain conditions, e.g congestion, the transmitter could discard PDU sets identified as lower priority/importance, to consequently prioritize more important PDU sets e.g corresponding to an I-frame. Under user plane enhancements for XR, SA2 has introduced the parameter of PDU Set Importance [1]. This parameter identifies the importance of a PDU set within a QoS Flow. The understanding in SA2, as also explained in LS [2] is that RAN may use this parameter for PDU set level packet discarding in the presence of congestion. This scenario is shown in Figure 2.

RAN2 may use the PDU Set Importance parameter for PDU set level packet discarding in the presence of congestion as also concluded by SA2.

Figure 2: PDU Discard under Congestion

If the network is under congestion and is unable to allocate sufficient resources to the UE, the UE could prioritize high importance PDU sets. There could be two states of transmission here for the packets to be discarded. If the PDUs are discarded before attempting transmission, mechanism similar to the discardTimer based PDCP SDU discard could be adopted as baseline. However, if low importance data PDU set has already been submitted to lower layers, but are deprioritized to allow transmission of higher importance PDU sets, discard of such PDU sets could lead to SN gaps. This is discussed in more detail in the next section 2.2.

Data packets could be discarded at the transmitter side during the following stages: before any transmission is attempted, and after transmission attempt(s) have been made.

### PDU Discard due to Delay

SA2 has defined an optional parameter of PDU Set Delay Budget (PSDB) for downlink which is an upper bound for the duration between the reception time of the first PDU and the delivery time of last PDU of a PDU Set between the UE and UPF. It is assumed that RAN can use the existing PDB parameter if PSDB is not available. While the delay value between UE and UPF has been defined as PSDB, the need and definition for AN PSDB is still FFS in SA2 [4].

PDU Set Delay Budget (PSDB) is defined as an optional parameter; the need and definition for AN PSDB is still FFS in SA2

In the interest of time and progress, it is therefore more prudent for RAN to define maximum latency constraints for packet discard. This maximum latency constraint, max_delay, could be based on the PDB value or on the PSDB value if it is available. Since the gNB is aware of the PDB value, and PDU to PDU set mapping, the gNB may configure a max_delay value per PDU set (e.g delay from the arrival of the first PDU of the PDU set in the AS). This max_delay could be considered the maximum time duration before which the gNB must receive all packets in a PDU set considering all factors contributing towards end-to-end latency.  The UE can track the elapsed time duration and can discard packets if they are likely to not meet the maximum latency constraint of max_delay. This scenario is shown in Figure 3.

- RAN to define maximum latency constraints for packet discard. This constraint could be considered as the maximum time duration before which the gNB must receive all packets in a PDU set configured e.g from the arrival of the first PDU of the PDU set in the AS. How to set the value of this constraint is left up to network implementation.

Figure 3: PDU Discard due to delay

Similar to the case of discard under congestion, packets discarded due to delay could also be in two stages of transmission (Observation 4) as elaborated in the next section 2.2.

## Discard Mechanism

There exists a mechanism for PDCP discard in TS 38.323 using the discardTimer. When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity discards the PDCP SDUs along with the corresponding PDCP Data PDU. Different from legacy, however, in XR the decision to discard is made at a per PDU set level. Considering PDU set level discard, and using the existing PDCP discard mechanism as baseline, we consider two possible scenarios from implementation perspective

- Scenario 1: All PDUs of a PDU set arrive simultaneously at the transmitter

- Scenario 2: PDUs of a PDU set arrive sequentially at the transmitter (with some non-zero inter-arrival time between the first and last PDU of a PDU set)

From implementation perspective for packet discard, PDUs of a PDU set may arrive at the transmitter either (i) simultaneously or, (ii) sequentially (with some non-zero inter-arrival time between the first and last PDU of a PDU set).

To enable discard at PDU set level, a configurable parameter of discardTimerPerSet can be introduced such that PDUs of a PDU set are discarded at the expiration of discardTimerPerSet. It can also be assumed, as in Proposal 1, that all PDUs are required to be received (i.e PSIHI is enabled). For scenario 1 when all PDUs of a PDU set arrive simultaneously at the transmitter, the discardTimer for all PDUs of the PDU set is started at the same time, and hence expires at the same time since the packet delay budget (PDB) for all PDUs of a PDU set is the same (as PDB is defined per QoS flow, and PDUs within a PDU Set will have the same QoS requirement). In this case, the discardTimerPerSet value could be based on the PDB value of any PDU of the PDU set (or on the PSDB value if it is available) and is started concurrently with the discardTimer of any PDU of the PDU set, since all PDUs arrive simultaneously. Alternatively, in this case it may be prudent to use a single discardTimerPerSet for the PDU set; when this timer expires, all the remaining PDUs of the PDU set that have not been delivered can be discarded. If a single discardTimerPerSet is used, the legacy discardTimer could be set to infinity (i.e not used).

It may be possible that all PDUs of the PDU set may not arrive at the transmitting PDCP entity at the same time. Hence for the case when PDUs of a PDU set arrive sequentially at the transmitter, the discardTimer will therefore be started at different points in time for the different individual PDUs of the PDU set. If discardTimer for first arriving PDCP SDU of the PDU set expires but the maximum allowed latency threshold of the PDU set is not reached (e.g set based discard timer has not expired which could also be based on the PDB/PSDB value), the transmitter should not discard the first PDCP SDU in this case. For the case when PSDB is available, the maximum allowed latency in RAN could consider this value to set the value for the discardTimerPerSet timer. However, since PSDB is defined as an optional parameter, it may not always be received in RAN. In the approved CR for TS 23.502 [S2-2301472], it is added that “When PSDB is not available and/or maximum duration threshold is not met, NG-RAN may use the existing PDB parameter.” In this case when PSDB is not available, RAN2 can have two options

- To use the legacy discardTimer based on PDB of individual PDUs, where expiration of discardTimer leads to discard of complete PDU set if PSIHI is enabled.

- To use discardTimerPerSet which is started when the first PDU of the PDU set arrives at the transmitter. Then the PDU Set is discarded when the transmitter determines that it cannot deliver all PDUs of that PDU Set successfully over the radio within the PDB of the End PDU of the PDU Set.

If Proposal 2 is agreed, RAN defines a maximum latency constraint, which relies on the PSDB value when it is available, and on the PDB value otherwise.

Existing PDCP discard mechanism could be used as baseline.

- A new parameter of discardTimerPerSet is introduced to enable PDU set level discard for PDCP SDUs.

### Support of Discard at Lower Layers

From RAN2 perspective, data PDUs that are identified to be discarded, at the transmitter, could be in two stages of transmission. It is possible that the transmitter decides to discard certain PDUs (e.g unnecessary PDUs when some PDUs in the PDU set are lost), or certain PDU sets (e.g those which have low importance and are discarded in lieu of high importance PDU sets) before they are transmitted. It is also possible that the transmitter decides to discard certain PDUs after a transmission attempt has been made e.g when discard happens due to violating maximum latency constraint, even if part of the PDU set is already transmitted, the PDU set is considered to be lost/unnecessary as the latency requirement (based on PDB/PSDB) cannot be met.

For high volume of XR packets to be transmitted with large number of packets possibly in the transmitter’s buffer, it could be beneficial to discard packets even if they have already been submitted to lower layers (MAC layer) for transmission. An example would be, as explained in section 2.1.2, if under congestion certain PDU sets with low PDU Set Importance are deprioritized to allow transmission of higher importance PDU sets. Then such PDU sets may be required to be discarded based on some buffer queue time or upon breeching maximum allowed latency constraint as in section 2.1.3. The point to note here is that even for packets or RLC PDUs for which delivery has already been attempted by MAC, they could be discarded without attempting retransmission if e.g delay timer expires. This is different from current specification, where only those RLC PDUs/SDUs can be discarded which have not yet been submitted to the lower layers [TS 38.322].

For high volume of XR packets to be transmitted with large number of packets possibly in the transmitter’s buffer, it could be beneficial to discard packets even if they have already been submitted to lower layers (MAC layer) for transmission e.g in the case of surpassing latency constraint.

- Packet discard is allowed after packets are already submitted to lower layers (MAC layer).

### Impact on PDCP/RLC SNs (if any)

In the case when data PDUs are discarded before they have been transmitted to lower (MAC) layer or over the air interface, their associated L2 (PDCP/RLC) SNs could be reused for new packets. On the other hand, discarding a PDCP SDU which is already associated with a PDCP SN causes an SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity [TS 38.323]. In legacy, it is up to UE implementation how to minimize SN gap after SDU discard. However, for the case of XR, when the decision of discard is at a PDU set level (i.e multiple PDUs in the PDU set are discarded simultaneously), it is helpful to discuss in RAN2 whether/how this potential issue of PDCP/RLC SN gaps could be addressed e.g some feedback from the transmitter to the receiver (UE to gNB for UL or gNB to UE for DL) on the gap SNs to avoid reordering delays at the receiver side.

- If Proposal 4 is agreed, RAN2 to address PDCP/RLC SN gaps which may result from the discard of XR packets (e.g some feedback from the transmitter to the receiver on the gap SNs to avoid reordering delays at the receiver side).

## Impacts to BSR

The discard criteria in section 2.1 can be considered separately or in conjunction with one another, e.g discard related to the integrated handling of a PDU set may come in effect only after certain PDUs are considered lost/unnecessary due to expiration of their delay budget. The UE transmitter may hence decide to discard packets based on any or multiple discard criteria. A problem which may arise in the case of PDU discard is in relation to feedback and BSR from the UE side. In the event of autonomous packet discard by the UE, the network might have outdated BSR information. Given the usual high bit rate of XR traffic, and also expected large data volumes, it could be useful for the network to know of the reduction in UE’s buffered data volume when UL data is discarded from its buffers. This could be beneficial for resource allocation, such that if the network has not allocated UL grants yet, and it is informed of the reduced data volume, it can perform future scheduling and resource planning accordingly. Therefore, when data packets are discarded, a BSR can be triggered to report to the network of the reduction in data volume in the UL buffer for the case when packets are discarded at the transmitting entity i.e UE in uplink. This could potentially avoid the gNB unnecessarily allocating UL resources. This will need minor updates of current BSR e.g., to trigger a BSR when packet discard is performed. Further details are provided on this in our companion paper [3].

- UE can feedback to the gNB (e.g BSR) when XR packet discard is performed of data from UE’s buffers (i.e., data that was already reported to network in a previous BSR).

# Conclusion

The observations captured are the following:

Observation 1.	UE transmitter can perform discard on a per PDU-set basis based on UE’s knowledge of PDU to PDU-set mapping, either from upper layers or by implementation.

Observation 2.	The QoS parameter of PDU Set Integrated Handling Indication (PSIHI) can be used as a discard criterion by UE transmitter if any PDU for a PDU set is lost/corrupted/delayed.

Observation 3.	RAN2 may use the PDU Set Importance parameter for PDU set level packet discarding in the presence of congestion as also concluded by SA2.

Observation 4.	Data packets could be discarded at the transmitter side during the following stages: before any transmission is attempted, and after transmission attempt(s) have been made.

Observation 5.	PDU Set Delay Budget (PSDB) is defined as an optional parameter; the need and definition for AN PSDB is still FFS in SA2

Observation 6.	From implementation perspective for packet discard, PDUs of a PDU set may arrive at the transmitter either (i) simultaneously or, (ii) sequentially (with some non-zero inter-arrival time between the first and last PDU of a PDU set).

Observation 7.	Existing PDCP discard mechanism could be used as baseline.

Observation 8.	For high volume of XR packets to be transmitted with large number of packets possibly in the transmitter’s buffer, it could be beneficial to discard packets even if they have already been submitted to lower layers (MAC layer) for transmission e.g in the case of surpassing latency constraint.

The proposals captured are the following:

Proposal 1.	RAN2 confirms SA2 conclusions that partial delivery of PDU sets is not considered in R18 XR since a PDU set is only successfully delivered when all PDUs of that set have been successfully received (as PSIHI is always considered enabled for R18).

Proposal 2.	RAN to define maximum latency constraints for packet discard. This constraint could be considered as the maximum time duration before which the gNB must receive all packets in a PDU set configured e.g from the arrival of the first PDU of the PDU set in the AS. How to set the value of this constraint is left up to network implementation.

Proposal 3.	A new parameter of discardTimerPerSet is introduced to enable PDU set level discard for PDCP SDUs.

Proposal 4.	Packet discard is allowed after packets are already submitted to lower layers (MAC layer).

Proposal 5.	If Proposal 4 is agreed, RAN2 to address PDCP/RLC SN gaps which may result from the discard of XR packets (e.g some feedback from the transmitter to the receiver on the gap SNs to avoid reordering delays at the receiver side).

Proposal 6.	UE can feedback to the gNB (e.g BSR) when XR packet discard is performed of data from UE’s buffers (i.e., data that was already reported to network in a previous BSR).

# Reference

- TR 23.700-60, Study on XR (Extended Reality) and media services, v18.0.0, December 2022.

- S2-2301378, Reply LS on PDU Set Handling, From: SA2, To: RAN2, Cc: RAN3, SA4, LS, January 2023.

- R2-2300433, BSR Enhancements, Intel Corporation, February 2023

- S2-2301472, Support of PDU Set based handling, RAN2 Approved CR for TS 23.501, January 2023.