3GPP TSG RAN WG2 Meeting #121                                                         R2-2300434

# Athens, Greece, 27th February - 3rd March 2023

Agenda item:	8.18.2

Source:	Intel Corporation

Title:	MT-SDT mechanism (including RB, paging, resume and capabilities)

Document for:	Discussion and decision

# Introduction

This document continues the discussion on open topics to enable MT-SDT operation considering the agreements taken in last RAN2#120 meeting.

# Discussion

## RBs configured for MT-SDT

The following agreements confirms the understanding that MT-SDT feature should be able to be configured by itself and leaves FFS how the corresponding configuration is defined as shown below:

- “MT-SDT is data that belongs to bearers that are configured for SDT. The network can only trigger MT-SDT if the data belongs to those bearers. FFS whether the configuration is MO-SDT or MT-SDT specific.”

- “It is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT.  Subsequent UL/DL data belonging to SDT bearers while in INACTIVE is allowed like MO-SDT procedure.”

During the RAN2#120 online discussion, companies suggested that it might be preferable to have an MT-SDT specific configuration. This section analyzes the possible options and corresponding implications. This discussion evaluates the impact of how would the RBs configured for SDT (SDT RB) work with the understanding that the corresponding SDT configuration could easily be extrapolated later.

- All SDT RBs. When UE resumes for SDT, it would resume any SDT configuration available in UE. This approach A) does not mandate that network has to always configured both MO-SDT and MT-SDT. But when network does it, UE would always resume RBs associated with both MO-SDT and MT-SDT regardless of whether the resume was initiated due to MT-SDT or MO-SDT.

- Specific MO-SDT and MT-SDT RBs. This approach differentiates whether the resume is initiated due to MO-SDT or MT-SDT as it would only resume those specific RBs. If this approach B) were used, new transitions and new intermediate state might need to be discussed as summarized in below figure.

Figure 1. UE states and corresponding RRC transitions for the two approaches on how to handle the resume of RB/config when initiating resume for SDT considering MO-SDT and MT-SDT

In conclusion, the approaches B) would introduce additional complexity due to the new temporal states, new kind of sessions and new fallback transitions as shown in below. This complexity does not seem justifiable to only support MT-SDT standalone. Therefore. we suggest to first confirm the desirable behaviour and later discussed the handling of the configuration in alignment to approach A) above.

- When UE resumes for SDT, UE resumes all RBs configured for SDT (independently on whether the RBs are configured for MO-SDT and/or MT-SDT, as well as whether the resume is initiated for MO-SDT or MT-SDT).

Even though it is kind of implicit from RAN2#120 agreements, it would be good to confirm the understanding that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL data (even if this RB is not configured for MO-SDT) as shown in below figure.

Figure 2. Resume for MT-SDT triggering the resume of all RBs configured for SDT (as per approach A))

- To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs (even if the RB is configured for MO-SDT). This clarifies the RB behaviour of related RAN2#120 agreement.

## Paging for MT-SDT

The following agreements was taken on paging: “For RAN paging, MT-SDT indication (at least one bit) is explicitly included per UE via a paging message. FFS if more information for MT-SDT are needed. FFS what the indication will be called.  FFS signalling details“.

Firstly, it would be clarified that this explicit MT-SDT indication sent per UE is included in the paging record when MT data is detected in SDT RBs or when anchor gNB informs of this indication in the Xn AP paging message.

- MT-SDT indication per UE is included as part of the paging record. Inform RAN3 and ask to enable the corresponding update to the Xn AP signaling.

Secondly, RAN2 should discuss which may be this other information that might be good to also include in the paging message. The following list summarizes proposals brought to last RAN2#120 meeting:

- Indication on whether CG-SDT resources can be used when responding to MT-SDT. This kind of information may be indeed helpful, but we do not understand why it needs to be carried on paging which would increase the system load unnecessarily. It seems sufficient to add a related indication on RRCRelease msg. at most. Note that the usage of RA-SDT resources does not seem optimum unless there is UL SDT data to be exchanged.

- Indication on the subsequent UL grant e.g. dedicated preamble. It does not seem optimum as it is always challenging to know whether a UE is still in one cell when triggering paging message. Therefore, it might be an unnecessary usage of specific resources beforehand.

- If a UE is configured with MT-SDT and CG-SDT, the UE can initiate resume due to MT-SDT indication in one of its CG-SDT resource. To discuss whether there is need to define a way for network to indicate when this operation is allowed (e.g. a flag in RRCRelease msg).

## Resume procedure for MT-SDT

The following agreements was taken on resume for MT-SDT:

- “UE can use non-SDT random access resources for accessing the network for an MT-SDT transfer.  The UE can also use the configured grant resources and/or MO-RA resources.

- Network should be able to differentiate why the UL access was triggered, i.e. implicit or explicit indication by the UE. New Resume cause in RRC resume will be introduced, one code point MT-SDT indication

- Rel-18 MT-SDT after the MT-SDT paging trigger is detected, RA-SDT and CG SDT solutions/procedures specified in Rel-17 is re-used as a baseline.  The detailed triggers will be discussed on case by case.  FFS on resources used for access”

The impact to the resume procedure when initiated due to MT-SDT is very much tied with the discussion in previous section 2.1. If the behaviour is as explained in Proposal 1, the impact to the resume would be minimal. I.e., mainly to define the new resume cause value to be included when resume is initiated due to MT-SDT and the initiation section for SDT.

If UE resumes any RB configured for SDT when initiating resume due to MT-SDT indication, the impact to the resume procedure is minimal, i.e. the new resume cause and to the initiation procedure for SDT.

- Update the resume procedure, to include MT-SDT resume cause when MT-SDT indication is received in Paging message for the given UE.

The conditions for initiation of SDT (section 5.3.13.1b of TS 38.331) includes the following checks:

- the upper layers request resumption of RRC connection; and

- SIB1 includes sdt-ConfigCommon; and

- sdt-Config is configured; and

- all the pending data in UL is mapped to the radio bearers configured for SDT; and

- lower layers indicate that conditions for initiating SDT as specified in TS 38.321 are fulfilled.

The (1) would need to be updated as the resume is triggered by AS when responding to paging during RRC_INACTIVE. For (2) and (3), it might be best to wait until the stage-3 solution is more developed. The (4) would not need to be met when responding to MT-SDT indication. For (5), it would make sense to at least check that RSRP threshold to perform SDT is met, however same threshold as for Rel-17 SDT could be reused.

- Update the initiation section of resume to also consider when AS responds to a MT-SDT indication. The following conditions would apply:

  - (1) AS layer triggers resumption of an RRC connection due to MT-SDT indication.

  - (2) Cell allows SDT, e.g., sdt-ConfigCommon is in SIB1. FFS on changes needed, if any.

  - (3) UE is configured to use MT-SDT, e.g., sdt-Config is configured. FFS on changes needed, if any.

  - (4) There is no need to check whether there is any UL data in UE’s buffer.

  - (5) Lower layers indicate that conditions for initiating MT-SDT are met. FFS on other changes needed, if any.

    - RSRP threshold defined for Rel-17 SDT is equally applicable to Rel-18 MT-SDT. If this SDT RSRP threshold is not met, UE initiates legacy resume as it was triggered by legacy RAN paging.

## UE radio capabilities for MT-SDT

A new radio capability per UE will be defined to identify UEs supporting MT-SDT, however this new feature could be defined in different ways:

- MT-SDT capability is defined as independent/standalone feature to be used with any RB

- MT-SDT capability is defined as independent/standalone feature to be used with any DRB and another MT-SDT-SRB capability is defined when MT SDT is to be sent over SRB.

- MT-SDT capability is defined to be dependent to the support and/or configuration of Rel-17 SDT features, i.e. RA-SDT, CG-SDT and/or SRB-SDT

It could be claimed that RAN2 agreed that network should be able to configure MT-SDT by independent to MO-SDT (as shown in related agreement in previous section 2.1) and therefore option 3) is not excluded. However, current agreement is on how network configures the usage of a feature, i.e. it does not need to mean that the support of this new feature should be defined as standalone. Therefore, we suggest RAN2 to discuss the three options.

- A new optional radio capability signaling is defined to indicate UE’s support of Rel-18 MT-SDT.

  - To discuss whether this MT-SDT radio capability is defined differently for DRB vs SRB.

  - To discuss whether this MT-SDT radio capability is defined conditionally dependent on support of Rel-17 SDT features (RA-SDT, CG-SDT and/or SRB-SDT).

# Conclusion

The observations captured are the following:

Observation 1.	If UE resumes any RB configured for SDT when initiating resume due to MT-SDT indication, the impact to the resume procedure is minimal, i.e. the new resume cause and to the initiation procedure for SDT.

The proposals captured are the following:

Proposal 1.	When UE resumes for SDT, UE resumes all RBs configured for SDT (independently on whether the RBs are configured for MO-SDT and/or MT-SDT, as well as whether the resume is initiated for MO-SDT or MT-SDT).

Proposal 2.	To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs (even if the RB is configured for MO-SDT). This clarifies the RB behaviour of related RAN2#120 agreement.

Proposal 3.	MT-SDT indication per UE is included as part of the paging record. Inform RAN3 and ask to enable the corresponding update to the Xn AP signaling.

Proposal 4.	If a UE is configured with MT-SDT and CG-SDT, the UE can initiate resume due to MT-SDT indication in one of its CG-SDT resource. To discuss whether there is need to define a way for network to indicate when this operation is allowed (e.g. a flag in RRCRelease msg).

Proposal 5.	Update the resume procedure, to include MT-SDT resume cause when MT-SDT indication is received in Paging message for the given UE.

Proposal 6.	Update the initiation section of resume to also consider when AS responds to a MT-SDT indication. The following conditions would apply:

Proposal 6.1.	(1) AS layer triggers resumption of an RRC connection due to MT-SDT indication.

Proposal 6.2.	(2) Cell allows SDT, e.g., sdt-ConfigCommon is in SIB1. FFS on changes needed, if any.

Proposal 6.3.	(3) UE is configured to use MT-SDT, e.g., sdt-Config is configured. FFS on changes needed, if any.

Proposal 6.4.	(4) There is no need to check whether there is any UL data in UE’s buffer.

Proposal 6.5.	(5) Lower layers indicate that conditions for initiating MT-SDT are met. FFS on other changes needed, if any.

Proposal 6.5.1.	RSRP threshold defined for Rel-17 SDT is equally applicable to Rel-18 MT-SDT. If this SDT RSRP threshold is not met, UE initiates legacy resume as it was triggered by legacy RAN paging.

Proposal 7.	A new optional radio capability signaling is defined to indicate UE’s support of Rel-18 MT-SDT.

Proposal 7.1.	To discuss whether this MT-SDT radio capability is defined differently for DRB vs SRB.

Proposal 7.2.	To discuss whether this MT-SDT radio capability is defined conditionally dependent on support of Rel-17 SDT features (RA-SDT, CG-SDT and/or SRB-SDT).