3GPP TSG-RAN WG2 Meeting #121	R2-2300435

Athens, Greece, 27 Feb – 2 Mar 2023

Agenda item:	8.17.2

Source:	Intel Corporation

Title:	Scenarios and requirements for the capability restriction request for Rel-18 MUSIM

WID/SID:	NR_DualTxRx_MUSIM-Core

Document for:	Discussion and Decision

# Introduction

In the last RAN2 meeting, the basic scenario for initiating the capability restriction request was discussed with the following agreement based on [1]:

- RAN2 aims to address at least the Scenario 1: the UE in network A in RRC_CONNECTED indicates (i.e. adds/removes) its preference on temporary UE capability due start/stop connection in NW B. This can be e.g. CA/DC capability restriction.

Several assumptions were also discussed, and the following are agreed:

- 2	The following is assumed when defining the solution:

- The two networks are independent (i.e. no inter-network communication);

- The Core Network is not aware of the temporary restrictions of the UE capability;

On the triggering, RAN2 only make the following agreement:

- A7: The UE can initiate signaling for UE capability restrictions on NW A if NW A allows it. The specification will not capture NW B events which can cause such need.

In this contribution, the additional scenarios are discussed and are then used to analyse the actions that are need by the UE in network A. The interactions between Rel-17 and Rel-18 MUSIM are also discussed.

# Discussion

## Additional scenarios to consider for Rel-18 MUSIM

In the last RAN2 meeting, RAN2 agreed to address at least Scenario 1: the UE in network A (NW A) in RRC_CONNECTED indicates (i.e. adds/removes) its preference on temporary UE capability due to start/stop connection in NW B.  Once UE is in RRC_CONNECTED in both network A and B, capability restriction may still be triggered by the UE over NW A. This scenario should also be further analysed to see whether further actions will be performed by UE over network A. In addition, we also think that the scenario where not both network support Rel-18 MUSIM needs to be discussed as well.

Additional Scenarios: RAN2 should consider the following additional scenarios for analysing the changes needed by Rel-18 MUSIM on top of Scenario 1:

- Scenario 2: UE in NW A and NW B in RRC_Connected indicates its preference on temporary UE capability due to reconfiguration in NW B

- Scenario 3: Only one network supports Rel-18 MUSIM

In the subsequent section, we analysed the possible actions required for Rel-18 MUSIM based on the Scenario 1 and the additional scenarios in Section 2.

## Initiation of capability restriction request

The typical initiation of the capability restriction request [2] is to indicate to network A when the resource that is needed for the connection in network B (NW B) is in use in network A (NW A). The following shows such typical initiation based on the scenarios in Section 2.1

Scenario 1: UE in network A in RRC_CONNECTED indicates (i.e. adds/removes) its preference on temporary UE capability due to start/stop connection in NW B

The table below shows the further sub-scenarios that may occur and the reactive UE actions in NW A:

| Sub-scenario | NW A | NW B | Reactive UE actions in NW A | Comments |
| --- | --- | --- | --- | --- |
| 1A | Connected, with SCell/SCG (NW A supports MUSIM) | Initiating connection (either RRC Setup or Resume), PCell in NW B is incompatible with NW A SCell/SCG | Request release of the SCell/SCG in NW A before initiating connection to NW B | Delay in establishing connection in NW B (See sub-scenario 2A and 2B) |
| 1B | Connected, with only PCell | Initiating connection, PCell is incompatible with NW A PCell | Request restriction to PCell in NW A (NW A has to HO UE) before initiating a connection to NW B | Delay in establishing connection in NW B. Another frequency may not always be available in NW A to HO the UE. |
| 1C | Connected | Stopping connection | If UE capability restriction has been sent by UE (e.g. sub-scenario 1A), the UE informs NW A to release the restriction. | No issue |

Scenario 2: UE in NW A and NW B in RRC_Connected indicates its preference on temporary UE capability due to reconfiguration in NW B

The table below shows the further sub-scenarios that may occur and the reactive actions in NW A:

| Sub-scenario | NW A | NW B | Reactive UE actions in NW A | Comments |
| --- | --- | --- | --- | --- |
| 2A | Connected with SCell/SCG | Receives reconfiguration that is not compatible with configuration in NW A (e.g. CA or SCG) | Request release of the configuration in NW A before initiating accepting the reconfiguration in NW B | Delay in reconfiguration connection in NW B (it can also be NW A) Alternatively reject the Reconfiguration message in NW B |

From the above, the following observations can be derived for the reactive approach:

Observation#1: Reactive approach may result in delay in establishing/resuming connection to network A as in sub-scenarios 1A and 1B.

Observation#2: Reactive approach may result in delay in reconfiguration as in sub-scenario 2A if it needs to inform capability restriction to the other network

### Scenarios where only one network supports MUSIM

This scenario when the UE is initiating connection in NW B, and UE is connected to NW A that does not support MUSIM.  Here, the capability restriction has to be sent to NW B (i.e., the network where the UE is initiating the connection). The table below shows the further sub-scenarios that may occur and the reactive UE actions in NW A.

Scenario 3: Only one network supports Rel-18 MUSIM

| Sub-scenario | NW A | NW B | Reactive UE actions in NW A | Comments |
| --- | --- | --- | --- | --- |
| 3A | Resuming with stored configuration not compatible with configuration in NW B (e.g. CA or SCG) | Connected (Rel-18 MUSIM is not supported by NW B) |  | This cannot be solved with UE sending the reactive capability restriction request. A new solution is needed as in Section 2.2.3. |
| 3B | Connected; UE receives configuration that is not compatible with UE capability based on configuration in NW B | Connected; (Rel-18 MUSIM is not supported by NW B) | UE can only reject the configuration from NW A | Reactive UE actions are not sufficient. |

Observation#3: For sub-scenario 3A, UE may be configured in network A during connection resumption with configuration (e.g. SCell and/or SCG) that is incompatible to its configuration with network B.

Observation#4: For sub-scenario 3B, UE can only reject the configuration from NW A as the configuration cannot be restricted in NW B.

In the following sections, we discuss how the above shortcoming of the reactive can be resolved.

### Mitigating the delay or rejection in reconfiguration or when Reactive solutions are not sufficient

The reconfiguration/resume delay in sub-scenarios 1A in Observation#1, 2A in Observation#2 and 3B in Observation#4 is because one network does not know that UE has some capability restrictions due to some resources being in use in the other network. If the capability restriction due to resource usage from one network is proactively provided by the UE to the other network(s), reconfiguration delay or rejection can be avoided.

| Sub-scenarios | NW A | NW B | UE actions in NW A | Comments |
| --- | --- | --- | --- | --- |
| 1A | Connected, with SCell/SCG (NW A supports MUSIM) | Initiating connection (either RRC Setup or Resume), PCell in NW B is incompatible with NW A SCell/SCG | Proactively provide capability restriction to NW A related to the PCell incompatibility to NW A SCell/SCG | Avoids NW A from configuring SCell/SCG incompatible to the PCell in NW B. |
| 2A | Connected with SCell/SCG | Connected and receives reconfiguration that may cause incompatibility with future configurations in NW A (e.g. CA or SCG) | Proactively provide capability restriction to NW A | Avoids delay for a subsequent configuration in NW A that UE cannot accept without signalling to NW B |
| 3B | Connected | Connected; (Rel-18 MUSIM is not supported by NW B); current configuration may cause incompatibility with future configurations in NW A | Proactively provide capability restriction to NW A | Avoids the need for reject procedure when receiving a subsequent configuration in NW A that UE cannot accept |

Observation#5: If the capability restriction due to resource usage from one network is proactively provided by the UE to the other network(s), reconfiguration delay or rejection can be avoided.

Proposal#1: UE is allowed to proactively provide capability restriction request to the other network(s).

However, one main issue with proactive approach is that it may result in unnecessary signalling overhead to NW A, particularly if this capability restrictions indicated by UE are not going to be configured (now or in the future) by NW A. This can be solved partially if NW A can optionally indicates the bands of concern (e.g. bands or CCs that it may configure) to the UE explicitly or implicitly (e.g. the neighbour cell list in the SIB, measurement objects). If and only if UE needs to restrict these bands in network A due to activity in NW B, UE will proactively provide the UAI or other form of UE feedback to inform network A about the restriction. For other bands (I.e., the bands not included in the “bands of concern” by network A), UE does not provide the UAI or other form of UE feedback. This will limit the amount of signalling to network A as the restrictions on bands not of concern are not signalled.

Observation#6: One main issue with UE proactively provides the UE assistance to NW A is that it may result in unnecessary signalling overhead to NW A, particularly if this capability restrictions indicated by UE are not going to be configured (now or in the future) by NW A.

Proposal#2: RAN2 to discuss how to reduce the unnecessary signalling overhead due to possible proactive sending capability restriction request/indication

### Avoid incompatible configuration during resumption

As in sub-scenario 3A in Observation#3, UE may have a stored configuration (CA/DC) in INACTIVE  in network A that is incompatible to its configuration with network B. Under normal scenario, the network A will resume the UE with CA/DC in the Resume message itself before the UE can provide any capability restriction.  Hence network A needs to be informed that there is capability restriction to avoid the network A configuring resources incompatible to the configuration in network B during the resumption. Given the size of ResumeRequest message, UE can only provide a single bit indication of the restriction.  That is, UE should indicate release/incapability of e.g. CA/DC during connection/resume to the network so that the network knows that there is possible restriction on the UE capabilities.  The network can release the CA/DC configuration in Resume message and should wait until the UE provides the UE cap restriction before reconfiguring the UE further, e.g. for higher throughput via CA or SCG, MIMO Layers, bandwidth etc.  Such indication can be included in the Resume Request message or Resume Complete message.  The table summarises the solution:

| Sub-scenario | NW A | NW B | New UE actions in NW A | Comments |
| --- | --- | --- | --- | --- |
| 3A | Resuming with stored configuration not compatible with configuration in NW B (e.g. CA or SCG) | Connected (Rel-18 MUSIM is not supported by NW B) | Inform NW A in ResumeRequest of the restriction (1 bit indication) such that network can release the CA/SCG in Resume message. Once CONNECTED, UE should send capability restriction request. | This two-step indication is needed as otherwise, the NW A will resume the UE with CA/DC that cannot be supported by UE |

Observation#7: During resumption (ResumeRequest), network A needs to be informed of possible capability restriction to avoid the network A configuring resources in Resume message incompatible to the configuration in network B.

Proposal#3: UE should indicate release/incapability of CA/DC during connection resume to the network so that the network knows that there is possible restriction on the UE capabilities and should release CA/DC and wait until the UE provides the UE capability restriction before reconfiguring the UE further with higher capability configuration (e.g. CA, SCG and/or MIMO layers, larger BW etc.).

## Rel-17 and Rel-18 MUSIM interactions

In Rel-17, the MUSIM UE operation between NW A and B can request gaps for operation in NW B or if a longer period is needed in NW B, the UE can request to enter Idle mode in NW A. For the gap request, the UE can request for periodic gaps or aperiodic gaps from NW A to satisfy the different use cases in NW B and UE can also request for multiple gaps from NW A to meet multiple use cases in NW B. Such Rel-17 MUSIM operation is used for the case where the MUSIM UE is single TX/RX.  The specifications do not explicitly support simultaneous RRC connection in Network A and Network B (though not precluded by specific implementations).

Rel-18 MUSIM assumes an UE RF architecture where dual TX/RX can be performed, allowing simultaneous RRC connection in both networks and there is no need for gaps at least for some of the bands/frequencies used by NW A and NW B. However, for some bands/frequencies used by NW A and B, dual TX/RX may not be possible and thus fall back to Rel-17 MUSIM operation may still be useful in these cases.

Depending on the UE state in network A and network B, it could be more optimal to use Rel-17 or Rel-18 MUSIM feature.  The following table is an example of the possible solutions a Rel-18 and Rel-17 capable MUSIM UE can use for the different UE states when UE is dual TX/RX capable:

| Scenarios | UE state in Network A | UE state in Network B | MUSIM Feature |
| --- | --- | --- | --- |
| A | Connected with CA | Idle/INACTIVE | Rel-17 MUSIM gap |
| B | Connected without CA | Idle/INACTIVE | Neither Rel-17 or Rel-18 MUSIM needed |
| C | Connected (Dual Tx/RX possible) | Connected (Dual Tx/RX possible) | Rel-18 MUSIM |
| D | Connected (Dual Tx/RX not possible due to RF restrictions in the particular band in NW A and B) | Connected (Dual Tx/RX not possible due to RF restrictions in the particular band in NW A and B) | Request release to IDLE using Rel-17 MUSIM |

Table 1: Possible solutions that a Rel-18 and Rel-17 capable MUSIM UE can use for the different UE states

From the above table, it can be seen that a UE capable of Rel-17 MUSIM and Rel-18 MUSIM feature can use different options depending on the UE state and bands in use.  Hence it should be possible to configure both Rel-17 MUSIM and Rel-18 MUSIM simultaneously for a UE and use the most appropriate solution depending on the scenario.

Observation#8: Supporting and configuring Rel-17 and Rel-18 MUSIM features simultaneously for a UE can lead to more optimal performance by using the most appropriate solution depending on the scenario and the UE state.

Proposal#4: It should be possible to configure both Rel-17 and Rel-18 MUSIM features (if supported) simultaneously for a UE.

On the other hand, it is possible that a UE may support just Rel-18 MUSIM feature.  This may not cover all the scenarios optimally (as discussed in the table above e.g. there is no solution for Scenario A if Rel-17 MUSIM gap is not supported).   Since Rel-18 MUSIM operation is yet to be discussed, this question of whether UE supporting Rel-18 MUSIM also needs to support of Rel-17 MUSIM UE capabilities can be postponed till the Rel-18 MUSIM operation is clear.

Proposal#5: Postpone the discussion on whether UE supporting Rel-18 MUSIM also needs to support of Rel-17 MUSIM UE capabilities till Rel-18 MUSIM operation is defined.

# Conclusion

It is requested that RAN2 agree to the observations and proposals below:

Additional Scenarios: RAN2 should consider the following additional scenarios for analysing the changes needed by Rel-18 MUSIM on top of Scenario 1:

- Scenario 2: UE in NW A and NW B in RRC_Connected indicates its preference on temporary UE capability due to reconfiguration in NW B

- Scenario 3: Only one network supports Rel-18 MUSIM

Observation#1: Reactive approach may result in delay in establishing/resuming connection to network A as in sub-scenarios 1A and 1B.

Observation#2: Reactive approach may result in delay in reconfiguration as in sub-scenario 2A if it needs to inform capability restriction to the other network

Observation#3: For sub-scenario 3A, UE may be configured in network A during connection resumption with configuration (e.g. SCell and/or SCG) that is incompatible to its configuration with network B.

Observation#4: For sub-scenario 3B, UE can only reject the configuration from NW A as the configuration cannot be restricted in NW B.

Observation#5: If the capability restriction due to resource usage from one network is proactively provided by the UE to the other network(s), reconfiguration delay or rejection can be avoided.

Proposal#1: UE is allowed to proactively provide capability restriction request to the other network(s).

Observation#6: One main issue with UE proactively provides the UE assistance to NW A is that it may result in unnecessary signalling overhead to NW A, particularly if this capability restrictions indicated by UE are not going to be configured (now or in the future) by NW A.

Proposal#2: RAN2 to discuss how to reduce the unnecessary signalling overhead due to possible proactive sending capability restriction request/indication

Observation#7: During resumption (ResumeRequest), network A needs to be informed of possible capability restriction to avoid the network A configuring resources in Resume message incompatible to the configuration in network B.

Proposal#3: UE should indicate release/incapability of CA/DC during connection resume to the network so that the network knows that there is possible restriction on the UE capabilities and should release CA/DC and wait until the UE provides the UE capability restriction before reconfiguring the UE further with higher capability configuration (e.g. CA, SCG and/or MIMO layers, larger BW etc.).

Observation#8: Supporting and configuring Rel-17 and Rel-18 MUSIM features simultaneously for a UE can lead to more optimal performance by using the most appropriate solution depending on the scenario and the UE state.

Proposal#4: It should be possible to configure both Rel-17 and Rel-18 MUSIM features (if supported) simultaneously for a UE.

Proposal#5: Postpone the discussion on whether UE supporting Rel-18 MUSIM also needs to support of Rel-17 MUSIM UE capabilities till Rel-18 MUSIM operation is defined.

# References

[1] RP-213549 New WID on Dual Tx/Rx MUSIM, Vivo

[2] R2-230436 Signalling to indicate temporary capability reduction for Rel-18 MUSIM, Intel Corporation