3GPP TSG-RAN WG2 Meeting #121		R2-2300445

AThens, Greece, Feb. 27th- Mar. 3rd, 2023

Agenda Item:	8.3.4

Source:	vivo

Title:	RAN2 impact from inter-band SSB-less SCell operation

Document for:	Discussion and Decision

Introduction

According to RAN#98 discussion, one objective of the R18 network energy saving WID[1] is the following:

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2]

In this contribution, we provide our consideration on the technique of SSB-less SCell operation for inter-band CA  for network energy saving.

Discussion

For SSB-less SCell operation, it can be considered an SCell does not have to transmit SSB for network energy saving, where the time/frequency synchronization and L1/L3 measurements of the SCell can be derived from the PCell or another SCell. SSB-less SCell operation in intra-band CA has already been supported in Rel-15. In Rel-18, inter-band CA with SSB-less need to be further discussed.

Currently, if the SSB-less SCell works it should have its UL timing reference, pathloss reference, and determine the location of the CSI-RS resource based on the timing information of the serving cell transmitted SSB if CSI-RS is transmitted in SSB-less SCell.

- if the SSB-less SCell works it should have its

- UL timing reference,

- pathloss reference

- QCL reference for CSI-RS measurements

When an SCell does not transmit SSB, from the perspective of RAN2, we can analyze whether RAN2 signaling is currently supported in the case of SSB-less SCell operation, as state below:

  - CSI-RS measurements

Currently, the CSI-RS may be used for time/frequency tracking, CSI computation, L1-RSRP computation and mobility. Among them, the configuration of CSI-RS for mobility in IE CSI-RS-ResourceConfigMobility is completely independent of the resource configuration of CSI-RS for other purposes. The resource of CSI-RS for other purposes, such as TRS, CSI-RS for BM, CSI-RS for CSI, can be uniformly configured in IE NZP-CSI-RS-ResourceSet, and distinguish them through repetition and trs-info.

| Purpose of CSI | Parameter configuration |
| --- | --- |
| For time/frequency tracking | Repetition: off/absent trs-info: true CSI-RS-Resource-Mobility: not configured |
| For CSI computation | Repetition: off/absent trs-info: absent CSI-RS-Resource-Mobility: not configured |
| For L1-RSRP computation | Repetition: on/absent trs-info: absent CSI-RS-Resource-Mobility: not configured |
| For mobility | Repetition: off/absent trs-info: absent CSI-RS-Resource-Mobility: configured |

Table 1 Parameter configuration for different CSI purposes

### CSI-RS for RRM

In the scheme of CSI-RS based RRM measurements, the time/frequency synchronization is obtained through the SSB indicated by cellId and associatedSSB in CSI-RS-ResourceConfigMobility:

• When associatedSSB is configured: the UE first detects the SSB associated with the CSI-RS, determines the target cell through the SSB, and then determines the location of the target CSI-RS resource based on the timing information of the target cell.

• When associatedSSB is not configured: the location of CSI-RS resources can be directly determined by the IE refServCellIndex, which indicates the serving cell providing the timing reference for the CSI-RS resource. There is no need to do additional target cell synchronization signal detection.

From RAN2 perspective, there is no restriction whether the target cellId and the cellId associated with the parameter of associatedSSB or refServCellIndex are intra-band or inter-band. Therefore, RAN2 signaling is currently supported for CSI-RS based RRM measurements in the case of SSB-less SCell operation for inter-band CA.

CSI-RS-ResourceConfigMobility information element

CSI-RS-ResourceConfigMobility ::=   SEQUENCE {

subcarrierSpacing                   SubcarrierSpacing,

csi-RS-CellList-Mobility            SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM)) OF CSI-RS-CellMobility,

...,

[[

refServCellIndex                    ServCellIndex                                                           OPTIONAL    -- Need S

]]

}

CSI-RS-CellMobility ::=             SEQUENCE {

cellId                              PhysCellId,

csi-rs-MeasurementBW                SEQUENCE {

nrofPRBs                            ENUMERATED { size24, size48, size96, size192, size264},

startPRB                            INTEGER(0..2169)

},

density                             ENUMERATED {d1,d3}                                                      OPTIONAL,   -- Need R

csi-rs-ResourceList-Mobility        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility

}

CSI-RS-Resource-Mobility ::=        SEQUENCE {

csi-RS-Index                        CSI-RS-Index,

slotConfig                          CHOICE {

ms4                                 INTEGER (0..31),

ms5                                 INTEGER (0..39),

ms10                                INTEGER (0..79),

ms20                                INTEGER (0..159),

ms40                                INTEGER (0..319)

},

associatedSSB                       SEQUENCE {

ssb-Index                           SSB-Index,

isQuasiColocated                    BOOLEAN

}                                                                                                           OPTIONAL, -- Need R

- RAN2 signaling is currently supported for CSI-RS based RRM measurements in the case of SSB-less SCell operation for inter-band CA.

### CSI-RS for other purposes

TRS plays an very important role in the time/frequency precise synchronization of the whole system. For TRS, it can obtain QCL type C or QCL type C+D from the QCL source SSB signal transmitted by other SCell. Further, CSI-RS for BM or CSI-RS for CSI can use the obtained TRS or SBB of other Scell as QCL source reference signal to obtain QCL type A or QCL type B or type C +D information. Supported QCL relationship are listed in table 2.

| QCL source signal | Target reference signal | Large-scale parameters | Type |
| --- | --- | --- | --- |
| SSB | TRS | Doppler shift, average delay | Type C |
| TRS | CSI-RS for CSI | Doppler shift, Doppler spread, average delay, delay spread | Type A |
| TRS | DMRS for PDCCH/PDSCH | Doppler shift, Doppler spread, average delay, delay spread | Type A |
| TRS | CSI-RS for CSI | Doppler shift, Doppler spread | Type B |
| CSI-RS | DMRS for PDCCH/PDSCH | Doppler shift, Doppler spread, average delay, delay spread | Type A |
| SSB | TRS | Doppler shift, average delay, Spatial Rx parameter | Type C + D |
| TRS | CSI-RS for BM | Doppler shift, Doppler spread, average delay, delay spread | Type A + D |
| SSB | CSI-RS for BM | Doppler shift, average delay, Spatial Rx parameter | Type C + D |

Table 2 QCL relationship between RSs

From RAN2 perspective, distinguish CSI-RS resources with different purposes through repetition and trs-info, which are configured in NZP-CSI-RS-ResourceSet. For each NZP-CSI-RS-Resource in NZP-CSI-RS-ResourceSet, TCI state can be configured to indicate its corresponding QCL relationship. Therefore, from the current signaling configuration of RAN2, there is no restriction that intra-band or inter-band cross carrier QCL can be configured on the RSs of the cell without SSB. RAN2 signaling is currently supported for QCL relationship between RSs in the case of SSB-less SCell operation for inter-band CA.

NZP-CSI-RS-ResourceSet information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCESET-START

NZP-CSI-RS-ResourceSet ::=          SEQUENCE {

nzp-CSI-ResourceSetId               NZP-CSI-RS-ResourceSetId,

nzp-CSI-RS-Resources                SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId,

repetition                          ENUMERATED { on, off }                                                  OPTIONAL,   -- Need S

aperiodicTriggeringOffset           INTEGER(0..6)                                                           OPTIONAL,   -- Need S

trs-Info                            ENUMERATED {true}

NZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCE-START

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}                 OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                                     OPTIONAL,   -- Cond Periodic

...

}

TCI-State information element

-- ASN1START

-- TAG-TCI-STATE-START

TCI-State ::=                       SEQUENCE {

tci-StateId                         TCI-StateId,

qcl-Type1                           QCL-Info,

qcl-Type2                           QCL-Info                                                    OPTIONAL,   -- Need R

...,

}

QCL-Info ::=                        SEQUENCE {

cell                                ServCellIndex                                               OPTIONAL,   -- Need R

bwp-Id                              BWP-Id                                                      OPTIONAL, -- Cond CSI-RS-Indicated

referenceSignal                     CHOICE {

csi-rs                              NZP-CSI-RS-ResourceId,

ssb                                 SSB-Index

},

qcl-Type                            ENUMERATED {typeA, typeB, typeC, typeD},

...

}

A-TRS

The Rel-17 fast SCell activation introduces A-TRS. TCI-state can be configured in A-TRS, which can replace SSB to complete time/frequency synchronization of the SCell. From RAN2 perspective, there is no restriction that intra-band or inter-band cross carrier QCL information can be configured on the A-TRS of the SCell without SSB. Therefore, RAN2 signaling is currently supported for A-TRS in the case of SSB-less SCell operation for inter-band CA.

SCellActivationRS-Config information element

-- ASN1START

-- TAG-SCELLACTIVATIONRS-CONFIG-START

SCellActivationRS-Config-r17 ::= SEQUENCE {

scellActivationRS-Id-r17         SCellActivationRS-ConfigId-r17,

resourceSet-r17                  NZP-CSI-RS-ResourceSetId,

gapBetweenBursts-r17             INTEGER (2..31)                                                            OPTIONAL, -- Need R

qcl-Info-r17                     TCI-StateId,

...

}

- RAN2 signaling is currently supported for QCL relationship between RSs in the case of SSB-less SCell operation for inter-band CA.

  - UL Timing reference

Cite from TS38.133

- 7.1.1 Introduction The UE shall have capability to follow the frame timing change of the reference cell in connected state or when transmiting PUSCH on CG resources for SDT in RRC_Inactive. The uplink frame transmission takes place before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For serving cell(s) in pTAG, UE shall use the SpCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. For serving cell(s) in sTAG, UE shall use any of the activated SCells as the reference cell for deriving the UE transmit timing for the cells in the sTAG. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

From the TS38.133 spec, UL timing of UEs mainly depends on serving cell in which TAG. If the SSB-less SCell in pTAG, UE shall use SpCell as the reference cell. If the SSB-less SCell in sTAG, UE shall use and of activated SCells as the reference cell. The tag-id is configured in ServingCellConfig. From RAN2 perspective, there is no restriction that only intra-band SCell can be configured in pTAG or sTAG. Therefore, RAN2 signaling currently can also support to UL timing reference for UE in the case of SSB-less SCell operation for inter-band CA

ServingCellConfig information element

-- ASN1START

-- TAG-SERVINGCELLCONFIG-START

ServingCellConfig ::=               SEQUENCE {

…,

crossCarrierSchedulingConfig        CrossCarrierSchedulingConfig                                            OPTIONAL,   -- Need M

tag-Id                              TAG-Id,

dummy1                              ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

pathlossReferenceLinking            ENUMERATED {spCell, sCell}                                              OPTIONAL,   -- Cond SCellOnly

servingCellMO                       MeasObjectId                                                            OPTIONAL,   -- Cond MeasObject

...,

- RAN2 signaling is currently supported to obtain UL timing reference for UE in the case of SSB-less SCell operation for inter-band CA.

  - Pathloss reference

Currently, CSI-RS or SSB can both be used for path loss estimation. When a SCell does not transmit SSB, UE can estimate the path loss through the CSI-RS resources configured by the network. Through the previous analysis of CSI-RS, the corresponding QCL relationship can be obtained by the configuration of TCI state, which can determine the location of the CSI-RS resource based on the timing information of the SCell transmitted SSB. Therefore, RAN2 signaling is currently supported to obtain pathloss reference for UE in the case of SSB-less SCell operation for inter-band CA.

PUCCH-PowerControl information element

-- ASN1START

-- TAG-PUCCH-POWERCONTROL-START

PUCCH-PowerControl ::=              SEQUENCE {

…,

PUCCH-PathlossReferenceRS ::=                   SEQUENCE {

pucch-PathlossReferenceRS-Id                PUCCH-PathlossReferenceRS-Id,

referenceSignal                             CHOICE {

ssb-Index                                   SSB-Index,

csi-RS-Index                                NZP-CSI-RS-ResourceId

}

}

- RAN2 signaling is currently supported to obtain pathloss reference for UE in the case of SSB-less SCell operation for inter-band CA.

Cross the discussion of RAN4, we think that RAN2 signaling is currently supported in the case of SSB-less SCell operation. What corresponding enhancements are needed for the RAN2 signaling or a new UE capability in the next step, we need to wait for the specific solution of RAN4. Therefore, based on this and we propose the following:

- RAN2 to further discuss the possible configuration enhancements or a new UE capability until further guidance from RAN4 on how the inter-band SSB-less CA is supported.

Conclusion

Based on the discussion, we have the following observation and proposals:

- if the SSB-less SCell works it should have its

  - UL timing reference,

  - pathloss reference

  - QCL reference for CSI-RS measurements

- RAN2 signaling is currently supported for CSI-RS based RRM measurements in the case of SSB-less SCell operation for inter-band CA.

- RAN2 signaling is currently supported for QCL relationship between RSs in the case of SSB-less SCell operation for inter-band CA.

- RAN2 signaling is currently supported to obtain UL timing reference for UE in the case of SSB-less SCell operation for inter-band CA.

- RAN2 signaling is currently supported to obtain pathloss reference for UE in the case of SSB-less SCell operation for inter-band CA.

- RAN2 to further discuss the possible configuration enhancements or a new UE capability until further guidance from RAN4 on how the inter-band SSB-less CA is supported.

References

[1] RP-223540, Network energy savings for NR, Huawei.