3GPP TSG-RAN WG2 #121	R2-2300452

Athens, Greece, 27th February – 3rd March 2023

Agenda Item:	8.20.2

Source:	Ericsson

Title:	SDT Enhancements for Configured grants

Document for:	Discussion, Decision

# 1	Introduction

The Work Item RP-200954 ‘New Work Item on NR small data transmissions in INACTIVE state’ was finalized in 3GPP Rel-17 with the focus of optimizing the transmission for small data payloads by reducing the signaling overhead. The WI contains the following relevant objectives:

This work item enables small data transmission in RRC_INACTIVE state as follows: - For the RRC_INACTIVE state: - UL small data transmissions for RACH-based schemes (i.e. 2-step and 4-step RACH): - General procedure to enable UP data transmission for small data packets from INACTIVE state (e.g. using MSGA or MSG3) [RAN2] - Enable flexible payload sizes larger than the Rel-16 CCCH message size that is possible currently for INACTIVE state for MSGA and MSG3 to support UP data transmission in UL (actual payload size can be up to network configuration) [RAN2] - Context fetch and data forwarding (with and without anchor relocation) in INACTIVE state for RACH-based solutions [RAN2, RAN3] Note 1: The security aspects of the above solutions should be checked with SA3 - Transmission of UL data on pre-configured PUSCH resources (i.e. reusing the configured grant type 1) – when TA is valid - General procedure for small data transmission over configured grant type 1 resources from INACTIVE state [RAN2] - Configuration of the configured grant type1 resources for small data transmission in UL for INACTIVE state [RAN2]

For NB-IoT and LTE-M similar signaling optimizations for small data have been introduced through Rel-15 Early Data Transmission (EDT) and Rel-16 Preconfigured Uplink Resources (PUR). Somewhat similar solutions were specified for NR with the difference that the Rel-17 NR Small Data is only to be supported for RRC INACTIVE state and also includes 2-step RACH based small data transmissions. One major difference regarding the applicability and usefulness is for PUR versus CG-SDT, where PUR supports much larger periodicities compared to CG-SDT.

This contribution discusses improvements to CG-SDT resulting in more efficient resource utilization.

# 2	Discussion

## 2.1	Problem

The Rel-17 specification of CG-SDT resulted in a procedure which is not very useful for important use cases where the data periodicity is longer than the maximum CG periodicity of 640ms. An inherent issue with configured grants is that if the traffic pattern does not match the transmission opportunities, there will be a high overhead in terms of unused but reserved resources. In Rel-17, only the same periodicities as in connected mode were agreed which implies a high resource waste for CG-SDT when applied to applications with long periodicities. In fact, if the traffic matches reasonably well to the short available periodicities and TB sizes, CG in connected mode is a more attractive alternative than CG-SDT since CG in connected mode supports connected mode DRX. The waste of resources is also typically higher in CG-SDT, since the resources may be configured but not used for a long time in cases when the traffic is sporadic. An illustration of the problem is depicted in Figure 1.

Figure 1. Illustration of unused CG-SDT resources when data does not match CG periodicity

The maximum periodicity which can be configured for CG transmission occasions is 640ms. For many of the possible use cases with periodic data, such as transmitting meter readings from static devices, the periodicity is much longer. Using CG-SDT for this type of application results in that many of the reserved CG-transmission occasions are never used and a considerable overhead of unused reserved resources.

The argumentation against allowing longer periodicities is the fact that the initial transmission needs to be acknowledged before any new transmission can be done. In the current specification, retransmissions in case of absence of acknowledgment is done at the next CG transmission opportunity. In case of long periodicity, the procedure would be slow in case a retransmission of the initial transmission is needed and potentially even time out (expiry of T319a) before any retransmission can be performed.

An illustration of how a successful operation could look like in case of long periodicities were allowed is shown in Figure 2. In this case, the periodicity is much longer than the maximum possible configuration of T319a which is 4000ms. However, if the UE is released after every CG transmission (or after possible retransmissions or subsequent transmissions) the failure timer still serves its purpose, i.e. to handle failure of single or short bursts of data at the RRC level.

Available timer settings and operation of T319a are sufficient to handle long periodicities for CG-SDT if the UE is released after the CG transmission.

Figure 2. Illustration of operation with longer periodicities. UE released after each transmission.

The problem of allowing long CG periodicities is if the gNB does not detect the CG transmission and consequently does not acknowledge the CG transmission. In this case the UE would according to current standard need to wait until the next CG transmission occasion to perform a retransmission. However, this would not be possible since T319a would time out before next CG transmission occasion and the UE would transit to Idle. In this case a more efficient way to retransmit than waiting for the next CG occasion is needed to prevent the UE from going to Idle.

Retransmissions of the initial transmission must be performed earlier than the next CG occasion if the CG-SDT periodicity is long.

In order to perform a retransmission a grant is needed. For SDT, no PUCCH SR resources are configured. In RAN2#118-e it was also agreed that RA can only be performed after the initial CG-transmission has been acknowledged. Hence, using RA to obtain a grant before the initial transmission has been acknowledged is not allowed in current specification. Furthermore, an RA-SDT procedure cannot be initiated once a CG-SDT procedure has been initiated.

There are no means to obtain a grant before the initial CG-SDT transmission has been acknowledged according to current specification.

## 2.2	Fallback to RA-SDT when no acknowledgement is obtained

To solve the problem of slow retransmissions when having a long CG-SDT periodicity we think that the option of triggering random access in case of no acknowledgement would be beneficial. This could preferably be done by using RA-SDT resources where the RRCResumeRequest and data is retransmitted. This is essentially equivalent to initiating a new RA-SDT procedure. This also seems to be a reasonable approach since the reason why no acknowledgement or grant for retransmission is obtained would typically be either radio conditions not suitable for CG-SDT or an insufficient time alignment. Both of these conditions imply that RA-SDT should be used. It should also be noted that the need to trigger RA-SDT is only for the initial transmission when the gNB is unaware whether the UE has initiated an SDT procedure or not. This minimizes the specification changes since only the initial MAC PDU is in the HARQ buffer so only this MAC PDU needs to be moved to the MSG3 buffer.

If no feedback is received for the initial CG transmission when CG periodicity is long, a RA-SDT procedure is triggered.

What periodicity that can be considered as a long periodicity or other conditions for when a RA-SDT procedure can be triggered could be configurable. In a similar way, the time period to wait for a response before triggering the RA-SDT could be configurable. Conditions for when it is allowed to trigger RA-SDT when initial CG-SDT transmission fails can be configurable.

Given that the above proposals are accepted, we believe that longer CG-SDT periodicities should be added to the current periodicities agreed for CG-SDT.

Longer periodicities for CG-SDT can be configured.

Agree the TP for TS 38.321 v17.3.0 in Section 3

# 3	Text Proposal for TS 38.321 v17.3.0

### *************** First change********************************

### 5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted on a non-terrestrial network:

3>	start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the PDCCH occasion as specified in TS 38.213 [6].

2>	else:

3>	start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission.

2>	monitor for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while ra-ResponseWindow is running.

1>	else:

2>	if the Random Access Preamble was transmitted on a non-terrestrial network:

3>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the PDCCH occasion as specified in TS 38.213 [6].

2>	else:

3>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission.

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running.

1>	if notification of a reception of a PDCCH transmission on the search space indicated by recoverySearchSpaceId is received from lower layers on the Serving Cell where the preamble was transmitted; and

1>	if PDCCH transmission is addressed to the C-RNTI; and

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	consider the Random Access procedure successfully completed.

1>	else if a valid (as specified in TS 38.213 [6]) downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded:

2>	if the Random Access Response contains a MAC subPDU with Backoff Indicator:

3>	set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI.

2>	else:

3>	set the PREAMBLE_BACKOFF to 0 ms.

2>	if the Random Access Response contains a MAC subPDU with Random Access Preamble identifier corresponding to the transmitted PREAMBLE_INDEX (see clause 5.1.3):

3>	consider this Random Access Response reception successful.

2>	if the Random Access Response reception is considered successful:

3>	if the Random Access Response includes a MAC subPDU with RAPID only:

4>	consider this Random Access procedure successfully completed;

4>	indicate the reception of an acknowledgement for SI request to upper layers.

3>	else:

4>	apply the following actions for the Serving Cell where the Random Access Preamble was transmitted:

5>	process the received Timing Advance Command (see clause 5.2);

5>	indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP);

5>	if the Random Access procedure for an SCell is performed on uplink carrier where pusch-Config is not configured:

6>	ignore the received UL grant.

5>	else:

6>	process the received UL grant value and indicate it to the lower layers.

4>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s):

5>	consider the Random Access procedure successfully completed.

4>	else:

5>	set the TEMPORARY_C-RNTI to the value received in the Random Access Response;

5>	if this is the first successfully received Random Access Response within this Random Access procedure:

6>	if the transmission is not being made for the CCCH logical channel:

7>	indicate to the Multiplexing and assembly entity to include a C-RNTI MAC CE in the subsequent uplink transmission.

6>	if the Random Access procedure was initiated for SpCell beam failure recovery and spCell-BFR-CBRA with value true is configured:

7>	if there is at least one Serving Cell of this MAC entity configured with two BFD-RS sets:

8>	indicate to the Multiplexing and assembly entity to include an Enhanced BFR MAC CE or a Truncated Enhanced BFR MAC CE in the subsequent uplink transmission.

7>	else:

8>	indicate to the Multiplexing and assembly entity to include a BFR MAC CE or a Truncated BFR MAC CE in the subsequent uplink transmission.

6>	else if the Random Access procedure was initiated for beam failure recovery of both BFD-RS sets of SpCell:

7>	indicate to the Multiplexing and assembly entity to include an Enhanced BFR MAC CE or a Truncated Enhanced BFR MAC CE in the subsequent uplink transmission.

6> if the RA procedure was triggered as a result of not receiving a PDCCH for the initial CG-SDT transmission:

7> obtain the MAC PDU from the HARQ buffer containing the MAC PDU of the initial CG-SDT and place it in the Msg3 buffer

7> flush the HARQ buffer containing the MAC PDU of the initial CG-SDT transmission

6>	else:

7> obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the Msg3 buffer.

NOTE:	If within a Random Access procedure, an uplink grant provided in the Random Access Response for the same group of contention-based Random Access Preambles has a different size than the first uplink grant allocated during that Random Access procedure, the UE behavior is not defined.

### *************** End of first change****************************

### *************** Second change********************************

### 5.8.2	Uplink

There are two types of transmission without dynamic grant:

-	configured grant Type 1 where an uplink grant is provided by RRC, and stored as configured uplink grant;

-	configured grant Type 2 where an uplink grant is provided by PDCCH, and stored or cleared as configured uplink grant based on L1 signalling indicating configured uplink grant activation or deactivation.

Type 1 and Type 2 are configured by RRC for a Serving Cell per BWP. Multiple configurations can be active simultaneously in the same BWP. For Type 2, activation and deactivation are independent among the Serving Cells. For the same BWP, the MAC entity can be configured with both Type 1 and Type 2.

Only configured grant Type 1 can be configured for CG-SDT. CG-SDT can only be configured on initial BWP.

RRC configures the following parameters when the configured grant Type 1 is configured:

-	cs-RNTI: CS-RNTI for retransmission;

-	cg-SDT-CS-RNTI: CS-RNTI for CG-SDT retransmission;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT;

-	periodicity: periodicity of the configured grant Type 1;

-	timeDomainOffset: Offset of a resource with respect to SFN = timeReferenceSFN in time domain;

-	timeDomainAllocation: Allocation of configured uplink grant in time domain which contains startSymbolAndLength (i.e. SLIV in TS 38.214 [7]) or startSymbol (i.e. S in TS 38.214 [7]);

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant configured with cg-RetransmissionTimer for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant not configured with cg-RetransmissionTimer;

-	timeReferenceSFN: SFN used for determination of the offset of a resource in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the configured grant configuration.

RRC configures the following parameters when the configured grant Type 2 is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	periodicity: periodicity of the configured grant Type 2;

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant configured with cg-RetransmissionTimer for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant not configured with cg-RetransmissionTimer.

-	CG-SDT-to-RA-SDT-Fallback: if this parameter is configured, the UE can fallback to RA-SDT if no response to the initial CG-SDT transmission is received.

RRC configures the following parameter when retransmissions on configured uplink grant is configured:

-	cg-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process when the UE shall not autonomously retransmit that HARQ process;

-	cg-SDT-RetransmissionTimer: the duration after a configured grant (re)trasnmission of a HARQ process of the initial CG-SDT transmission with CCCH message when the UE shall not autonomously retransmit the HARQ process.

Upon configuration of a configured grant Type 1 for a BWP of a Serving Cell by upper layers, the MAC entity shall:

1>	store the uplink grant provided by upper layers as a configured uplink grant for the indicated BWP of the Serving Cell;

1>	initialise or re-initialise the configured uplink grant to start in the symbol according to timeDomainOffset, timeReferenceSFN, and S (derived from SLIV or provided by startSymbol as specified in TS 38.214 [7]), and to reoccur with periodicity.

After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider sequentially that the Nth (N >= 0) uplink grant occurs in the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	(timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity)
	modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

For an uplink grant configured for configured grant Type 1 for CG-SDT on the selected uplink carrier as in clause 5.27, when CG-SDT is triggered and not terminated, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if, after initial transmission for CG-SDT with CCCH message has been performed according to clause 5.4.1, PDCCH addressed to the MAC entity's C-RNTI has not been received:

2> if cg-SDT-to-RA-SDT-Fallback is configured:

3> initiate Random Access procedure for SDT in clause 5.1.

2>	else if the SSB corresponding to the configured UL grant has the same SSB index as the SSB selected for initial transmission for CG-SDT with CCCH message (i.e., retransmission of initial transmission of CG-SDT):

3>	select this SSB;

3>	indicate the SSB index corresponding to the configured uplink grant to the lower layer;

3>	consider this configured uplink grant as valid.

1>	else if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available:

### *************** End of second change****************************

# 4 Conclusion

In the previous sections we made the following observations:

Observation 1	Available timer settings and operation of T319a are sufficient to handle long periodicities for CG-SDT if the UE is released after the CG transmission.

Observation 2	Retransmissions of the initial transmission must be performed earlier than the next CG occasion if the CG-SDT periodicity is long.

Observation 3	There are no means to obtain a grant before the initial CG-SDT transmission has been acknowledged according to current specification.

Based on the discussion in the previous sections we propose the following:

Proposal 1	If no feedback is received for the initial CG transmission when CG periodicity is long, a RA-SDT procedure is triggered.

Proposal 2	What periodicity that can be considered as a long periodicity or other conditions for when a RA-SDT procedure can be triggered could be configurable. In a similar way, the time period to wait for a response before triggering the RA-SDT could be configurable. Conditions for when it is allowed to trigger RA-SDT when initial CG-SDT transmission fails can be configurable.

Proposal 3	Longer periodicities for CG-SDT can be configured.

Proposal 4	Agree the TP for TS 38.321 v17.3.0 in Section 3.1

# 4 References

RP-200954 ‘Work Item on NR small data transmissions in INACTIVE state’, ZTE Corporation, RAN#86, December 2019.