3GPP TSG-RAN WG2 #121                                             R2-2211838

Athens, Greece, February November, 2022

Source: 			OPPO

Title: 	Consideration on RAT-dependent positioning integrity

Agenda item:		8.2.3

Document for: 	Discussion and Decision

- Introduction

In the last RAN#98e meeting, agreements regarding the RAT-dependent integrity have been made as follows:

Specify the error modelling parameters, signaling, and procedures to support UE-based and LMF-based integrity of RAT-dependent positioning methods

In this paper, we would like to address our views on the RAT-independent integrity

- Discussion

- 2.1 Report regarding error sources to support positioning integrity

As could be found in the latest TR 38.859[1], the positioning integrity error source comes from two sides: UE and TRP, as indicated as follows:

- UE measurement:

- RSTD measurement performed by the UE for the DL-TDOA

- UE Rx-Tx time difference measurement performed by the UE for the Multi-RTT

- DL-PRS RSRPP of the first path or RSRP measurement performed by the UE for the DL-AOD

- TRP measurement:

- RTOA measurement performed by the TRPs for the UL-TDOA

- Angle of arrival measurement performed by the TRPs for the UL-AOA

- gNB Rx-Tx time difference measurement performed by the TRPs for the Multi-RTT

- TRP information:

- TRP locations for various positioning methods

- Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.) for various positioning methods

- ARP location for the UL-AOA

Obviously, UE needs to send the integrity information related to the DL measurement performed to the LMF. Otherwise, the integrity result derived by the LMF may not reflect the practical situation. In our opinion, the simplest way is to introduce the integrity related IE into the positioning method related ProvideLocationInformation IEs such as nr-DL-TDOA-ProvideLocationInformation, nr-DL-AoD-ProvideLocationInformation, nr-Multi-RTT-ProvideLocationInformation in the LPP ProvideLocationInformation msg. Another way is to introduce a new msg dedicated to transmitting the positioning measurement result related integrity information towards the LMF. For now, we prefer the first option due to its simplicity, the LMF could immediately know the integrity related info. upon reception of the positioning measurement result.

Proposal 1: RAN2 to agree to embed the integrity related IE into the positioning method related ProvideLocationInformation IEs such as nr-DL-TDOA-ProvideLocationInformation, nr-DL-AoD-ProvideLocationInformation, nr-Multi-RTT-ProvideLocationInformation in the LPP ProvideLocationInformation msg.

Secondly, the gNB should send UL SRS measurement related integrity related information towards the LMF. In the latest NRPPa protocol, two NRPPa signalling msgs, NRPPa MEASUREMENT REPORT and NRPPa MEASUREMENT RESPONSE, are employed to send the positioning measurement results towards the network. Hence, to support positioning integrity, enhancement on these two signalling msgs should be made. Since RAN3 is in charge of NRPPa spec, we propose RAN2 to send a LS to RAN3 triggering enhancement on the two signalling msgs regarding supporting the positioning integrity.

Finally, there may exist uncertainties regarding TRP location information and inter-TRP SFN synchronization performance, and such uncertainties need to be also reported to the LMF or the UE for derivation of the positioning integrity. In the latest NRPPa protocol, the TRP Information IE included in the NRPPa TRP INFORMATION RESPONSE msg embraces the TRP location information. As the spec impact details need to be further discussed in RAN3, a LS triggering them to initialize NRPPa enhancement discussion may be needed also. For the UE-based positioning method, the TRP information such as nr-TRP-LocationInfo IE and NR-RTD-Info IE is conveyed to the UE in the NR-PositionCalculationAssistance embedded in the LPP ProvideAssistanceData msg. As a result, RAN2 is kindly proposed to introduce the positioning integrity IE in the LPP ProvideAssistanceData msg.

Proposal 2: RAN2 to agree to send a LS to RAN3 to trigger discussion of positioning integrity enhancement on the two kind of signalling msgs: NRPPa measurement report related msgs and TRP/ARP information related msgs.

Proposal 3: RAN2 to agree to embed the integrity related IE into NR-PositionCalculationAssistance IE in the LPP ProvideAssistanceData msg for various UE based DL positioning methods.

- 2.2 Signalling enhancement to support UE-based positioning integrity

In the latest TR 38.859[1], the positioning integrity signalling impacts on the current LMF-based and UE-based positioning signalling have been found as follows:

- LMF-based:

-	UE sends capability info to LMF for LMF-based positioning integrity mode using LPP capability transfer procedure

-	LMF sends the request of results related to integrity for integrity error sources to UE for integrity of LMF-based mode

-	LMF sends the request of results related to integrity for integrity error sources to RAN for integrity of LMF-based mode

-	RAN sends results related to positioning integrity to LMF using NRPPa message.

NOTE 1: The signalling to transmit integrity KPI and integrity results can be discussed during normative work.

NOTE 2: Whether UE sends results related to integrity to LMF using LPP message or not can be discussed during normative work.

- UE-based:

- UE sends capability info to LMF on integrity for UE-based mode using LPP capability transfer procedure.

-	LMF sends the assistance data for integrity calculation to UE. LMF provides, in assistance data, the information of error sources (e.g., originated from RAN node) to UE for integrity in UE-based mode.

-	LMF sends integrity requirement e.g., TIR to UE in LPP request location information message for integrity of UE-based mode.

-	UE sends integrity result to LMF using LPP location information Transfer message.

Firstly, the LPP capability transfer procedure needs to be enhanced to support requesting and conveying the positioning integrity information between the UE and LMF. In our opinion, there are two options regarding definition of the positioning integrity capability. The first is to include new flags in each positioning method related ProvideCapabilities IEs, such as otdoa-ProvideCapabilities, etc. The second is to include a new flag generally indicating whether or not the UE supports integrity or not in the LPP RequestCapabilities and ProvideCapabilities msgs but outside each positioning method-related RequestCapabilities and ProvideCapabilities IEs.

Proposal 4: RAN2 to discuss the granularity of the RAT-dependent positioning integrity capability flag to be included in at least the LPP ProvideCapabilities msgs, which valids either for all RAT-dependent positioning methods or per RAT-dependent positioning method.

Secondly, we think that it is reasonable for the LMF to send the LPP RequestLocationInfomration msg towards the UE not only requesting for the location measurement results but also the results related to positioning integrity for integrity error sources. Also, the integrity KPI related information could be optionally included in the LPP RequestLocationInfomration msg, if it is UE to derive the positioning results.

Proposal 5: RAN2 to agree to include the IEs related to request of results related to positioning integrity for integrity error sources, and optionally, the integrity KPI, in the LPP RequestLocationInfomration msg.

- 2.3The feasibility of decouple the entity for integrity computation from positioning

In the R18, a question raised that whether or not the entity for integrity computation and the one for positioning could be decoupled. Let us take DL-TDOA as an example.

Firstly, we decouple the entity for integrity computation and positioning computation: choosing the UE as the positioning entity and the LMF as the integrity computation entity. After performing the DL measurement result, the UE needs to send the integrity related assistance information, e.g., feared events of the measurement errors to the LMF. Subsequently, the LMF derives the positioning integrity result based on the assistance information and sends it back to the UE, if it is the UE trigger the LCS service. As can be found, to apply the positioning result in practice for positioning integrity-sensitive services, the UE needs to wait for at least two pieces of LPP msg transmission time to obtain the related positioning integrity result from the LMF, indicated as follows:

Figure 1: UE needs to wait for at least two pieces of LPP msg transmission time to obtain the related positioning integrity result from the LMF

Observation 1: if UE serves the positioning entity and the LMF serves as the integrity computation entity, for applying the DL positioning result in practice for positioning integrity-sensitive services, the UE needs to wait at least two pieces of LPP msg transmission and processing time to obtain the related positioning integrity result from the LMF.

Secondly, if we choose the LMF as the positioning entity and the UE as the integrity computation entity, after performing the DL-PRS measurement, the UE needs to send the DL-PRS measurement result towards the LMF. Then, the LMF derives the positioning result based on the DL-PRS measurement result and sends it back to the UE, if it is the UE triggering the LCS service. Similar with the above implementation, it could be found, UE at least needs to wait for two pieces of LPP msg transmission and processing time prior to the availability of the integrity checked positioning result.

Figure 1: UE needs to wait for at least two pieces of LPP msg transmission time to obtain the related positioning result from the LMF

Observation 2: if LMF serves the positioning entity and the UE serves as the integrity computation entity, for applying the DL positioning result in practice for positioning integrity-sensitive services, the UE needs to wait for at least two pieces of LPP msg transmission and processing time to obtain the related positioning integrity result from the LMF.

As a result, from the perspective of saving unnecessary processing time for the time-sensitive positioning services, we propose RAN2 to agree that the entity for integrity computation and the one for positioning computation should not be decoupled.

Proposal 6: RAN2 to agree that the entity for integrity computation and the one for positioning computation should not be decoupled, from the perspective of saving unnecessary processing time for the time-sensitive positioning services.

- Conclusion and proposals

RAN2 is kindly asked to discuss and adopt the following observations and proposals:

Proposal 1: RAN2 to agree to embed the integrity related IE into the positioning method related ProvideLocationInformation IEs such as nr-DL-TDOA-ProvideLocationInformation, nr-DL-AoD-ProvideLocationInformation, nr-Multi-RTT-ProvideLocationInformation in the LPP ProvideLocationInformation msg.

Proposal 2: RAN2 to agree to send a LS to RAN3 to trigger discussion of positioning integrity enhancement on the two kind of signalling msgs: NRPPa measurement report related msgs and TRP/ARP information related msgs.

Proposal 3: RAN2 to agree to embed the integrity related IE into NR-PositionCalculationAssistance IE in the LPP ProvideAssistanceData msg for various UE based DL positioning methods.

Proposal 4: RAN2 to discuss the granularity of the RAT-dependent positioning integrity capability flag to be included in at least the LPP ProvideCapabilities msgs, which valids either for all RAT-dependent positioning methods or per RAT-dependent positioning method.

Proposal 5: RAN2 to agree to include the IEs related to request of results related to positioning integrity for integrity error sources, and optionally, the integrity KPI, in the LPP RequestLocationInfomration msg.

Observation 1: if UE serves the positioning entity and the LMF serves as the integrity computation entity, for applying the DL positioning result in practice for positioning integrity-sensitive services, the UE needs to wait at least two pieces of LPP msg transmission and processing time to obtain the related positioning integrity result from the LMF.

Observation 2: if LMF serves the positioning entity and the UE serves as the integrity computation entity, for applying the DL positioning result in practice for positioning integrity-sensitive services, the UE needs to wait for at least two pieces of LPP msg transmission and processing time to obtain the related positioning integrity result from the LMF.

Proposal 6: RAN2 to agree that the entity for integrity computation and the one for positioning computation should not be decoupled, from the perspective of saving unnecessary processing time for the time-sensitive positioning services.