3GPP TSG-RAN WG2 #121	R2-2300463 
Athens, Greece, February 2023

Agenda Item:	8.5.4

Source:	OPPO

Title:	Discussion on capacity improvement

Document for:	Discussion, Decision

Introduction

This paper will discuss XR-specific capacity improvement.

Discussion

Characteristic-related information report

In Rel-16 URLLC, TSCAI is introduced to describe TSC traffic characteristics. The TSCAI is provided to the RAN from SMF per QoS flow, which contains the flow direction, periodicity and burst arrival time. Knowing that the TSC traffic characteristics are useful for RAN on more efficient scheduling for the QoS flows that have periodic and deterministic traffic characteristics.

Table 1: TSC Assistance Information (TSCAI) in TS 23.501

| Assistance Information | Description |
| --- | --- |
| Flow Direction | The direction of the TSC flow (uplink or downlink). |
| Periodicity | It refers to the time period between start of two data bursts. |
| Burst Arrival Time (optional) | The latest possible time when the first packet of the data burst arrives at either the ingress of the RAN (downlink flow direction) or the egress of the UE (uplink flow direction). |
| Survival Time (optional) | Survival Time, as defined in TS 22.261 [2], refers to the time period an application can survive without any data burst. |

- TSCAI is designed to provide the gNB with the TSC characteristics which contain direction, periodicity and burst arrival time.

Similar to TSC, XR is also periodic traffic with latency requirements. To guarantee efficient scheduling for the XR service, it would be good to let the gNB know XR characteristics.

In the RAN2#120 meeting, RAN2 assumes XR characteristics may be informed to the gNB by using TSCAI, but details depend on the SA2 conclusions.

- RAN2 agrees some assistance information can be beneficial (e.g. periodicity, packet size). RAN2 assumes baseline could be TSCAI (pending SA2 conclusions), can discuss during WI phase whether something additional is needed on top of that. If any assistance information is needed, its definition should be standardized.

According to TR 23.700-60, SA2 concludes to provide burst periodicity to the gNB from the SMF.

- 8.4.1.2	AF Information Provisioning

PDU Set related assistance information provisioning by AF is supported for dynamic PCC. AF may provision one or more of the following PDU Set related assistance information to NEF/PCF during AF QoS request procedure:

-	PDU Set QoS parameters listed in clause 8.4.1.1.

-	Burst periodicity.

-	Description of Service Protocol: Indicates RTP/SRTP header type to be used for PDU Set Identification at UPF.

- 8.8	Conclusions for Key Issue #8

The following information, to be provided to the NG-RAN at PDU Session Establishment/Modification via an NGAP Message, is taken as baseline for normative work:

-	Periodicity for UL and DL traffic of the QoS Flow. In addition to integer periodicity values, non-integer values associated to, e.g. 15 FPS, 30 FPS, 45FPS, 60 FPS, 72 FPS, 90FPS, 120FPS, shall be supported. Such information shall be exchanged by re-using/extending the TSCAI/TSCAC definitions in clause 5.27.2.1 of TS 23.501 [2].

On Burst Arrival Time, no conclusion has been achieved in SA2. From our perspective, this parameter is very useful for the RAN to configure a CG configuration which well-matches with the XR service characteristics/requirements. Otherwise, the transmission latency of XR packets would be introduced inevitably. Since the UL packets are generated by the UE application layer, it is possible for a UE AS layer to obtain Burst Arrival Time information through its internal implementation. Since the gNB knows the XR periodicity, once the UE AS layer may know the time of the first XR packet, the gNB can know the arrival time of subsequent XR packets in advance if the UE AS can report the burst arrival time of this first XR packet, Accordingly, the gNB can use this information to configure CG for the subsequent traffic transmissions.

RAN2 assumes the UE to provide the gNB with the burst arrival time to configure a proper CG for XR service.

Regarding the burst size, SA2 does not conclude to provide it to the gNB either. Also, SA4 indicates in their reply LS that the gNB can not be aware of each burst size via the information from the CN even for DL. In our view, if the burst size information can not be provided by CN, it would be good if the UE would report it to the gNB to assist the efficient CG allocation.

Feedback enhancement

In the RAN2#120 meeting, RAN2 agrees to report delay information which is associated with data volume.

- RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g. what exactly is reported) and how to ensure this information is up-to-date e.g. considering UL scheduling delay.

In our view, the latency information would be the remaining time.

One issue associated is the granularity of the remaining time. We understand that the remaining time can be reported in the unit of PDU Set since the required operation and QoS-related characteristics (e.g. PSDB) are per PDU Set. When a UE multiplexes the delay information into a PUSCH, the latency information included would be calculated based on the boundary of this PUSCH occasion. On the other hand, if we assume that all PDU Sets in one Data burst arrive in a very short period of time and if resource allocation is performed per Data burst, the granularity of delay information can also be per data burst.

RAN2 considers that the remaining time is reported in the granularity of PDU Set or data burst.

When a UE multiplexes the remaining time into an uplink grant, the remaining time included would be calculated based on the boundary of this uplink grant.

Another issue is whether to extend BSR to carry the latency information. In legacy, BSR is designed to report data volume information per LCG. It is obvious that the reporting granularity of the latency and legacy BSR is not well-matched. Meanwhile, legacy BSR does not contain any latency-related information. Thus, extending the legacy BSR to contain the latency information would not save RAN2 work.

RAN2 considers to introduce a new MAC CE to at least contain the remaining time.

The following triggering criteria would be considered for remaining time reporting, e.g.

- A report is triggered when a fresh PDU Set/data burst is available;

- A report is triggered when the latency of a PDU Set/data burst buffered in the MAC layer approaches the PDSB.

In TR 38.835, the following enhancements related to capacity are captured and concluded to be implemented in the WI phase:

-	Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration;

-	Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE;

UCI is used to indicate the existence of unused CG PUSCH occasion(s). Once such information is indicated in advance, the gNB knows uplink resources are over-allocated and LCH(s) associated with these CG occasions would have no remaining data to be transmitted. If that is the case, once BSR is contained in the first CG PUSCH occasion and UCI indicates the unused CG PUSCH occasion existing from the third occasion, there is no need to report the BS information of the LCH(s) associated with these CGs. For example, assume two LCGs (e.g. LCG1 and LCG2) have triggered BSR and LCG1 associates with a CG configuration. If both two LCGs have data available for transmission when the MAC PDU containing the BSR is to be built, and if the UE knows the unused resource existing in the subsequent CG PUSCH occasions in advance, it would be good not to report the BS of LCG1 and the UE would report Short BSR.

In the case BSR is contained in a PUSCH together with UCI indicating the existence of unused CG PUSCH occasion(s) in advance, RAN2 is suggested to consider the BSR reporting enhancement.

CG enhancement

In TR 38.835, the following enhancements related to capacity are captured and concluded to be implemented in the WI phase:

-	Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration;

-	Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE;

We understand that multiple CG PUSCH transmission occasions in a period can be used to resolve two different issues. RAN2 would depend on RAN1 on how to configure/determine the occasions of each CG PUSCH.

- Issue 1: Efficient transmission of the XR traffic with large size.

- Issue 2: Non-integer period. One example is shown in Figure 1, to support 60fps for UL video, the periodicity is configured as 50ms instead of 50/3ms, and 3 CG PUSCH occasions are distributed in each 50ms.

Figure 1: Multiple CG PUSCH occasions in a 50ms-period (60fps)

In the case that multiple CG PUSCH transmission occasions are supported in each CG period of a CG configuration, how to configure/determine these occasions depends on RAN1’s progress.

In the case that multiple CG PUSCH transmission occasions are configured in each CG period of a CG configuration, another issue is how to calculate HPI for each CG occasion.

In R17, HPI is calculated based on either of the following formulas:

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

HARQ Process ID = [floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

When multiple CG PUSCH occasions are used in one period, the above formulas lead to the same HPI applying to those CG PUSCH occasions. RAN2 may consider either of the following to resolve this problem:

- Option 1: HPI of the first CG PUSCH occasion in one period is calculated similarly to R17. HPI of the other CG PUSCH occasion(s) in one period is an offset to the HPI of the first CG PUSCH occasion.

- Option 2: HPI of the first CG PUSCH occasion in one period is calculated similarly to R17. HPI of the other CG PUSCH occasion(s) in one period is selected by the UE implementation and indicated in CG-UCI.

- Option 3: For each CG PUSCH occasion, HPI is selected by the UE implementation and indicated in CG-UCI.

In the case that multiple CG PUSCH transmission occasions are supported in each CG period of a CG configuration, RAN2 discusses how to determine the HPI of each occasion.

Conclusion

We have the following observations:

Observation 1	TSCAI is designed to provide the gNB with the TSC characteristics which contain direction, periodicity and burst arrival time.

We have the following proposals:

Proposal 1	RAN2 assumes the UE to provide the gNB with the burst arrival time to configure a proper CG for XR service.

Proposal 2	RAN2 considers that the remaining time is reported in the granularity of PDU Set or data burst.

Proposal 3	When a UE multiplexes the remaining time into an uplink grant, the remaining time included would be calculated based on the boundary of this uplink grant.

Proposal 4	RAN2 considers to introduce a new MAC CE to at least contain the remaining time.

Proposal 5	In the case BSR is contained in a PUSCH together with UCI indicating the existence of unused CG PUSCH occasion(s) in advance, RAN2 is suggested to consider the BSR reporting enhancement.

Proposal 6	In the case that multiple CG PUSCH transmission occasions are supported in each CG period of a CG configuration, how to configure/determine these occasions depends on RAN1’s progress.

Proposal 7	In the case that multiple CG PUSCH transmission occasions are supported in each CG period of a CG configuration, RAN2 discusses how to determine the HPI of each occasion.

Reference

[1] RP-220285 New SID “Study on XR Enhancements for NR”

[2] TR 23.700-60 Study on XR (Extended Reality) and media services

[3] R2-2213229 TR 38.835 v040, Nokia (Rapporteur)

[4] RP-223502 New WID on XR Enhancements for NR