3GPP TSG RAN WG2 #121	R2-2300466

Meeting, February 27th – March 3rd, 2023

Source:	Panasonic

Title:		Neighbour cell signalling overhead reduction

Agenda Item:	8.7.4.1.2

Document for:	Discussion

# Introduction

The Rel.18 work item on Non-Terrestrial Network (NTN) [1] has been started with the following scope:

- Coverage enhancement

- NR-NTN deployment in above 10 GHz bands

- Network verified UE location

- NTN-TN and NTN-NTN mobility and service continuity enhancements

On “NTN-TN and NTN-NTN mobility and service continuity enhancements”, the study phase in RAN has been finished. The following is the scope of study in the working groups.

NTN-TN and NTN-NTN mobility and service continuity enhancements This work considers existing methods from NR TN as well as outcome of Rel-17 NR NTN WI outcome as baseline for NTN-TN mobility. - Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2,RAN3,RAN4] - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4] - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3] - Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4] - Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g. exchange of necessary information between gNBs. [RAN3]

In this document, we discuss cell reselection enhancements for earth moving cell.

# Discussion

1. Targeting reduced signalling overhead in earth-moving
    satellite constellations

Currently, information about neighbouring NTN cells and orbits is provided in a similar form as the information about the serving cell. The ntn-NeighCellConfigList and ntn-NeighCellConfigListExt loop structures below – being part of SIB19 – provide information about one particular neighbouring satellite per cycle, i.e. information about six neighbouring satellites means six loop cycles to be signalled.

Table 1: ntn-NeighCellConfigList and ntn-NeighCellConfigListExt loops being part of SIB19

As ntn-Config-r17 consists of several elements that do not change for the same orbit, there is a high degree of redundancy connected to this signalling scheme. That is especially burdensome in LEO earth-moving scenarios with satellites illuminating only small areas.
The elements hardly changing are mainly part of the ephemerisInfo-r17 sequence:

- Position relative to the reference satellite

- Velocity (remains constant)

- Orbit

Target of this document is to eliminate a large part of the aforementioned redundancy by only providing the differences, the delta information with reference to the parameters of the reference satellite (which might be the serving satellite) and the related reference orbit. At the same time, it will be possible to describe larger parts of a satellite network or even the entire network with limited signalling effort – especially in case of a regular network structure.

Observation 1:	With the current ephemerisInfo signalling – being part of SIB19 – redundant overhead is produced in conjunction with the indication of the attributes of multiple neighbouring satellites and orbits when done in the same way as for the “first”, the serving satellite up to now. These redundant elements are multiplied with increasing number of neighbouring satellites.

Proposal 1:	Find a neighbour cell/satellite signalling solution that is more efficient.

2. Background

2.1	SIB19 signalling hierarchy

Signalling overhead in the earth-moving cell case is mainly imposed by providing information about the relevant attributes of neighbouring cells (vulgo satellite beams) to the affected UEs, i.e. those that will experience the disappearance of the current cell/satellite beam and the arrival of the next one. In the end, all UEs being connected in the current cell will be affected by the reselection to a neighbouring cell.

For the description of the neighbouring cells, we need to identify first which cells might be directly relevant.

So far, the relevant information about the orbit and the position of a satellite within that orbit is provided via SIB19 and consists of

- The 3D position of the current satellite in space and its velocity components in three directions (six parameters altogether) and/or the description of the orbit with the six Keplerian elements

- The geographical area on the ground that is illuminated by a beam of the aforementioned satellite (centre of a circle and radius, i.e. three parameters (two-dimensional plane))

Now for the description of the neighbouring satellites, we need to identify first which satellites might be relevant. That is depicted with figure 1 below:

Figure 1: Typical full coverage scenario of neighbouring cells (green serving, all others neighbours)

We can see that the number of cells whose relevant parameters need to be signalled to the UE might be a multiple of three, e.g. the six identified in figure 1 above. That is feasible today already on the basis of existing signalling. But is that efficient?

The number of relevant neighbouring cells certainly depends on the orbit’s altitude and the number of satellites in that orbit – in turn, depending on their opening angle and herewith the size of their coverage areas. For the extreme case TR 38.821 envisages an illumination zone diameter of only 50 km being connected to an illumination period of some 7 seconds and below – see table 1 below:

| Cell Diameter Size (km) | UE Speed (km/hr) | Satellite Speed (km/s) | Time to HO (s) |
| --- | --- | --- | --- |
| 50 (lower bound) | +500 | 7.56 (NOTE 1) | 6.49 |
|  | -500 |  | 6.74 |
|  | +1200 |  | 6.33 |
|  | - 1200 |  | 6.92 |
|  | Neglected |  | 6.61 |
| 1000 (upper bound) | +500 |  | 129.89 |
|  | -500 |  | 134.75 |
|  | +1200 |  | 126.69 |
|  | - 1200 |  | 138.38 |
|  | Neglected |  | 132.28 |

Table 2: Time to handover (taken from ref. [2])

For the (quasi-)earth-fixed case the neighbour cell signalling is part of SIB19 – also carrying the serving cell description – consisting of the IE ntn-Config-r17 as well as the two parameters carrierFreq-r17 and physCellId-r17. ntn-Config-r17 is repeated for each neighbour cell that shall be described. But that sequence features several parameters that are hardly changing from satellite to satellite. One example is neighbouring satellites in the same orbit. There might be no need for describing the orbit again. Also the multi-parameter indication of the neighbouring satellite’s position is widely redundant. It is sufficient to e.g. describe the angle of distance between the neighbouring satellite in question and the currently serving satellite that we take as the reference satellite for our purposes here.

Savings regarding redundant signalling become relevant especially for LEO scenarios with small diameters of the covered area on the ground, e.g. the LEO-600 case of 50 km diameter of the area that the related satellite beam illuminates.

The signalling hierarchy of the neighbour cell elements in SIB19 looks like this

Table 3: ntn-NeighCellConfigList-r17 (part of SIB19)

Table 4: NTN-Config-r17

Table 5: EphemerisInfo-r17

The two options for signalling an orbit and a satellite position therein are illustrated with figure 2 below:

Figure 2: Keplerian and position/velocity signalling approaches

As illustrated above, a number of elements being part of the IE ephemerisInfo-r17 of each neighbouring cell are not always required, because they are representing a repetition of information provided already in connection with the ephemerisInfo-r17 of the serving cell (also called reference satellite in this context) already.

The elements hardly changing are mainly part of the ephemerisInfo-r17 sequence – see figure 3 above:

- Position relative to the reference satellite

- Velocity (remains constant)

- Orbit

Observation 2:	Redundant fields are mainly found in the ephemerisInfo IE. Hence that IE provides an opportunity for saving redundant overhead.

3. Providing the same information with reduced overhead

3.1	Overview

Instead of the repeated fields the minimum amount of needed information should be provided for describing position and extent of neighbouring orbits as well as position and speed of neighbouring satellites. That goal can be achieved by providing – as much as needed – information about the difference between the attributes of the reference satellite and reference orbit and those of neighbouring satellites and orbits.

Proposal 2:	Instead of the aforementioned redundant information, signalling for neighbouring satellites should be restricted to an indication of differences between the neighbouring satellite’s attributes and the attributes of the reference satellite.

How could such an approach be realised, i.e. where can the aforementioned delta approach be applied?

- Satellites in the same orbit as the reference satellite:

  - Angle between the reference satellite and neighbouring satellite (anchor for that angle indication is the geocentre)

- Satellites in a neighbouring orbit:

  - Angle between reference orbit (the orbit the reference satellite is in) and neighbouring orbit

  - Phase shift between reference satellite in reference orbit and neighbouring satellite in neighbouring orbit

  - AltitudeDiff-r18 indicates a (potential) difference in altitude of the (quasi-)parallel orbit

  - OrbitAngleDiff-r18 indicates a (potential) difference from perfect parallelism

In summary, a new IE structure – possibly called ephemerisDeltaInfo-r18 – would look like this:

Table 6: Basic structure of ephemerisDeltaInfo-r18

3.2	Description of neighbouring satellites in the same orbit as the serving satellite

Since the light green beam/cell (the serving one in figure 1) and the following light yellow ones all sit on the same orbit with identical distances between one another, one could probably save signalling overhead by simply indicating their spacing, i.e. the distance between e.g. cell A and cell a. Because the centre of the orbit is the centre of the earth, the aforementioned distance might be given in the form of the angle built by the two lines connecting the two satellites with the geocentre, e.g. 20 degrees in case of 18 satellites distributed equally over the related circular orbit if the satellite generates one cell which may consist of multiple SSB beams. That would be more metadata-overhead-efficient than the full description required for the first satellite.

Figure 3: Reference-satellite-to-neighbouring-satellite distance in the same orbit expressed as an angle anchored in the geocentre (unit and range might be the same as those of meanAnomaly-r17)

The corresponding element of ephemerisDeltaInfo-r18 would be satToSatPhaseShiftAngle-r18.

Observation 3:	Indicating a neighbouring satellite in the same orbit does not require all the information being part of the ephemerisInfo IE.

Proposal 3:	For indicating a neighbouring satellite in the same orbit as the reference satellite it is sufficient to indicate the distance between the two in the form of an angle being anchored in the geocentre.

3.3 Description of neighbouring orbits

Similar story for parallel orbits in the presence of knowledge regarding the first orbit. Also for the parallel neighbouring orbit only the angle between the two lines connecting the two orbits with the geocentre is sufficient (at positions on both orbits with smallest distance to each other). A parallel story to the angle between two satellites on the same orbit.

Figure 4: Reference-orbit-to-neighbouring-orbit distance (expressed as an angle with the geocentre as anchor point, unit and range might be the same as those of meanAnomaly-r17)

Orbits are quasi-parallel for the coverage extent of several neighbouring satellites. The corresponding element of ephemerisDeltaInfo-r18 would be orbitToOrbitAngle-r18.

Observation 4:	Indicating a neighbouring, quasi-parallel orbit does not require all the information being part of the ephemerisInfo IE.

Proposal 4:	For indicating a neighbouring, quasi-parallel orbit it is sufficient to indicate the distance between the two orbits in the form of an angle being anchored in the geocentre. The latter indication might be accompanied by information regarding a deviation from perfect parallelism.

3.4 Description of shifts of satellites towards each other in parallel orbits

Going further, the positioning of satellites on parallel orbits can be seen as a phase shift. The maximum shift is given when the satellite positions on parallel orbits are such that a line orthogonal to the two neighbouring orbits and hitting one satellite’s centre point reaches exactly the middle between two satellites on the neighbouring orbit. Here the metric might again be degrees. So in case the distance between two sats on the same orbit is 20°, the maximum phase shift would be +10°.

Figure 5: Phase shift of satellite in neighbouring orbit with reference to reference satellite in reference orbit (expressed as an angle with the geocentre as anchor point, unit and range the same as those of meanAnomaly-r17)

The corresponding element of ephemerisDeltaInfo-r18 would be satToSatPhaseShiftAngle-r18.

Observation 5:	Indicating a satellite in a neighbouring, quasi-parallel orbit does not require all the information being part of the ephemerisInfo IE.

Proposal 5:	For indicating a neighbouring satellite in a neighbouring, quasi-parallel orbit it is sufficient to indicate the phase shift between the neighbouring satellite in the neighbouring orbit with reference to the reference satellite in the reference orbit.

3.5 Additional elements and overview over all elements

In addition to the aforementioned delta approach, a few more attributes of LEO/MEO satellites might be indicated to the UEs.

- An indication of equivalence indicates the following – depending on the case:

The angle between two neighbouring satellites is the same as the one between the reference satellite and the first neighbouring satellite (applicable to all or a part of the satellites in the same orbit).

The angle between two neighbouring orbits is the same as the one between the reference orbit and the directly neighbouring orbit (applicable to all or a part of the orbits)

The phase shift between a neighbouring satellite in a neighbouring orbit and the directly neighbouring satellite in the same neighbouring orbit is twice as large as phase shift between the reference satellite in the reference orbit and the first neighbouring satellite in the neighbouring orbit (applicable to all or a part of the neighbouring satellites in the neighbouring orbit)

For that purpose an additional identity flag might be adopted.

- Indication of equidistant distribution of satellites in an orbit and indication of operational status

It should be possible to indicate that a particular number of satellites is equidistantly distributed over a certain orbit, e.g. 18 satellites are distributed equally over orbit A (with 20°distance between one another)

The latter indication might be accompanied by an indication regarding the operational status of the satellites in the related orbit realized with e.g. a bit image register referring to the reference satellite with its most significant bit. An example consists of 18 satellites in an orbit, of which 16 are in operation and 2 are currently out-of-order (binary representation): 11 1101 1111 1011 1111

On the aforementioned basis the entire new ephemerisDeltaInfo sequence would be this:

Table 7: Proposed Extended ephemerisDeltaInfo-r18

The fields of the ephemerisDeltaInfo-r18 IE are to be interpreted in the following way:

Table 8: Definition of fields of ephemerisDeltaInfo-r18

Observation 6:	Overhead might be further reduced for the indication of larger parts of satellite networks by covering regularities with the signalling.

Proposal 6:	RAN2 adopts the proposed additional entries in the ephemerisDeltaInfo-r18 IE.

1.5 Summary

In a fully regular set-up the description of all relevant neighbouring satellites would in the best case be covered with three angle indications, i.e. an angle expressing the distance between centre of two adjacent cells, an angle expressing the distance between two neighbouring orbits and an angle expressing the position of satellites in the neighbouring orbits – as outlined above.

Further information on neighbouring cells and orbits can be provided as described with this paper as well.

# 3. Conclusions

In this document, we discussed issues in conjunction with NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point.

Observation 1:	With the current ephemerisInfo signalling – being part of SIB19 – redundant overhead is produced in conjunction with the indication of the attributes of multiple neighbouring satellites and orbits when done in the same way as for the “first”, the serving satellite up to now. These redundant elements are multiplied with increasing number of neighbouring satellites.

Observation 2:	Redundant fields are mainly found in the ephemerisInfo IE. Hence that IE provides an opportunity for saving redundant overhead.

Observation 3:	Indicating a neighbouring satellite in the same orbit does not require all the information being part of the ephemerisInfo IE.

Observation 4:	Indicating a neighbouring, quasi-parallel orbit does not require all the information being part of the ephemerisInfo IE.

Observation 5:	Indicating a satellite in a neighbouring, quasi-parallel orbit does not require all the information being part of the ephemerisInfo IE.

Observation 6:	Overhead might be further reduced for the indication of larger parts of satellite networks by covering regularities with the signalling.

Proposal 1:	Find a neighbour cell/satellite signalling solution that is more efficient.

Proposal 2:	Instead of the aforementioned redundant information, signalling for neighbouring satellites should be restricted to an indication of differences between the neighbouring satellite’s attributes and the attributes of the reference satellite.

Proposal 3:	For indicating a neighbouring satellite in the same orbit as the reference satellite it is sufficient to indicate the distance between the two in the form of an angle being anchored in the geocentre.

Proposal 4:	For indicating a neighbouring, quasi-parallel orbit it is sufficient to indicate the distance between the two orbits in the form of an angle being anchored in the geocentre. The latter indication might be accompanied by information regarding a deviation from perfect parallelism.

Proposal 5:	For indicating a neighbouring satellite in a neighbouring, quasi-parallel orbit it is sufficient to indicate the phase shift between the neighbouring satellite in the neighbouring orbit with reference to the reference satellite in the reference orbit.

Proposal 6:	RAN2 adopts the proposed additional entries in the ephemerisDeltaInfo-r18 IE.

# 4. References

- RP-220953, “WID: NR NTN (Non-Terrestrial Networks) enhancements”, Thales

- TR38.821 V16.1.0, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Solutions for NR to support non-terrestrial networks (NTN) (Release 16)