3GPP TSG-RAN WG2 Meeting #121	R2-2300474

Athens, Greece, 27 February – 03 March 2023

Agenda item:	8.4.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On RRC Configuration for LTM

WID/SID:	NR_Mob_enh2-Core - Release 18

Document for:	Discussion and Decision

# Introduction

According to the agreements from RAN2#120 [1]:

- RAN2 assumes that LTM (intra DU and inter DU) is network-controlled mobility where the control is from the source, i.e. measurements (L1 measurements) are configured in the UE from the source Cell, and the decision to switch cell is by the source cell, and enhancements considered for LTM before cell switch, e.g. pre-synchronization, TA handling, target beam mgmt (to the extent it is supported) may be by the source cell. RAN2 understands that this may require cooperation source DU CU target DU and/or OAM coord. RAN2 don’t see any blocking issue to share information between DUs but the support of this is in RAN3 domain. RAN2 see no necessity for a direct inter-DU-interface to support this. - RAN2 to confirm that the CellGroupConfig IE is (mandatory) needed within an LTM candidate cell configuration. - The RadioBearerConfig IE can be optionally supported in an LTM candidate configuration - The MeasConfig IE can be optionally supported in an LTM candidate configuration. - The OtherConfig IE is not required to be part of the LTM candidate cell configuration. - The LTM candidate cell configuration should be designed as a To AddMod/ToRelease structure. - The LTM candidate cell configuration ASN.1 structure comprises at least a CellGroupConfig IE and a configuration ID. - FFS if it should be possible to perform SCell activation/deactivation (amongst SCells associated with the candidate configuration) simultaneously with L1 L2 mobility trigger MAC CE (if so, FFS how this is determined). - FFS how the UE determine the BWPs (for DL and UL) to be used upon the execution of L1/L2 inter-cell mobility

This document discusses the aspects of the RRC Reconfiguration and which Model should be used for the provision of the target cell configuration in LTM. Additionally, it discusses the bearer configuration handling as well as the BWP configuration of the target cell.

# 2	Target Cell RRC Configuration Provision in LTM

## 2.1		Discussion on RRC Models 1 and 2

In R2#119bis it was agreed that RAN2 will continue the discussion on the RRC models by focusing on Model 1 and Model 2 and stage-3 details [2]. Model 1 resembles CHO since each target configuration is modelled by a separate RRCReconfiguration message and it has the most flexibility in terms of target cell preparation. Additionally, since it is based on the full RRC Reconfiguration message, it works for all scenarios (i.e., intra-DU and inter-DU Intra CU), without complex implementations and big standardization impact. If a similar number of prepared cells (e.g., up to 8) as in CHO will be used for LTM, Model 1 may incur increased signalling overhead as the RRC Reconfiguration message size may be quite large in some cases (if many parameters are different for each given cell). Furthermore, the use of a full RRCReconfiguration message for the cell change may introduce increased interruption time (due to change of radio bearers and security reconfiguration during cell change). However, both drawbacks mentioned above can be avoided by restricting the number of configuration parameters in the reconfigurations that are allowed for LTM, by using delta configurations. Specifically, for the signalling overhead, with the use of delta configuration for a target cell (over initial source or over a reference RRCReconfiguration) the size of the target cell configuration will be significantly reduced. Furthermore, if delta configuration is coupled with restrictions in the allowed configurations (in terms which parameters/IEs can be modified), the size of the target cell configurations will be further reduced. Regarding the increased interruption time, the restrictions in the reconfigurations that are allowed in LTM will enable low interruption values.

Observation 1: Model 1 has several similarities to CHO since it involves a full RRCReconfiguration message. This is expected to make it simpler to be specified compared to Model 2.

Observation 2: Model 1 combined with some restrictions in the possible reconfiguration content can reduce the signaling overhead and interruption time to a level similar to Model 2.

On the other hand, in Model 2, the RRCReconfiguration message includes a list of CellGroupConfigs, each associated to a configuration of one LTM candidate target cell. Since DU generates CellGroupConfig fully, this approach allows DU-level flexibility for target cell configuration and works in both intra-DU and inter-DU scenarios. However, this approach is not as flexible as Model 1, wherein all IEs can be fully configurable. Additionally, delta configuration (both over the initial source and over reference RRCReconfiguration) is not as straightforward as it is in Model 1. Anyway, for each reconfiguration, the entire RRC message would need to be sent, not a single IE. Furthermore, in the preparation of the RRCReconfiguration, multiple IEs need to be provided to enable the UE to access the target cell (e.g., SIB1 of the target cell), which is not part of the CellGroupConfig. Finally, since each target cell’s configuration is largely covered in CellGroupConfig IE a big part of each target cell configuration will not be omitted anyway.

Observation 3: Application of delta configurations for CellGroupConfig IE in Model 2 is more complex compared to delta configuration for the entire RRCReconfiguration message (of Model 1).

Thus, we propose the following:

Proposal 1: Adopt Model 1 since it requires significantly less specification impact compared to Model 2. Define restrictions in the fields that can be configured to ensure acceptable signaling overhead and reduced interruption time for the LTM cell change procedure.

## 2.2	Discussion on release of candidate cell configuration

In the WID [3] the description of the LTM is:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction:

- Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3]

- Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1]

- […]

The above description contains two parts, namely, the configuration and maintenance of the configuration to be applied to enable mobility with reduced interruption time, and mechanisms to enable dynamic switch among the serving and target cells. To our understanding, this means that LTM has two operation modes, one where the configurations are released upon the execution of the LTM, and one where these configurations are maintained for dynamic switching. Whether the former or the latter is applied depends on the NW-originated configuration.

Observation 4: LTM is expected to have two operation modes, one where the configurations are released upon the execution of the LTM, and one where these are maintained for dynamic switching.

Observation 5: Whether the UE releases the configurations upon the execution of LTM or maintains them for further dynamic switching is a matter of configuration from the network.

Specifically, if the UE is moving towards new candidate cells, the new candidate cells should be added for LTM mobility and at the same time some of the earlier cells of LTM may not be needed for further mobility. Additionally, some parts of the configuration may become obsolete after some time (e.g., RACH preambles in case of CFRA). In such scenarios, CU can decide to release the candidate cell and update the UE configuration via RRC signaling.  But if the radio condition of one of the prepared candidate LTM cells are not suitable over long duration for dynamic switching, it may be preferred that source DU decides on the release of the candidate cell as it has the knowledge of radio condition of candidate cells based on L1 measurements. One further alternative could be to allow the UE to release the configuration considering various factors, e.g., time-based or depending on the recently accessed cell, etc.

Proposal 2: Release of the RRC configurations in the UE can be decided by the CU, the serving DU or the target DU. FFS on the interactions between the units for each of the case and scenarios wherein each case can be used.

Even though the delay in releasing the candidate cell is not time critical, the additional signalling to CU and RRC signalling procedure to release the candidate cell can be avoided if the DU can trigger the same actions, or if the releasing criteria are included as a part of the initial RRC configuration.

Proposal 3: The criteria for release of the configurations can be configured already within RRC message which provides the candidate cells RRC Configurations.

The actual release can be further triggered via MAC CE or an RRC message.

Proposal 4: Release of the configurations can be triggered by the DU using MAC CE if this is possible from the RRC model that it is used in LTM.

## 2.3	Discussion on Bearer Handling in LTM

One of the agreements in RAN2#120 [1] meeting is:

The RadioBearerConfig IE can be optionally supported in an LTM candidate configuration

In case the RadioBearerConfig IE is not part of the LTM candidate configuration, it is expected that after the cell switch execution UE re-uses the RadioBearerConfig of the source cell at the target cell.

This agreement enables the minimization of the radio signalling overhead related to the LTM candidate configuration update for dynamic switching scenario. In dynamic switching scenario, it is expected that UE maintains the LTM candidate configurations for a long time, during which the Radio Bearer Configuration can be updated multiple times. Each re-configuration will lead to an update of the LTM candidate configuration, which must be provided using an RRC Reconfiguration with significant signalling impact in both F1 and the radio interface. This can be avoided if the RadioBearerConfig for the target cell being taken from the source cell configuration.

Observation 6: If RadioBearerConfig IE is not part of the LTM candidate configuration, the need for interaction between the CU and the target DU can be avoided for the update of the configuration of the candidate cell, i.e. signalling can be reduced.

Proposal 5: RadioBearerConfig IE of the source cell should be taken out of the LTM configuration of the target cell, and if it is the same as that of the source, a common RadioBearerConfig IE can be used.

In case any of the candidate cells cannot accommodate all the bearers’ setup by the source cell (e.g., due to load), using the common RadioBearerConfig IE of the source cell would not work. In that case, the RadioBearerConfig IE for the candidate cell must be updated and signalled separately to the UE. It also should contain an indication that the RadioBearerConfig of the source cell should not be used by the target.

Observation 7: In case a candidate cell cannot accommodate new bearers (e.g., due to load), new interactions between the CU and the target DU are needed, thus causing signaling overhead.

Proposal 6: The radio signalling caused by DRB handling for candidate cell configurations should be avoided.

An improvement to avoid this radio signaling is to always allocate to the UE a candidate cell RadioBearerConfig IE that accommodates specific DRBs. After each radio bearer configuration update, the source DU can be informed whether the candidate DU is aligned with the update or not. This may require interactions of the CU with the DUs. At the time of cell switch, the source DU can indicate to the UE to use the candidate RadioBearerConfig IE or source RadioBearerConfig IE using the cell switch command without any RRC signalling.

Proposal 7: MAC CE for cell change can indicate the DRB to be used in the target cell.

## 2.4	BWP handling

One point raised in RAN2#119bis [2] was the BWP handling in LTM. Specifically, it was discussed how the UE should be made aware of which BWP to use in the DL and the UL upon the LTM triggering. For this, two options are available:

- Option 1: indicated directly within the candidate cell configuration for L1/L2 inter-cell mobility (e.g., by means of firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id)

- Option 2: indicated in the LTM triggering.

Option 1 means that the BWPs are provided in the RRC Configuration of the target cell, and this occursis provided in advance. In Option 2 the source cell decides the BWP to be used and indicates it to the target cell.

To our understanding Option 1 is more suitable since the BWP to be used in the target cell upon LTM is part of the target cell RRC configuration already. Option 2 can also be used since it has benefits; if the same BWP is used in the target and the source at the time of  LTM, the interruption time for LTM cell-change is reduced since BWP change is not needed in this case. In such case, the change of BWP in source node needs to be provided, at least for the RACH-less case to the target node.

Proposal 8: BWP to be used upon the LTM execution is either indicated directly within the candidate cell configuration for L1/L2 inter-cell mobility using RRC Configuration message or indicated in the LTM triggering using MAC CE.

Observation 8: Usage of the same BWP between the source and the target reduces the interruption time, but in case the BWP changes in the source, it requires the target cell to be notified, at least for the RACH-less scenario.

Proposal 9: If Option 2 is followed, the notification from the source to the target about the BWP used before the LTM is FFS.

# 3	Conclusion

This document has made the following observations:

Observation 1: Model 1 has several similarities to CHO since it involves a full RRCReconfiguration message. This is expected to make it simpler to be specified compared to Model 2.

Observation 2: Model 1 combined with some restrictions in the possible reconfiguration content can reduce the signaling overhead and interruption time to a level similar to Model 2.

Observation 3: Application of delta configurations for CellGroupConfig IE in Model 2 is more complex compared to delta configuration for the entire RRCReconfiguration message (of Model 1).

Observation 4: LTM is expected to have two operation modes, one where the configurations are released upon the execution of the LTM, and one where these are maintained for dynamic switching.

Observation 5: Whether the UE releases the configurations upon the execution of LTM or maintains them for further dynamic switching is a matter of configuration from the network.

Observation 6: If RadioBearerConfig IE is not part of the LTM candidate configuration, the need for interaction between the CU and the target DU can be avoided for the update of the configuration of the candidate cell, i.e. signalling can be reduced.

Observation 7: In case a candidate cell cannot accommodate new bearers (e.g., due to load), new interactions between the CU and the target DU are needed, thus causing signaling overhead.

Observation 8: Usage of the same BWP between the source and the target reduces the interruption time, but in case the BWP changes in the source, it requires the target cell to be notified, at least for the RACH-less scenario.

And proposed the following:

Proposal 1: Adopt Model 1 since it requires significantly less specification impact compared to Model 2. Define restrictions in the fields that can be configured to ensure acceptable signaling overhead and reduced interruption time for the LTM cell change procedure.

Proposal 2: Release of the RRC configurations in the UE can be decided by the CU, the serving DU or the target DU. FFS on the interactions between the units for each of the case and scenarios wherein each case can be used.

Proposal 3: The criteria for release of the configurations can be configured already within RRC message which provides the candidate cells RRC Configurations.

Proposal 4: Release of the configurations can be triggered by the DU using MAC CE if this is possible from the RRC model that it is used in LTM.

Proposal 5: RadioBearerConfig IE of the source cell should be taken out of the LTM configuration of the target cell, and if it is the same as that of the source, a common RadioBearerConfig IE can be used.

Proposal 6: The radio signalling caused by DRB handling for candidate cell configurations should be avoided.

Proposal 7: MAC CE for cell change can indicate the DRB to be used in the target cell.

Proposal 8: BWP to be used upon the LTM execution is either indicated directly within the candidate cell configuration for L1/L2 inter-cell mobility using RRC Configuration message or indicated in the LTM triggering using MAC CE.

Proposal 9: If Option 2 is followed, the notification from the source to the target about the BWP used before the LTM is FFS.

# References

[1] R2-2300001	RAN2#120 Meeting Report 3GPP TSG-RAN WG2 Meeting #121	Athens, Greece, 27 February – 03 March 2023

[2] R2-2211101	RAN2#119bis-e Meeting Report 3GPP TSG-RAN WG2 Meeting #120	Toulouse, France, 14 – 18 of November 2022

[3] RP-223520 	Revised WID on Further NR mobility enhancements RAN#98, December 2022