3GPP TSG-RAN WG2 Meeting #121	R2-2300504

# Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0615 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 38300 on IUC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2023-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The two terminogies “UE-A” and “UE-B” are used in the description texts on IUC. However, the two teminlgoies are not defined, and therefore should be avoided to be used. - Changes indicated in the RAN1 LS R1-2210591, In RAN1#111 meeting, RAN1 made the following agreement on cast type(s) for inter-UE coordination information transmission triggered by a condition other than explicit request reception in Scheme 1. Agreement: - The following working assumption is confirmed as follows: - Working Assumption (RAN1#107bis-e meeting): - For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set - FFS: Under which conditions groupcast/broadcast can be supported - Unicast for preferred resource set and non-preferred resource set - FFS: Under which conditions unicast can be supported From the above texts, GC and BC are supported for IUC information transmission comprising non-preferred resource set. This needs to be captured in the spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removal of terminologies “UE-A” and “UE-B”. - Added texts to capture the support of GC and BC for IUC information indicating non-preferred resource set. Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: SL IUC Inter-operability: If the network implemens the changes according to the CR, but not the UE, there is no inter-operability issue, since the changes concern only UE action. If the UE implemens the changes according to the CR but not the network, there is no inter-operability issue, since the changes concern only UE action. If one UE implements the changes according to the CR but not another UE, the both UEs may have different interpretation to IUC functions as captured in clause 16.9.8. This may affect IUC trigger and report between UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs may have different interpretation to IUC functions as captured in clause 16.9.8. This may affect IUC trigger and report between UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.9.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.331 CR 3809 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 16	Verticals Support

## 16.9	Sidelink

<<<<Skipped>>>>

### 16.9.8	Inter-UE Coordination (IUC)

The SL UE can support inter-UE coordination (IUC) in Mode 2, whereby a UE sends information about resources to a peer UE, which the peer UE then uses for resource (re)selection. The following schemes of inter-UE coordination are supported:

-	IUC scheme 1, where the IUC information sent from a UE to a peer UE is the preferred or non-preferred resources for the peer UE's transmission, and

-	IUC scheme 2, where the IUC information sent from a UE to a peer UE is the presence of expected/potential resource conflict on the resources indicated by the peer UE's SCI.

In scheme 1, the transmission of IUC information from a UE can be triggered by an explicit request from a peer UE, or by a condition at the UE. The UE determines the set of resources reserved by other UEs or slots where the UE, when it is the intended receiver of a peer UE, does not expect to perform SL reception from the peer UE due to half-duplex operation. The UE uses these resources as the set of non-preferred resources, or excludes these resources to determine a set of preferred resources and sends the preferred/non-preferred resources to the peer UE. The peer UE's resources for resource (re)selection can be based on both the peer UE's sensing results (if available) and the IUC information received from the UE, or it can be based only on IUC information received from the UE. For scheme 1, MAC CE and second-stage SCI or MAC CE only can be used to send IUC information. The transmission of the explicit request and reporting for IUC information in unicast manner is supported. For IUC information triggered by a condition other than explicit request reception, IUC information indicating preferred resource set is transmitted in unicast manner, while IUC information indicating non-preferred resource set is transmitted in unicast, groupcast or broadcast manner.

In scheme 2, a UE determines the expected/potential resource conflict within the resources indicated by a peer UE's SCI as either resources reserved by other UEs and identified by the UE as fully/partially overlapping with the resources indicated by the peer UE's SCI, or as slots where the UE is the intended receiver of the peer UE and does not expect to perform SL reception on those slots due to half-duplex operation. The peer UE uses the conflicting resources to determine the resources to be reselected and exclude the conflicting resources from the reselected resources. For scheme 2, PSFCH is used to send IUC information.

END OF FIRST CHANGE