3GPP TSG-RAN WG2 Meeting #121                                   R2-2300518

Athens, Greece, 27th Feb - 03rd Mar, 2023

Agenda item:		8.5.2.3

Source:				Samsung

Title:					PDU Set and PDCP Discard Handling

Document for:		Discussion & Decision

# 1 Introduction

In RAN2#119bis-e, following agreements were reached [1]:

- For UE transmitter, the PDCP discard should be performed per PDU set basis. - For UE transmitter, the PDCP discard is managed per SDU for PDU set, the PDCP entity discards all PDCP SDUs associated with the PDU set. - From RAN2 viewpoint, the following information would be useful for PDU set handling in UL and DL: - Semi-static information (from CN to RAN): At least PSER and PSDB. - Dynamic information: At least identifying which PDU belongs to which data burst/PDU set is also needed, including means to determine at least PDU set boundaries.

In RAN2#120, following agreements were made [2]:

Proposal2: RAN2 to discuss whether PDU/PDU set discard function is supported at lower layers (e.g. RLC entity) - Noted (need to wait for SA2 conclusions). - RAN2 to support timer-based discarding of UL transmit side of PDCP PDU/SDUs of a PDU set. FFS how this is modelled in PDCP specification, can be discussed in WI phase.

In this contribution, we further discuss the details on PDCP discard issue.

# 2 Discussion

### 2.1 PDU Set – PDU Set Integrated handling Indication (PSIHI) and discard operation

According to SA2 reply LS [3], it is confirmed that PDU Set Integrated handling Indication (PSIHI) is one of PDU Set QoS parameters and this parameter should be applied to all the PDU Sets of the same QoS flow.

SA2 agreed to define new 5G QoS parameters for PDU Set concept. The PDU Set comprises of one or more PDUs for which the following PDU Set QoS parameters are applicable: - PDU Set Delay Budget (PSDB) - PDU Set Error Rate (PSER) - PDU Set Integrated handling Indication (PSIHI) All the PDU Sets within one QoS flow should apply the same PSER, PSDB and PSIHI.

If PSIHI information impacts on discard operation at AS layer e.g., whether to enable PDU set based discard or not is dependent on PSIHI information, then the discard handling should be applied to all the PDU Sets in the same QoS flow since all the PDU Sets within the same QoS flow has the same PSIHI information. Given that gNB can obtain the PDU Set QoS parameters for UL/DL XR traffics through CN, and the discard configuration is in gNB remit, no additional AS impact is expected except that PDCP discard operation can be configured per PDU Set basis.

Observation 1: It is in the gNB remit to use PSIHI information when configuring PDCP discard per PDU Set basis.

### 2.2 PDU Set – PDCP Discard Considerations

RAN2 agreed that PDCP discard should be performed per PDU Set basis. In general, PDCP SDUs pertaining to the PDU Set may be received by PDCP from upper layers all at once or they may be received by PDCP with some time gaps in between (this may be dependent on the application and may be beyond RAN2 control).

Observation 2: In general, PDCP SDUs pertaining to the PDU Set may be received by PDCP from upper layers all at once or they may be received by PDCP with some time gaps in between (this may be dependent on the application and may be beyond RAN2 control).

PDCP discardTimer Operation

Another issue concerns how the PDCP discardTimer is operated. There seems the following possible options:

Option 1: PDCP discardTimer is operated per PDCP SDU as in the legacy

This is in line with the legacy specification where a discardTimer is started on arrival of PDCP SDU at the PDCP layer. However, since PDCP discard is to be performed per PDU Set basis, only one timer pertaining to PDU Set is actually effective and there would be undesired redundancy and complexity for maintaining and operating multiple timers for a given PDU Set. Moreover, if we consider all SDUs of the PDU Set arrive at same time, redundancy as described is quite obvious. If there is gap between arrivals of the SDUs of the PDU Set at PDCP, the approach would always cause discard of the PDU Set when the discardTimer of the first SDU of the PDU set expires. That is, not all SDUs of the PDU Set get equal processing time in accordance with PSDB and related discard timer configuration.

Option 2: One PDCP discardTimer is operated per PDCP PDU set

This is a new behavior and it needs to be specified that only one discardTimer is operated for the PDU Set. However, this can be achieved with a simple specification change, just for an example, “At the reception of the first PDCP SDU of the PDU set from upper layers, start the discardTimer associated with this PDU set”. If we consider all SDUs of the PDU Set arrive at the same time, the approach seems to have no redundancy unlike option 1. If there is gap between arrivals of the SDUs of the PDU Set at PDCP, one timer should still suffice.

Option 3: Configurable by network to use PDCP discardTimer per PDCP SDU or per PDCP PDU Set

This option provides greater flexibility and control to network to configure one of the option 1 or option 2 as required.

Proposal 1: RAN2 is kindly asked to discuss and decide which option is selected for PDCP discardTimer operation in the uplink

- Option 1: PDCP discardTimer is operated per PDCP SDU as in the legacy

- Option 2: One PDCP discardTimer is operated per PDCP PDU set

- Option 3: Configurable by network to use PDCP discardTimer per PDCP SDU or per PDCP PDU Set

Discard enhancements on PDCP and/or RLC?

In the legacy when a PDCP SDU is discarded, PDCP indicates the discard to the RLC entity if the corresponding PDCP data PDU is already submitted to RLC. RLC procedure is specified in TS 38.322 as below:

## 5.4 SDU discard procedures When indicated from upper layer (i.e. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

It is clear that in many cases RLC may not be able to perform discard. Question is whether RLC procedure/protocol needs to be enhanced to achieve discard always or maximum possible times. Primarily, we understand if RLC transmission is already undertaken it is not easy to achieve discard lest it will introduce RLC SN gap and there may be undesired complexity for Tx and Rx RLC entities. It seems reasonable to avoid such large specification impact and efforts involved.

Observation 3: Discard operation at RLC layer is not always achievable. Discard enhancements on RLC may introduce RLC SN gap, and there may be undesired complexity for Tx and Rx RLC entities. It seems reasonable to avoid such large specification impact and efforts involved.

Proposal 2: Discard enhancements for PDU set should be limited to the PDCP layer and no enhancements are pursued for RLC layer.

### 2.3 PDU Set – Rx Side Considerations

We further explore the impact of the PDU Set based PDCP discard on the receiver side PDCP. It can be noted that in spite of discard of PDU Set at transmitter side PDCP, all constituent PDUs of the PDU Set may not be discarded e.g. when some of the constituent PDUs are already transmitted or assigned sequence number. That is, it is possible that receiver side PDCP layer may receive incomplete PDU Set.

Observation 4: It is possible that receiver side PDCP may receive incomplete PDU Set, when PDU Set based PDCP discard is carried out at the transmitter side PDCP.

In the reply LS to SA2 [4], SA4 share their view on the dependency between IP packets that make up PDU Sets (e.g. a frame /slice), including examples that provide some insight into possible dependencies, from none to a dependency that the information unit is prefix-dependent to the case that if any piece is lost of an information unit, the entire unit is useless. It is mentioned that different application media layer mappings and receiver implementations can be addressed by the PDU Set concept and the media/application layer should be able to configure the appropriate handling.

Observation 5: Different application media layer mappings and receiver implementations can be addressed by the PDU Set concept and the media/application layer should be able to configure the appropriate handling.

PDCP discard can be performed at the transmitter side PDCP based on the PSIHI. Emphatically, this is equally true and applicable for the receiver side PDCP operation, wherein application layer can have the following cases:

- Case A: Application layer can still recover the part of the information unit, even when PDU Set could not be completely received.

- In this case, the received PDUs of the PDU Set should be delivered to the application layer

- Case B: Application layer needs all the PDUs of the PDU Set to receive the information unit. For example, application can process information unit only when all constituent PDUs of the PDU Set are received. Incompletely received PDU Set is not useful and rather may add to undesired processing burden and complexity.

- In this case, the received PDUs of the incompletely received PDU Set should not be delivered to the application layer

That is, whether all PDUs are needed for the usage of PDU Set by application layer (PDU Set Integrated Handling Indication) [5] determines the delivery approach of the received PDCP PDUs.

Proposal 3: RAN2 is kindly asked to discuss potential enhancement on receiver side PDCP to handle and deliver received PDUs to application layer considering following:

- If all PDUs are needed for the usage of PDU Set by application layer, the PDCP does not deliver the received PDUs of the incompletely received PDU Set to the application.

- If all PDUs are not needed for the usage of PDU Set by application layer, PDCP delivers the received PDUs of the incompletely received PDU Set to the application.

- It is configurable to the receiver PDCP entity whether a) or b) is required.

### 2.4 PDU Set – Identification Signalling Considerations

For the purpose of PDU Set integrated handling including the cases as discussed in section 2.2 and 2.3, PDU Set identification information (e.g. start of PDU Set, end of PDU Set, sequence number of PDU in PDU Set) is essential.

From PDCP perspective, it is not really needed to replicate the sequence number for the constituent PDCP PDUs for the PDU Set. On the contrary, the existing PDCP SN can be reused and be further complimented with additional embedded signalling information to indicate start of the PDU Set and end of the PDU Set as part of the PDCP header.

For instance, a 2-bit bitmap in the PDCP header of the PDU can represent 4 code-points viz.

- Start PDU of PDU Set,

- End PDU of PDU Set,

- In-between PDU of PDU set, and

- Both start & end PDU of the PDU Set.

With the PDU Set identification information, PDCP operation can be facilitated e.g. receiver side PDCP can easily identify whether the PDU Set is completely received, or is incompletely received. Notably, PDU set identification information i.e. PDCP header usage can be applicable for both UL and DL directions and may be configured independently of the PDU Set based PDCP discard which is for UL only.

Observation 6: With the PDU Set identification information signalling, PDCP operation can be facilitated e.g. receiver side PDCP can easily identify whether the PDU Set is completely received, or is incompletely received. Existing PDCP SN can be reused and be further complimented with additional embedded signalling information to indicate start PDU, in-between PDU and end PDU of the PDU Set as part of the PDCP header.

Proposal 4: RAN2 is kindly asked to discuss the PDU Set identification information signalling to facilitate the receiver PDCP operation.

# 3 Conclusion

Based on discussion in section 2, the following observations are made:

Observation 1: It is in the gNB remit to use PSIHI information when configuring PDCP discard per PDU Set basis.

Observation 2: In general, PDCP SDUs pertaining to the PDU Set may be received by PDCP from upper layers all at once or they may be received by PDCP with some time gaps in between (this may be dependent on the application and may be beyond RAN2 control).

Observation 3: Discard operation at RLC layer is not always achievable. Discard enhancements on RLC may introduce RLC SN gap, and there may be undesired complexity for Tx and Rx RLC entities. It seems reasonable to avoid such large specification impact and efforts involved.

Observation 4: It is possible that receiver side PDCP may receive incomplete PDU Set, when PDU Set based PDCP discard is carried out at the transmitter side PDCP.

Observation 5: Different application media layer mappings and receiver implementations can be addressed by the PDU Set concept and the media/application layer should be able to configure the appropriate handling.

Observation 6: With the PDU Set identification information signalling, PDCP operation can be facilitated e.g. receiver side PDCP can easily identify whether the PDU Set is completely received, or is incompletely received. Existing PDCP SN can be reused and be further complimented with additional embedded signalling information to indicate start PDU, in-between PDU and end PDU of the PDU Set as part of the PDCP header.

Request RAN2 to discuss and agree to the following proposals:

Proposal 1: RAN2 is kindly asked to discuss and decide which option is selected for PDCP discardTimer operation in the uplink

- Option 1: PDCP discardTimer is operated per PDCP SDU as in the legacy

- Option 2: One PDCP discardTimer is operated per PDCP PDU set

- Option 3: Configurable by network to use PDCP discardTimer per PDCP SDU or per PDCP PDU Set

Proposal 2: Discard enhancements for PDU set should be limited to the PDCP layer and no enhancements are pursued for RLC layer.

Proposal 3: RAN2 is kindly asked to discuss potential enhancement on receiver side PDCP to handle and deliver received PDUs to application layer considering following:

- If all PDUs are needed for the usage of PDU Set by application layer, the PDCP does not deliver the received PDUs of the incompletely received PDU Set to the application.

- If all PDUs are not needed for the usage of PDU Set by application layer, PDCP delivers the received PDUs of the incompletely received PDU Set to the application.

- It is configurable to the receiver PDCP entity whether a) or b) is required.

Proposal 4: RAN2 is kindly asked to discuss the PDU Set identification information signalling to facilitate the receiver PDCP operation.

# 4 Reference

[1] RAN2#119bis-e meeting report v1

[2] RAN2#120 meeting report

[3] R2-2300071 (S2-2301378) Reply LS on PDU Set Handling

[4] R2-2206969 (S4-220505) LS Reply on QoS support with PDU Set granularity

[5] TR 23700-60 v18.0.0 Study on XR (Extended Reality) and media services