3GPP TSG-RAN WG2 Meeting #121	R2-2300522

# Athens, Greece, 27th February – 3rd March 2023

Agenda Item:	8.10.2 FDM solution enhancements

Source:	Ericsson

Title:	More granular FDM indications

Document for:	Discussion, Decision

# 1	Introduction

In this document we discuss further details for the more granular FDM indication.

# 2	Discussion

## 2.1	Down selection of more granular FDM indication

in the below email discussion, companies have been invited to further down select among the options

[Post120][652][IDC] Further details of FDM solution (Huawei)

Scope:

- Comparison and down-selection of Solution 1, 2 or 2a based on ASN.1 details (granularity for bandwidth, e.g. PRB, RBG, explicit Bandwidth, etc).

  - Option 1: Central frequency + Bandwidth of the actual affected frequency range

  - Option 2: Starting frequency + Ending frequency of the actual affected frequency range

  - Option 2a: starting frequency + Bandwidth of the actual affected frequency range

- Identify the impact of MR-DC, e.g. whether SN can configure IDC for SN (including both FDM and TDM), the coordination granularity of inter-node message, per CG pattern (TDM);

- Signalling details of FDM, e.g. how to configure, how to report..

Intended outcome: Report to next meeting (with Text proposal)

Deadline: Long

As described in [1], the three options in terms of the Signalling overhead based on the ASN.1 structure is summarised in the below table

| Option | Signalling component (with # of bits in bracket) | Overhead (bit) |
| --- | --- | --- |
| 1 | Centre Freq - ARFCN-ValueNR (22) + affected BW (4) | 26 |
| 2 | Starting Freq - ARFCN-ValueNR (22) + Ending Freq - ARFCN-ValueNR (22) | 44 |
| 2a | Starting Freq ARFCN-ValueNR (22) + affected BW (4) | 26 |

Either Option 1 or 2a has the equal signalling overhead. They are somewhat identical. Therefore, either of both options is fine to us. In addition, even for option 2, the signalling overhead is not a big issue since it only affects the dedicated signalling. Given there is majority support to adopt Option 1 in the email discussion 652 [1]. We are fine to adopt Option 1.

RAN2 to select one of option 1 for more granular FDM indications.

A likely gNB-approach is that it configures the BWPs that the gNB has available as candidates for the UE. With either option 1, we assume instead that the gNB configures the candidate frequency ranges explicitly that covers the frequencies interesting to the gNB. This would be beneficial to configure the UE to measure the gNB concerned/interested frequency regions.

With option 1, the gNB configures candidate frequency ranges. The UE reports IDC issues within those ranges.

In addition, it is worth to note that it is beneficial to discuss the option on more granular IDC indication and the option on gNB configuration of IDC indication together.  Thus, the same option shall be adopted for both IDC indication report and gNB configuration to enable more granular IDC indication.

It is beneficial to discuss the option on more granular IDC indication and the option on gNB configuration of IDC indication together.

In [1], the BW value range is suggested as the below

affectedBandwidth-r18           ENUMERATED {mhz5, mhz10, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz200, mhz300, mhz400, FFS_spare_values}

However, it is raised by companies that the proposed BW values for Option 1 or Option 2a need to be further discussed. Companies have indicated that even lower bandwidth values may be needed due to reasons

1) a typical BLE channel is 2MHz, so the expected affected BW due to adjacent channel interference should be even less than that.

2) beneficial to have lower values to cover all cases including Wifi, BT, GNSS etc.

We agree with the above reasonings. The designed granularities shall be sufficient to cover all possible cases. In addition, UE reports IDC issues in dedicated signalling, therefore, the increased signalling overhead due to finer granularities are acceptable.

Therefore, we suggest RAN2 to adopt lower BW values than 5MHz.

Adopt 2 MHz as the smallest BW value.

## 2.2	Simultaneous configuration of legacy and new granularity

RAN2 is now going to increase the granularity of the FDM indications, e.g., by indicating affected frequency ranges. We expect the RRC spec to end up looking something like the following where we have the legacy candidateServingFreqListNR and a list of entities of the new granularity:

OtherConfig-v1610 ::=                   SEQUENCE {

idc-AssistanceConfig-r16         SetupRelease {IDC-AssistanceConfig-r16}  OPTIONAL

...

}

IDC-AssistanceConfig-r16 ::=    SEQUENCE {

candidateServingFreqListNR-r16  CandidateServingFreqListNR-r16         OPTIONAL, -- Need R

...,

candidateFreqRangeListNR-r18			CandidateFreqRangeListNR-r18					OPTIONAL

}

CandidateServingFreqListNR-r16 ::= SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF ARFCN-ValueNR

CandidateFreqRangeListNR-r18 ::= SEQUENCE (SIZE (1..maxFreqRangesIDC-r18)) OF <something>

candidateServingFreqListNR Indicates for each candidate NR serving cells, the center frequency around which UE is requested to report IDC issues.

candidateFreqRangeListNR Indicates candidate frequency ranges which UE is requested to report IDC issues within.

One question is if the network could mix granularities? I.e. should the network be allowed to configure some ARFCNs in CandidateServingFreqListNR and at the same time some new candidates frequency ranges?

We believe that it will cause additional complexities if we do allow for the combination. If the network has enabled the higher granularity the network doesn’t need the ARFCNs since those are of lower granularity. The network would likely simply configure candidate frequency ranges to match the BWPs that the network has available. And if the UE has issues on any of them, the gNB would avoid using those BWPs.

Simultaneous configuration of legacy candidate ARFCNs and the new candidate frequency ranges is not supported.

## 2.3	Combinations of new granularity / IMD interference indication

Existing spec allows the gNB to configure candidate ARFCNs. The UE would report either individual ARFCNs which are problematic for the UE and the gNB would address them, or the UE would report combinations of ARFCNs.

One question is if RAN2 should support combinations of the new frequency ranges? For example, if RAN2 adopts the approach where the UE indicates problematic frequency ranges; should the UE be allowed to indicate problematic combinations of frequency ranges?

Spontaneously this may seem complex. But it may actually simplify the feature. The alternative would be that we rely on the legacy approach using ARFCNs for indicating problematic combinations, while we use the new granularity for indicating individually problematic frequencies (e.g. by frequency ranges or BWPs). This would then result in that we anyway end up with simultaneous combination of ARFCNs and the new granularity.

One can of course imagine that we would say that the legacy ARFCN-approach is used for combinations, and the new granularity used for individual frequency-entities, but that we believe would be even more complicated to specify.

In addition, the indication of combination of frequency ranges serves the purpose of indicating IMD inference from simultaneous Tx in EN-DC or NR-DC to non-3GPP RAT. According to the below agreement made in RAN2#119

The IMD interference from simultaneous Tx in EN-DC to non-3GPP should be considered for the FDM enhancement in Rel.18.

The IMD interference from simultaneous Tx in NR-DC to non-3GPP should be considered for the FDM enhancement in Rel.18.

Note: the solution (on freq granularity) for adjacent can be reused for IMD, we will not invent new solution on freq granularity for IMD. FFS on signalling details.

It is clear that the solution (on freq granularity) for adjacent channel interference (between NR Stand Alone (SA) or MN/SN of NR-DC and non-3GPP RAT) can be directly reused for IMD interference indication, after the option for adjacent channel interference is settled down.

The UE reuses the solution for adjacent channel interference for indicating IMD interference without further enhancement.

## 2.4	Victim system type

In existing IDC indication, the UE indicates the victim system type. This is indicated with this IE:

VictimSystemType-r16 ::=    SEQUENCE {

gps-r16                     ENUMERATED {true}        OPTIONAL,

glonass-r16                 ENUMERATED {true}        OPTIONAL,

bds-r16                     ENUMERATED {true}        OPTIONAL,

galileo-r16                 ENUMERATED {true}        OPTIONAL,

navIC-r16                   ENUMERATED {true}        OPTIONAL,

wlan-r16                    ENUMERATED {true}        OPTIONAL,

bluetooth-r16               ENUMERATED {true}        OPTIONAL,

...

}

The WID does not mention anything about extending the victim system type hence we assume that the legacy victim system types are applicable also in Rel-18.

RAN2 confirms that the legacy victim system types apply.

## 2.5	Interference direction

In the existing IDC indication, the UE indicates the interference direction, this is indicated by this field:

AffectedCarrierFreq-r16 ::=     SEQUENCE {

carrierFreq-r16                 ARFCN-ValueNR,

interferenceDirection-r16       ENUMERATED {nr, other, both, spare}

}

In [1], another issue on interference direction is discussed. Considering that the actually affected frequency range for the interference from NR TX to non-3GPP RX and the interference from non-3GPP TX to NR RX could be different as shown in Figure 5, the Rapporteur has suggested that the UE should be allowed to report two different frequency range for each candidate serving frequency.

Figure 1/Figure 5 – Different affected frequency range for the interference from NR TX to non-3GPP RX and the interference from non-3GPP TX to NR RX in [1]

Our view is that the intention seems to be aligned with the legacy FDM framework, where UE can indicate interferenceDirection. Therefore, the intention is ok to let UE to report the interferenceDirection.

However, we are uncertain on the meaning of “report two different frequency range”. UE only reports IDC and the affected frequency regions. For the same frequency region which is configured by the gNB, UE can not report two times (with slight differences on the affected bandwidtch) in the same report message. Since the reported affected bandwidtchs for the same frequency candidate which is configured for IDC measurement by the gNB are overlapping, the gain may be marginal. For each frequency region, UE can just reuse the legacy framework to indicate the detected interference direction (e.g., ‘NR’, ‘other’, or ‘both’).

We see no reason to change this, and we propose:

Support the legacy interference direction indication without additional enhancement.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	It is beneficial to discuss the option on more granular IDC indication and the option on gNB configuration of IDC indication together.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to select one of option 1 for more granular FDM indications.

Proposal 2	With option 1, the gNB configures candidate frequency ranges. The UE reports IDC issues within those ranges.

Proposal 3	Adopt 2 MHz as the smallest BW value.

Proposal 4	Simultaneous configuration of legacy candidate ARFCNs and the new candidate frequency ranges is not supported.

Proposal 5	The UE reuses the solution for adjacent channel interference for indicating IMD interference without further enhancement.

Proposal 6	RAN2 confirms that the legacy victim system types apply.

Proposal 7	Support the legacy interference direction indication without additional enhancement.

# 4 References

[1] R2-22xxxxx, summary of [Post120][652][IDC] Further details of FDM solution (Huawei), Huawei