3GPP TSG-RAN WG2 Meeting #121                                   R2-2300525

Athens, Greece, 27th Feb – 03rd Mar 2023

Agenda item:		8.11.2.1

Source:				Samsung

Title:					Discussion on PTM configuration aspects and mobility

Document for:		Discussion & Decision

# 1 Introduction

In RAN2#120 meeting, the following agreements were made for multicast reception in RRC_INACTIVE [1]:

We will have a mixed approach and we start with the following: - When NW configures UE to continue the multicast reception in INACTIVE state, NW provides the PTM configuration for the activated multicast session via the RRC dedicated signalling, at least for the serving cell (FFS other cases). - MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB. FFS session status change and other indications. - We assume that the UE can only receive multicast service after it joined the session. - FFS whether MCCH configuration is initially provided to the UE via dedicated signalling.

This contribution discusses the issues and further details for PTM configuration aspects and mobility.

# 2 Discussion

### 2.1 PTM configuration for UEs receiving multicast in RRC_INACTIVE

UEs may need to be transitioned to RRC_INACTIVE in overload/congestion situation, and can continue “certain” multicast session(s). Notably, UEs may have the multicast session(s) either activated (MRB configuration available) or deactivated (MRB configuration released) while in RRC_CONNECTED [2]. Further, these UEs are required to be configured to receive multicast in RRC_INACTIVE. Following options are considerable:

Dedicated signalling approaches:

Option 1A: RRC Release with suspendConfig provides the PTM configuration to be used in RRC_INACTIVE (includes both activated and deactivated multicast MRBs). Configuration may include only those MRBs which are to be continued to RRC_INACTIVE.

Option 1B: Pre-configured configuration implies UE may be indicated PTM configuration for MRBs in RRC reconfiguration message from which configuration may fully/ partially be continued to RRC_INACTIVE and UE preserves them. It may be possible to indicate which all MRBs can be continued to RRC_INACTIVE.

Broadcast signalling approaches:

Option 2A: SIB+MCCH may provide multicast session configuration to RRC_INACTIVE UEs, like in legacy for broadcast session.

Option 2B: MCCH configuration through dedicated signalling implies UE receives MCCH configuration through RRC reconfiguration in the RRC_CONNECTED and then utilizes it to set up MCCH channel and receive PTM configuration.

Analysis:

We understand Option 1A is efficient and sufficient with providing specific PTM configuration for relevant multicast MRBs at state transition. Comparatively, Option 1B seems to have larger spec impacts as

- UEs need to be configured in advance with “suitable for RRC_INACTIVE” multicast session configuration information - This may be a bit complex as relevant configuration parameters may also need to be indicated for specific RRC state(s) applicability

- UEs may also need to store and preserve certain multicast session configurations in spite of MRB release due to session deactivation, in order to support potential future transition to RRC_INACTIVE

Proposal 1: RRCRelease with suspendConfig provides the PTM configuration for multicast MRBs to be received in RRC_INACTIVE. Pre-configuration with RRC reconfiguration message is not adopted.

Whereas Option 2A has a major drawback:

- Unauthorized access and non-standalone approach: With this approach, UEs in RRC_INACTIVE cannot receive new multicast session configuration i.e. UEs need to “join” the multicast session and also need to be informed by network “which UEs” can receive “which multicast sessions” in RRC_INACTIVE. Allowing UEs in RRC_INACTIVE to read MCCH without joining multicast sessions causes unauthorised access, which is highly problematic.

Given the agreement to progress with a mixed approach (i.e. dedicated + MCCH), this drawback should be overcome. We think MCCH based indication for a change of PTM configuration and providing only the changed configuration parameters (delta signalling) for multicast session through MCCH can address this issue. This ensures that the UE must have joined multicast session and received configuration in RRC_CONNECTED, while UE can receive the update of PTM configuration in RRC_IDLE without transiting back to RRC_CONNECTED.

Further, to link the multicast configuration in dedicated signalling and updated configuration in MCCH for same multicast session, it is relevant to define a ‘multicast configuration identity’ which is present in dedicated signalling as well as MCCH. The advantage would be that such identity can be signalling efficient and be of sufficient size as needed for network’s deployment of multicast (unlike G-RNTI/G-CS-RNTI). Moreover, this mapping ensures that access to each session is available to authorized UE only,

Observation 1: RAN2 should consider the issue of unauthorized access to configuration of multicast sessions through MCCH.

Proposal 2: RAN2 to agree:

- UE joins multicast session and receives configuration in RRC_CONNECTED, while UE can receive the update of PTM configuration in RRC_INACTIVE through MCCH without transiting back to RRC_CONNECTED

- MCCH provides the indication for a change of PTM configuration and only the changed configuration parameters for multicast session (i.e. a delta signalling)

- Multicast configuration identity is defined to provide a mapping between multicast session configuration in the dedicated signalling and updated session configuration in the MCCH

Option 2B tries to address unauthorized access concerns with the PTM configuration provided over MCCH. Receiving MCCH configuration only through dedicated signalling ensures only authorised UEs can access the MCCH. However, there are some concerns with this approach.

- Need new MCCH channel: A new MCCH channel is required which is only accessible to authorised UEs that receive MCCH configuration through dedicated signalling. This implies significant specification impact and complexity.

- No dynamic configuration of MCCH channel: With this approach, RRC_INACTIVE UEs are assumed to always utilizing MCCH configuration received in RRC_CONNECTED and stored in a static manner. That is, network has no control or adaptability to reconfigure MCCH even if situation demands (unlike legacy which uses SIB based MCCH configuration). Moreover MCCH channel configuration is common to all the multicast services and all UEs, unlike PTM configuration which may be updated for a given multicast service only. Therefore, any change of the configuration of MCCH channel affects all the UEs receiving any of the services in RRC_INACTIVE and forces them all to go RRC_CONNECTED at once.

- Area Scope of MCCH: In legacy MCCH has been specified for a single cell. It will be a new complexity when UE moves across the cells or area scope for MCCH has to be redefined.

- Authorization for each multicast session is missing: MCCH channel is accessible for configurations of all other multicast sessions even by a UE which has joined only one multicast session. That is, such UE can still access PTM configuration for all other multicast sessions which are not even joined by it.

Option 2B introduces undesired specification impact and effort. The advantage of reusing legacy MCCH (option 2A) is lost.

Observation 2: MCCH configuration through dedicated signaling will bring the need for new MCCH channel, lack of dynamism of configuration update for MCCH, large complex specification impacts and lack of authroized access for each multicast session.

Proposal 3: RAN2 to not consider providing MCCH configuration through dedicated signalling

### 2.2 PTM configuration parameters

Broadly speaking, like the dedicated configuration for multicast, RRCRelease with suspendConfig can carry MBS radio bearer config, MAC config and PHY config for multicast reception in RRC_INACTIVE. These may exclude certain parameters which are specific to dedicated configuration used only for RRC_CONNECTED e.g. HARQ feedback, HARQ retransmission, CSI/SRS configuration, TA timer etc. Additionally, RRCRelease with suspendConfig may provide neighbour cells multicast configuration e.g., whether the neighbour cells support same multicast configurations as the serving cell.

Proposal 4: RRCRelease with suspendConfig can carry MBS radio bearer config, MAC config and PHY config for multicast reception in RRC_INACTIVE.

- These exclude certain parameters which are specific to dedicated configuration used only for RRC_CONNECTED.

- Additionally, RRCRelease with suspendConfig may provide neighbour cells multicast configuration e.g., whether the neighbour cells support same multicast configurations as the serving cell.

### 2.3 BWP/CFR aspects for multicast in RRC_INACTIVE

A UE in RRC_CONNECTED receives unicast transmission in an active Bandwidth Part (BWP) and multicast transmission is received over common frequency resources (CFR) which is associated with and confined within the unicast active BWP. For the UE in RRC_INACTIVE, it is relevant to receive same multicast transmission (i.e. in the MBS CFR), however, monitoring for the entire active BWP may not be necessary and not power efficient. Another possibility is to consider the linking of MBS CFR to the initial BWP. However, it is not guaranteed that initial BWP is overlapped with active BWP or MBS CFR. Therefore, it may be more relevant to consider to define a BWP same as MBS CFR for the multicast reception operation by UE in RRC_INACTIVE. UE can receive GC-PDCCH and GC-PDSCH for multicast over the BWP. RAN2 should, therefore, consult with RAN1.

Proposal 5: RAN2 to consider to define a BWP same as MBS CFR for multicast reception operation by UE in RRC_INACTIVE. RAN2 to send LS to RAN1.

### 2.4 DRX configuration for multicast in RRC_INACTIVE

DRX mechanism is required for multicast reception in RRC_INACTIVE to enable power saving. As an UE in RRC_INACTIVE needs to share the multicast transmission with RRC_CONNECTED UEs for given multicast MRB, the RRC_INACTIVE UE should be following same DRX configuration. However, UE in RRC_INACTIVE is not expected to provide HARQ feedback and/or receive HARQ retransmission (except for blind retransmissions), therefore, RTT timer and ReTx timer configurations are not required. There seems to be a few options for providing DRX configurations for RRC_INACTIVE:

Option 1: C-DRX configuration for multicast MRB is continued in RRC_INACTIVE expect RTT timer and ReTx timer configurations

Option 2: Explicit multicast DRX configuration for RRC_INACTIVE is provided (e.g. in RRCRelease with suspendConfig) which is simplified with no RTT timer and ReTx timer configurations

Option 2 seems preferable which is also aligned with the PTM configuration signalling approach as in proposal 1.

Proposal 6: Explicit multicast DRX configuration for RRC_INACTIVE is provided (e.g. in RRCRelease with suspendConfig) which is simplified with no RTT timer and ReTx timer configurations.

### 2.5 Mobility aspects

For a UE configured to receive multicast in RRC_INACTIVE, multicast session may or may not be provided on the neighbor cells. Consequently, for a UE reselecting a neighbor cell may lead to disruption of multicast service reception. For this reason, UE can be provided with neighbour cell multicast configuration information to support mobility.

Proposal 7: UE is provided with the neighbour cell multicast configuration information to support mobility.

Another important aspect is the RAN overload due to multicast. It should be feasible to reuse the RAN overload control mechanism of deprioritization specifically for multicast. In legacy LTE and NR, network can determine RAN overload situation and sends to the UE, a deprioritization request for the current frequency (or RAT) along with RRCRelease message. This ensures that the UE considers the indicated frequency (or all frequencies of the RAT) to be the lowest priority for cell reselection for a configured duration [2].

We think this well-established and proven mechanism of de-prioritization seems a good approach to address the congestion / overload scenario for multicast. Network can indicate de-prioritization request for multicast frequency along with RRCRelease with suspendConfig and only multicast UE can consider the indicated frequency to be the lowest priority for cell reselection for configured time duration while UE is in RRC_INACTIVE.

Proposal 8: UE is provided with the de-prioritization request for multicast to control RAN overload on specific frequency.

# 3 Conclusion

Request RAN2 to discuss and agree to the following observations and proposals:

Proposal 1: RRCRelease with suspendConfig provides the PTM configuration for multicast MRBs to be received in RRC_INACTIVE. Pre-configuration with RRC reconfiguration message is not adopted.

Observation 1: RAN2 should consider the issue of unauthorized access to multicast configuration through MCCH.

Observation 1: RAN2 should consider the issue of unauthorized access to multicast configuration through MCCH.

Proposal 2: RAN2 to agree:

- UE joins multicast session and receives configuration in RRC_CONNECTED, while UE can receive the update of PTM configuration in RRC_INACTIVE through MCCH without transiting back to RRC_CONNECTED

- MCCH provides the indication for a change of PTM configuration and only the changed configuration parameters for multicast session (i.e. a delta signalling)

- Multicast configuration identity is defined to provide a mapping between multicast session configuration in the dedicated signalling and updated session configuration in the MCCH

Observation 2: MCCH configuration through dedicated signaling will bring the need for new MCCH channel, lack of dynamism of configuration update for MCCH, large complex specification impacts and lack of authroized access for each multicast session.

Proposal 3: RAN2 to not consider providing MCCH configuration through dedicated signalling

Proposal 4: RRCRelease with suspendConfig can carry MBS radio bearer config, MAC config and PHY config for multicast reception in RRC_INACTIVE.

- These exclude certain parameters which are specific to dedicated configuration used only for RRC_CONNECTED.

- Additionally, RRCRelease with suspendConfig may provide neighbour cells multicast configuration e.g., whether the neighbour cells support same multicast configurations as the serving cell.

Proposal 5: RAN2 to consider to define a BWP same as MBS CFR for multicast reception operation by UE in RRC_INACTIVE. RAN2 to send LS to RAN1.

Proposal 6: Explicit multicast DRX configuration for RRC_INACTIVE is provided (e.g. in RRCRelease with suspendConfig) which is simplified with no RTT timer and ReTx timer configurations.

Proposal 7: UE is provided with the neighbour cell multicast configuration information to support mobility.

Proposal 8: UE is provided with the de-prioritization request for multicast to control RAN overload on specific frequency.

# 4 Reference

[1] RAN2#120 meeting report v1

[2] 3GPP TS 38.331 v17.3.0: Radio Resource Control (RRC) protocol specification