3GPP TSG-RAN2#121										     R2-2300560

Athens, GR, 27th Feb – 3rd Mar 2023

Source: 			ZTE Corporation, Sanechips

Title: 	BSR enhancements for XR

Agenda item:	8.5.4

Document for: Discussion and Decision

# Introduction

During the study item it was agreed that BSR enhancements including new BS Table(s) will benefit the capacity enhancements for XR. In this contribution, we study the potential BSR enhancements that seem promising.

# BSR reporting

Currently BSR reporting uses either short BSR table or the long BSR table. The BSR values reported are quantized and the UE will have to report the code point representing the value just above the amount of buffered data in the UE buffer. Unfortunately, this results in over reporting of buffer status values (referred to as the quantization error below) and the issue is more critical at higher BSR values as shown below.

| Figure 1: Quantization error Values (i.e., amount of over reported buffer data) with existing BSR tables |  |
| --- | --- |

Observation 1: The amount of over reported buffer data increases with the higher BSR indices (i.e., the error is larger when there is large amount of buffered data at the UE)

Based on the above observation, we can conclude that the enhancement schemes should mainly target the higher BSR indices (e.g., the buffer status values that are above BSR Index 150 or so in case of Long BSR).

Then, one solution to the above problem is for the UE to include more than one BSR to reduce the quantization error. Together with the first BSR value, the additional BSR value (pointing to the same logical channel) would indicate the total buffered data volume for the corresponding logical channel. With just two BSR MAC CEs included, the quantization error can be reduced quite significantly and this is depicted in the figure below.

Figure 2: Reduction in quantization error with two BSRs

Observation 2: With just one additional BSR included for the same logical channel, the quantization error for BSR reporting is significantly reduced (even with existing BSR tables)

Observation 3: The quantization error is fairly negligible at low values of BSR and hence the gains with the 2nd BSR being included for such small BSR values is also negligible.

Based on the above, the following proposals are made:

Proposal 1: RAN 2 should define a mechanism where the UE may include 2 BSRs for the same logical channel to minimize the quantization errors for the BSR reporting

Proposal 2: The UE should include the 2nd BSR only if the quantization error is larger than a threshold that is configured by the network

# Conclusion

Observation 1: The amount of over reported buffer data increases with the higher BSR indices (i.e., the error is larger when there is large amount of buffered data at the UE)

Observation 2: With just one additional BSR included for the same logical channel, the quantization error for BSR reporting is significantly reduced (even with existing BSR tables)

Observation 3: The quantization error is fairly negligible at low values of BSR and hence the gains with the 2nd BSR being included for such small BSR values is also negligible.

Proposal 1: RAN 2 should define a mechanism where the UE may include 2 BSRs for the same logical channel to minimize the quantization errors for the BSR reporting

Proposal 2: The UE should include the 2nd BSR only if the quantization error is larger than a threshold that is configured by the network

# References

[1] xxx