3GPP TSG RAN WG2 #121	                                    			R2-2300577

Athens, Greece, 27th February – 3rd March, 2023

Agenda Item:	8.4.2.2

Source:	InterDigital, Inc.

Title:	LTM candidate configurations.

Document for:	Discussion

# Introduction

The WI on further NR mobility enhancements [1] includes the following objectives for L1/2 based inter-cell mobility:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

In the previous meeting, the following agreements were made.

- RAN2#119bis-e - A L1/L2 inter-cell mobility candidate (target) configuration is received within an RRC message before the L1/L2 dynamic switch is triggered. - RAN2 continues the discussion on the RRC models by focusing on Model 1 and Model 2 and stage-3 details. - Model 1: One RRCReconfiguration message (or FFS RRCReconfiguration IEs) for each candidate target configuration - Model 2: One CellGroupConfig IE (FFS additional IEs) for each candidate target configuration Chair: FFS if there are strong reasons to go back to discuss other models. If so, we can consider modifications to the decision above, e.g. if R1 preferences gives strong reasons. - RAN2 assumes that sequential L1L2 cell change between Candidates without RRC reconfiguration can be supported. - RAN2 assumes that candidate cell configuration can only be modified / released by Network (FFS later whether some optimization should be applied e.g. for release). - For L1L2 mobility will support that candidate configurations are delta configurations on top of a reference configuration. FFS if the reference configuration is a separate reference configuration or e.g. the current configuration. - For L1L2 mobility, Target Pcell/SCell can be current SCell/PCell, i.e., current SCell/PCell can be configured as candidates. - FFS how the UE determine the BWPs (for DL and UL) to be used upon the execution of L1/L2 inter-cell mobility RAN2#120 - RAN2 to confirm that the CellGroupConfig IE is (mandatory) needed within an LTM candidate cell configuration. - The RadioBearerConfig IE can be optionally supported in an LTM candidate configuration - The MeasConfig IE can be optionally supported in an LTM candidate configuration. - The OtherConfig IE is not required to be part of the LTM candidate cell configuration. - The LTM candidate cell configuration should be designed as a To AddMod/ToRelease structure. - The LTM candidate cell configuration ASN.1 structure comprises at least a CellGroupConfig IE and a configuration ID. - On Delta Configuration - A UE stores the reference configuration as a separate configuration. - The reference configuration is managed separately

# Discussion

Considering minimising the configuration overhead, we need to consider how to support a delta configuration of the target cell compared to the source. In addition, since it is assumed that for certain cases (at least intra-DU) that some of the candidate configuration will be common, then we should consider how to configure this in an efficient way.

## Common configuration parts

If the RRC model is either one RRC reconfiguration message per candidate or one CellGroupConfig IE per candidate, then this implies that, for example, when 8 target candidate configurations are provided then 8 CellGroupConfig or 8 RRC reconfiguration messages need to be provided by the NW.

However, this is unnecessary if several candidates share some common configuration. For example, if 4 of the candidate cells belong to one DU, and the other 4 to another DU, then it is possible that the CellGroupConfig (and SDAP, PDCP config, and potentially measurement config,) are common to the 4 candidates belonging to the same DU. In this case, the configuration needs to be provided only once. If the 4 candidates are provided in a list, we can provide the CellGroupConfig with the first entry in the list, and then refer to this by either providing a “NULL” IE or by referring to the index of the candidate entry in the list. The point is, it is not necessary to provide one CellGroupConfig (or one RRC reconfiguration message) per candidate if some of the information is common.

Proposal 1: Candidate configurations can be configured with some common configuration information to minimise overhead. For example, several candidate cells belonging to the same DU could be configured with the same CellGroupConfig and in this case it is not necessary to repeat the configuration for every candidate cell.

## Target configuration model

One potential approach to minimise overhead would be to allow the possibility to configure multiple target configurations, each with multiple serving cell options.

We have the following 2 main options for structuring the candidate configuration in the case of 3 potential target SpCells belonging to the same DU (the 2 examples assume CellGroupConfig candidate configuration model, but the same applies in case we have RRC Reconfiguration candidate configuration model):

Option A:

Every possible SpCell has a complete target configuration:

- Config 1: CellGroupConfig with cell 1 as SpCell and cells 2,3 as SCells - Config 2: CellGroupConfig with cell 2 as SpCell and cells 1, 3 as SCells - Config 3, CellGroupConfig with cell 3 as SpCell and cells 1, 2 as SCells

Option B:

One CellGroupConfig (or RRC Reconfiguration) which is provided with SpCell and SCell options for each target cell:

- Config 1 (includes CellGroupConfig) - Cell 1 – SpCellConfig, SCellConfig - Cell 2 – SpCellConfig, SCellConfig - Cell 3 – SpCellConfig, SCellConfig

This way, the configuration may be more compact and would better support both intra-DU and inter-DU handover. The L1/2 trigger would need to indicate the configuration to apply, as well as the SpCell, and the active SCells. This could also be used to differentiate between an inter-DU handover (in which case the whole configuration including CellGroupConfig would be changed) or an intra-DU handover (in which case only the SpCellConfig and/or SCellConfigs would change). More details of the L1/2 triggers for these options are provided in [3].

Proposal 2: RAN2 to discuss the following options:

- a complete target configuration needs to be provided for every single target SpCell (Option A)

- a target configuration can support multiple potential SpCells (Option B)

## Delta configuration handling

It has been agreed in the previous meeting that the reference configuration is stored and maintained separately. However, it was not agreed how the reference configuration is configured. The options could either be separately configured, based on the cell in which the configuration was received, or based on one of the candidate configurations.

Proposal 3: Discuss which of the options for configuration of the reference configuration are supported.

- Based on the current configuration

- Based on a separate explicit configuration

- Based on one of the candidate configurations (E.g. one candidate configuration is a full configurations, and others are delta based on this)

It has been agreed to support the possibility that candidate configurations are delta configurations on top of a reference configuration. Since it has also been agreed that subsequent cell switch without RRC reconfiguration is supported, then the delta configuration aspect becomes a bit more complicated.

Currently, RRC reconfiguration supports a delta configuration compared to the current configuration. RRC Configuration also supports “full configuration” which would mean to release everything and set up a new complete configuration.

One possibility for LTM would be that every target cell is configured with a delta configuration based on a reference configuration (which could either be separately configured, based on the cell in which the configuration was received, or based on one of the candidate configurations). For example, the UE may be configured with baseline_config when the PCell is cell1, and configA for candidate cellA, which is a delta configuration  from baseline_config, and configB for cellB, which is also a delta configuration from baseline_config. When the PCell changes to cellA, the UE applies configA. However, if the UE then have to change the PCell to cellB, configB can not be directly applied. Thus, the UE has to revert to the baseline_configuration (i.e., a full configuration) followed by the delta configuration configB.

When a UE does a full configuration, it releases all current dedicated radio configuration except for the MCG C-RNTI, AS security configuration and radio bearer (signalling and data), and logged measurements (the following (TS 38.331, section 5.3.3.11). This has several undesirable implications like the loss of any buffered data (waiting first transmission or re-transmission) at RLC/MAC level, which will cause data transmission interruption and may also lead to data loss (e.g., if the discard timer has expired for a packet at PDCP level).  That is one of the reasons that full configuration is rarely used (in addition to signalling overhead), mostly only on RRC re-establishments where the UE has to restart the connection from scratch after a failure such as a Radio Link Failure (RLF) or during connection resumption from INACTIVE state in the scenario where the target node/cell where the UE is resuming the connection has some incompatibility with the source node/cell where the UE was sent to the INACTIVE state.

In addition, it has been agreed in a previous meeting that it is assumed that L2 is continued whenever possible (e.g. intra-DU), without Reset, with the target to avoid data loss, and the additional delay of data recovery. What this means is, at least for the intra-DU case, the RLC/PDCP and MAC reset may not be necessary. For the intra-DU case, there is no reason to reset the MAC (as is done with conventional L3 handover in order to cover all cases) if the MAC configuration is identical for both cells since the MAC resides in the DU part of the network.

However, if the UE has to apply a full configuration based on a reference configuration then this implies MAC reset, and RLC/PDCP re-establishment.

Based on the above, it would be better if the UE is able to compare source and target configurations without having to revert to a baseline (full) configuration to apply a new candidate cell delta configuration, regardless of whether we follow option A or B as described above.

If proposal 1 is agreed, and multiple candidate cells can share common configurations (for the parts that are the same) then it also becomes possible to compare what the difference between candidate cells is, even after handover from the original cell/baseline configuration, and then apply the relevant delta configurations and execute the relevant procedures.

For example, if a handover is performed from cell 1 -> cell 2 -> cell 3, and if cells 2 and 3 use the same/common MAC configuration, then the UE does not need to perform MAC reset or apply a different MAC configuration when moving from cell 2 to cell 3. If cells 2 and 3 have a different MAC configuration then the UE could apply a (full) MAC configuration based on the reference + delta without having to apply the full configuration for everything else (E.g. RLC, PDCP).

This implies that a UE has to maintain a record of the difference between the current (delta) configuration and the reference configuration, so that only the necessary parts are either reverted to the reference configuration or applied with the new delta configuration parts. Otherwise the UE has to construct a full configuration based on the reference + delta, implying a full configuration procedure and unnecessary actions during the RRC reconfiguration procedure itself.

Proposal 4:  A solution is needed for the UE to compare the source and target configurations in order to avoid needing to perform a full configuration based on a reference configuration before applying the candidate (delta) configuration, and to allow only the relevant (changed between source and target) configuration parts to be applied.

## Maintenance of the candidate cells

In addition to the initial configuration of candidate cells, we need to support RRC reconfigurations. For example, the network may at any time use a (legacy) RRC reconfiguration of the current cell. It may also be necessary to update the reference configuration or a particular cell (the candidate configuration corresponding to the current SpCell or any candidate cell) may need to be reconfigured. It should also be possible to modify any part of the configuration without reconfiguring everything and setting up the entire candidate list again. Therefore, some consideration is needed for the signalling and procedure design for how to perform an RRC reconfiguration for various cases.

Proposal 5: RAN2 to discuss the following aspects of RRC reconfiguration after one or multiple subsequent cell switches, and how these interact:

- Reconfiguration of the current (serving cell) configuration.

- Reconfiguration of stored candidate (delta) configuration.

- Reconfiguration of the stored reference configuration.

When a UE is configured with a list of candidate cells, that list should be maintained/updated as the UE moves.  Specifically, a UE may trigger L1/L2 cell switch and change its current serving cell.  In doing so, the list of valid candidate cells that can be targets of L1/L2 mobility in the new serving cell will likely also be different. For example, in case candidate cells have been configured across an area larger than just one current SpCell, then each of the potential SpCells can have different neighbours, and hence different candidate LTM cells would be “valid”. At a minimum, the source cell of the L1/L2 mobility should become a potential candidate cell in the target, at least for a period of time.  Since the purpose of L1/L2 mobility is to avoid RRC reconfiguration, the update of the valid candidate cells that should occur with any L1/L2 mobility should happen without a reconfiguration procedure at L3.

Proposal 6: A UE can update the list of candidate cells upon successful L1/L2 cell switch without the need for RRC reconfiguration.

One way to achieve the update would be to associate a list of valid candidate cells with each current serving cell.  At each L1/L2 cell switch, the UE may apply the list of valid candidate cells associated with that serving cell.  Depending on the design of this approach, the signalling may become quite heavy.  Specifically, it requires indicating the list of valid candidate cells for each serving cell which could be a target of an L1/L2 mobility.  Another approach may be to provide a larger than needed list of cell configurations to the UE and have the UE determine its list of valid list of candidates at any time, based on, for example, L3 measurements of each of the cells.  In this way, the network may for example configure a list of cells which is agnostic to the UE’s capabilities, and each UE may perform measurements for L1/L2 mobility only on a subset of those.  While this approach may save on downlink signalling, it requires the UE to indicate updates of the valid cells using uplink signalling.  How feasible such an approach may be would require further discussion, as the uplink signalling could be incorporated into the measurement reports (either at RRC or MAC).  However, there are at least some specific cases (e.g., beam failure on a candidate cell) where the UE should inform the network that a cell in the candidate cell list is not valid.

Proposal 7: A candidate cell configuration may be evaluated at the UE (and indicated to the network) as a valid/invalid target for L1/L2 cell switch.  FFS the criteria used to determine validity (e.g., beam failure, RLM, RSRP). FFS the reporting mechanism (RRC or MAC CE)

## RRC_INACTIVE

Currently, when the UE receives RRCRelease to move to RRC_INACTIVE, stored candidate cell configurations are released (e.g. conditional reconfiguration targets). However, since LTM is designed to achieve multiple subsequent cell switches without the need for RRC Reconfiguration it is possible (likely) that LTM candidate configurations are still valid after resuming the connection, even if the UE has performed some cell reselection in the meantime (within a limited area). Since LTM is also designed to minimise RRC reconfiguration needed after cell switch, it would be beneficial if candidate configurations can be stored when the connection is suspended, so that they are available without the need for RRC Reconfiguration again after resume to (re) setup LTM candidates – at least if the UE stays in the same cell, or within a relatively small area (E.g. within the coverage of configured LTM candidates).

Proposal 8: LTM candidates are stored when entering RRC_INACTIVE so that LTM candidates can be available during and/or after RRC Resume.

If LTM candidates are stored when entering RRC_INACTIVE then LTM can immediately be used after resuming the RRC Connection (E.g. after Msg5). It is possible to further improve the RRC Resume procedure itself if the stored candidate configuration for the current cell (i.e. the PCell) could be enabled by indicating the configuration ID in RRC Resume, or in other words to trigger LTM towards the current cell in Msg4, by referencing the stored SpCell configuration of the current cell.

Proposal 9: Discuss whether LTM can be used to indicate the stored configuration for PCell (current cell) during RRC Resume.

If early measurements are reported then LTM could also be used to quickly setup SCG/SCells based on stored candidate configurations. In addition, it is still to be discussed exactly how the R18 early measurements for FR2 are reported and what to do with the reported measurements (e.g. how to report the measurement, what the content of the report is, and what to send from the NW in response). If these measurements can be reported during RRC resume then it might be possible to not only use LTM for setting up (resuming) the PCell but it could also be used for fast SCG/SCell setup based on measurements of potential SCells.

Proposal 10: Discuss whether LTM can be used to indicate stored configurations for SCG/SCells during RRC Resume (e.g. if early measurements are reported).

# Conclusion

In this paper we provide the following proposals to consider in the design of L1/2 triggered handover in Release-18.

Proposal 1: Candidate configurations can be configured with some common configuration information to minimise overhead. For example, several candidate cells belonging to the same DU could be configured with the same CellGroupConfig and in this case it is not necessary to repeat the configuration for every candidate cell.

Proposal 2: RAN2 to discuss the following options:

- a complete target configuration needs to be provided for every single target SpCell (Option A)

- a target configuration can support multiple potential SpCells (Option B)

Proposal 3: Discuss which of the options for configuration of the reference configuration are supported.

- Based on the current configuration

- Based on a separate explicit configuration

- Based on one of the candidate configurations (E.g. one candidate configuration is a full configurations, and others are delta based on this)

Proposal 4:  A solution is needed for the UE to compare the source and target configurations in order to avoid needing to perform a full configuration based on a reference configuration before applying the candidate (delta) configuration, and to allow only the relevant (changed between source and target) configuration parts to be applied.

Proposal 5: RAN2 to discuss the following aspects of RRC reconfiguration after one or multiple subsequent cell switches, and how these interact:

- Reconfiguration of the current (serving cell) configuration.

- Reconfiguration of stored candidate (delta) configuration.

- Reconfiguration of the stored reference configuration.

Proposal 6: A UE can update the list of candidate cells upon successful L1/L2 cell switch without the need for RRC reconfiguration.

Proposal 7: A candidate cell configuration may be evaluated at the UE (and indicated to the network) as a valid/invalid target for L1/L2 cell switch.  FFS the criteria used to determine validity (e.g., beam failure, RLM, RSRP). FFS the reporting mechanism (RRC or MAC CE)

Proposal 8: LTM candidates are stored when entering RRC_INACTIVE so that LTM candidates can be available during and/or after RRC Resume.

Proposal 9: Discuss whether LTM can be used to indicate the stored configuration for PCell (current cell) during RRC Resume.

Proposal 10: Discuss whether LTM can be used to indicate stored configurations for SCG/SCells during RRC Resume (e.g. if early measurements are reported).

# References

- RP-221799,  “Revised WID on Further NR mobility enhancements”

- 38.133, “NR; Requirements for support of radio resource management”

- R2-2300578, “LTM cell switch and triggering”, InterDigital, Inc.