3GPP TSG RAN WG2 #120	                                    			R2-2300579

Athens, Greece, 27th February – 3rd March, 2023

Agenda Item:	8.6.2.1

Source:	InterDigital, Inc.

Title:	Disabling HARQ feedback for IoT-NTN.

Document for:	Discussion

# Introduction

The WI on IoT NTN enhancements [1] includes the following objectives for performance enhancement:

- Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

In RAN2#119-e the following agreements were made:

Agreements:

- Disabling DL HARQ feedback is supported for NB-IoT and eMTC NTN. FFS on UE capability

- For UL HARQ operation, introduce two HARQ modes, i.e., HARQ mode A and HARQ mode B in IoT NTN (both NB-IoT and eMTC NTN), similarly to NR NTN

- From RAN2 perspective, at least for eMTC, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signalling. FFS for NB-IoT (and especially for CP solution for NB-IOT).

In RAN2#119bis-e the following agreements were made:

Agreements:

- For NB-IoT, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signaling (e.g. RRCConnectionSetup). This does not preclude other options (e.g. DCI-based). We can also revert this decision if requested by RAN1.

- Disabling HARQ feedback is supported for NB-IoT with single HARQ process, and it is up to eNB implementation whether to disable the HARQ feedback

Working Assumption:

- Blind retransmission can be used in IoT NTN when HARQ feedback is disabled and when HARQ mode B is used (RAN2 assumes there is no spec change for this)

- HARQ mode A/B for uplink transmission may be configured per UL HARQ process at least via UE specific RRC signalling for eMTC and NB-IOT NTN. We can also revert this decision if requested by RAN1

- RAN2 agree to take R17 NR NTN DRX solution as baseline for IoT NTN, e.g. for HARQ process with DL HARQ feedback disabled, the UE will not start the corresponding DL HARQ RTT timer.

- For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PDSCH reception (can still check whether the alternative to set the HARQ RTT timer to 0 also works)

- RAN2 agree to take R17 NR NTN DRX solution as baseline for IoT NTN, e.g. for HARQ process in HARQ mode B, the UE will not start the corresponding UL HARQ RTT timer.

- For NB-IoT NTN with single HARQ process in HARQ mode B, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PUSCH transmission (can still check whether other alternatives also work)

- The solutions of LCP restriction on allowed HARQ mode in NR NTN can be reused for eMTC NTN.

In RAN2#120 the following agreements were made:

Agreements: 1. For eMTC, the following LCH to HARQ process mapping rules are supported: 1) LCH is mapped only to a HARQ process configured with HARQ mode A; 2) LCH is mapped only to a HARQ process configured with HARQ mode B; 3) If an LCH is not configured with a mapping rule, it may be mapped to any HARQ process (HARQ mode A or B). 4) If UL HARQ mode is not configured, LCH mapping rules are not supported (legacy behaviour) 2. For eMTC, introduce allowedHARQ-mode for each logical channel, e.g. included in LogicalChannelConfig IE.

In addition, RAN1 has made the following working assumption.

Working assumption For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission: - Support Option 1 by default, and support Option 3 to override default configuration for corresponding transmission - Additional RRC signaling to enable Option 3 - If the bitmap for option 1 is not present and if option 3 is configured then the DCI directly indicates HARQ enable/disable. Option 3 can also be configured when the bitmap for option 1 is configured. - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ enabled and disabled processes or only applied to semi-statically HARQ disabled processes or only applied to semi-statically HARQ enabled processes. - FFS #2: whether/how to support Option 3 overriding default configuration for corresponding transmission for multiple TBs scheduled by single DCI For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A.

In this contribution we elaborate the potential impact.

# Background

## 2.1	Disabled HARQ feedback in Rel-17 NR NTN

Configuration of HARQ characteristics per HARQ process ID

A UE may be semi-statically configured via RRC to apply a specific HARQ behaviour to a set of HARQ process IDs. This configuration is per serving cell, and can be optionally configured for both UL and DL HARQ processes via the following optional configurations:

- downlinkHARQ-feedbackDisabled: configured per HARQ process ID, and indicates whether DL HARQ feedback is enabled or disabled.

- uplinkHARQ-Mode: Configured per HARQ process ID, and indicates whether an UL HARQ process uses HARQModeA or HARQmodeB.

  - RAN2 understanding is that HARQmodeA best applies to transmissions with UL HARQ retransmission enabled, and HARQmodeB best applies to transmissions with UL HARQ retransmission disabled or with blind UL retransmission.

HARQ-based LCP restriction

A UE may be configured to apply an LCP restriction based on the UL HARQ mode configured for the HARQ process ID an UL grant is assigned to. UE behaviour is specified via two optional RRC configurations:

- uplinkHARQ-Mode, which configures each HARQ process ID as either HARQModeA or HARQModeB, and;

- allowedHARQ-Mode, which is configured per logical channel and sets the allowed HARQ mode of a HARQ process mapped to this logical channel.

Upon reception of a new UL grant, the UE will determine if allowedHARQ-mode is configured for this LCH, and if a HARQ Mode has been configured for the HARQ process the UL grant is assigned to. If both are configured and the LCH is allowed to be mapped to the HARQ mode, the restriction is satisfied and data from this logical channel may be mapped to the UL grant. If either uplinkHARQ-Mode or allowedHARQ-Mode are not configured, the UE may map this logical channel to any HARQ process.

Adaptation of DRX based on configured HARQ mode

A UE adapts DRX timers based on the configured HARQ characteristics of a HARQ process. DRX is adapted for both UL and DL to either adapt DRX active time account for additional propagation delay (e.g. if HARQ feedback is enabled) or to enabled additional UE power saving (e.g. if HARQ feedback is disabled). Operation is adapted as follows:

- DL: if downlinkHARQ-FeedbackDisabled is configured for this serving cell, upon DL reception:

  - if HARQ feedback is enabled for the HARQ process the UE will extend the length of the DL HARQ RTT Timer by the UE-gNB RTT (i..e propagation delay).

  - If HARQ feedback is disabled for the HARQ process, the UE will not start drx-RetransmissionTimerDL to enable additional power saving.

  - If  downlinkHARQ-FeedbackDisabled is not configured, the UE will apply legacy behaviour (i.e. start drx-RetransmissionTimerDL after expiry of drx-HARQ-RTT-TimerDL).

- UL:  if uplinkHARQ-Mode is configured for this serving cell, upon UL transmission:

  - if HARQ process is configured as HARQModeA the UE will extend the length of the DL HARQ RTT Timer by the UE-gNB RTT (i..e propagation delay).

  - If  HARQ process is configured as HARQModeB, the UE will not start drx-RetransmissionTimerDL to enable additional power saving.

  - If   uplinkHARQ-Mode is not configured, the UE will apply legacy behaviour (i.e. start drx-RetransmissionTimerUL after expiry of drx-HARQ-RTT-TimerUL).

## 2.2	HARQ for NB-IoT/eMTC

Asynchronous HARQ is used in uplink for NR, however in LTE uplink HARQ can be synchronous or asynchronous:

- For asynchronous HARQ, the network may schedule any HARQ process by indicating a HARQ process ID in the DCI scheduling the HARQ transmission. The DCI further indicates whether a new transmission or a retransmission is scheduled for that HARQ process via the NDI bit.

- For synchronous HARQ, the HARQ process is determined via uplink timing. For example, when 8 HARQ processes are configured, a given HARQ process will transmit every 8ms (TTIs). Whether a MAC PDU can be (re)transmitted depends on the PHICH, which carries HARQ ACK/NACK information for PUSCH transmissions.

For eMTC and NB-IoT, uplink HARQ operation is asynchronous except for repetitions within a bundle. The number of UL repetitions in a bundle is given by parameter UL_REPETITION_NUMBER, and is set (per bundle) via indication from lower layers. The UL bundling operation has the following characteristics:

- The bundling operation uses the same HARQ process for each transmission belonging to the same bundle;

- Within a bundle, HARQ retransmissions are non-adaptive and triggered without waiting for HARQ feedback for previous transmissions.

- An uplink grant corresponding to a new transmission or a retransmission of the bundle is only received after the last repetition of the bundle.

- A retransmission of a bundle is also a bundle.

Similarly, in the DL the parameter DL_REPETITION_NUMBER provides the number of transmissions repeated in a bundle and is also set per bundle by lower layers. The DL bundling operation has the following characteristics:

- Within a bundle, after the initial (re)transmission, DL_REPETITION_NUMBER-1 HARQ retransmissions follow.

- HARQ feedback is transmitted for the bundle

- A downlink assignment corresponding to a new transmission or retransmission of the bundle is received after the last repetition of the bundle.

- A retransmission of a bundle is also a bundle.

# Discussion

## 3.1	HARQ-based LCP restriction:

It has been agreed in the previous meeting to introduce allowedHARQ-mode for each logical channel, at least for eMTC. The following mapping rules are agreed to be supported:

1) LCH is mapped only to a HARQ process configured with HARQ mode A;

2) LCH is mapped only to a HARQ process configured with HARQ mode B;

3) If an LCH is not configured with a mapping rule, it may be mapped to any HARQ process (HARQ mode A or B).

4) If UL HARQ mode is not configured, LCH mapping rules are not supported (legacy behaviour)

As observed in [2], by disabling HARQ feedback and not increasing the number of repetitions to compensate for no bundle retransmissions in the case of a failed transmission, the chance of protocol stalling is increased at RLC.

A compromise approach may be needed, such that UL HARQ is used or repetitions are increased only when necessary, to minimise the bottleneck/protocol stalling at HARQ and unnecessary retransmissions, and at the same time minimise protocol stalling at RLC. Most of the time, the number of repetitions can be relatively small (i.e. at a similar level as used for any given coverage enhancement level as in R17). However, this will cause an increased number of RLC retransmissions and therefore some transmissions need to be transmitted with higher reliability to avoid protocol stalling – RLC STATUS PDU and RLC retransmission have to be delivered with high reliability.

For these transmissions (RLC retransmissions and STATUS PDUs) we can consider either transmitting with a higher number of repetitions in these cases (increasing the probability of successful delivery) or transmitting using a HARQ process optimized for UL HARQ retransmission (and hence bundle retransmissions increase the probability of successful delivery).

By applying LCP restriction to RLC PDU Type, we can configure e.g. RLC initial transmissions to use HARQ processes configured with HARQ Mode B and RLC retransmissions to use HARQ mode A. RLC STATUS PDUs can also use HARQ processes configured with HARQ mode A to ensure reliable delivery. If RLC initial transmissions (the majority of RLC PDUs) are sent with HARQ processes configured with HARQ mode B we can reduce the possibility of protocol stalling at HARQ. By transmitting RLC STATUS PDUs and RLC retransmissions using higher reliability with HARQ processes configured with HARQ mode A, then we can ensure that the number of RLC retransmissions can be limited, so that the RLC window can advance and reduce the possibility of protocol stalling at RLC.

Observation: To minimise the risk of protocol stalling at RLC when using higher data rates, initial RLC transmission could be sent using HARQ processes configured with HARQ Mode B to improve throughput, and RLC retransmissions and RLC STATUS PDUs could be sent using HARQ processes configured with HARQ mode A to ensure reliable delivery.

Based on the above observation we propose the following:

Proposal 1: At least for eMTC: LCP restriction based on uplinkHARQ-Mode can be configured for different RLC PDU types, such that RLC initial transmissions, retransmission, and STATUS PDUs can be restricted to a particular HARQ Mode.

## 3.2	Adaptation of DRX based on DCI configured HARQ mode

RAN1 has made the following working assumption:

Working assumption For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission: - Support Option 1 by default, and support Option 3 to override default configuration for corresponding transmission - Additional RRC signaling to enable Option 3 - If the bitmap for option 1 is not present and if option 3 is configured then the DCI directly indicates HARQ enable/disable. Option 3 can also be configured when the bitmap for option 1 is configured. - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ enabled and disabled processes or only applied to semi-statically HARQ disabled processes or only applied to semi-statically HARQ enabled processes. - FFS #2: whether/how to support Option 3 overriding default configuration for corresponding transmission for multiple TBs scheduled by single DCI For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A.

Option 1 referred to by RAN1 is an RRC configuration which is already agreed in RAN2, and also implemented in NR Rel-17. Option 3 is the enabling/disabling of HARQ for the associated DL transmission with the DCI.

Since enabling/disabling of HARQ for the associated DL transmission with the DCI is not specified in NR we cannot exactly use this as a baseline. Adaptation of the DRX timers needs to take into account a potential dynamic change of setting of DL HARQ mode by L1. This behaviour needs to be specified in MAC. However, based on the current NR specification, the procedure sets the value of the relevant timers based on RRC setting only. For example, the NR text is as follows:

When DRX is configured, the MAC entity shall: 1> if a MAC PDU is received in a configured downlink assignment for unicast: 2> if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled: 3> if the corresponding HARQ process is configured with HARQ feedback enabled: 4> set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value; 4> start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. 2> else: 3> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

Some additional behaviour compared to NR needs to be specified such that the configuration of HARQ feedback enabled/disabled can be done via RRC or via DCI.

One way would be to use a similar sentence as in NR, but rather than “configured” which implies RRC, to use a different wording. For example:

3>	if the corresponding HARQ process has HARQ feedback enabled

Some additional text would be needed (e.g. in a separate section) to specify that a HARQ process can have HARQ feedback enabled or disabled based on RRC configuration or DCI indication.

Proposal 2: Assuming the RAN1 working assumption is confirmed, then:

Proposal 2a: Specify in MAC that a HARQ process can have HARQ feedback enabled or disabled, based on RRC configuration or DCI indication.

Proposal 2b: Study whether any further changes are needed to support DCI based HARQ feedback disabling (e.g. timer handling).

In addition, since DL HARQ feedback can be enabled/disabled by DCI, we should consider whether DCI can also be used to set the uplink HARQ mode. Currently the procedure in NR for UL is as follows:

1> if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers: 2> if this Serving Cell is configured with uplinkHARQ-Mode: 3> if the corresponding HARQ process is configured as HARQModeA: 4> set HARQ-RTT-TimerUL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerUL plus the latest available UE-gNB RTT value; 4> if drx-LastTransmissionUL is configured: 5> start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission. 4> else: 5> start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission

If this is the case, then a similar procedure needs to be introduced in MAC to set the uplink HARQ mode based on DCI.

Proposal 3: RAN2 to consider whether uplink HARQ mode based on DCI should be supported.

If so, then:

Proposal 3b: Send an LS to RAN1 indicating RAN2 conclusion.

Proposal 3c: Specify in MAC that a HARQ process use Uplink HARQ mode A or B, based on RRC configuration or DCI indication.

Proposal 3d: Study whether any further changes are needed to support DCI based HARQ mode setting (E.g. timer handling, LCP restriction handling)

# Conclusion

In this contribution we discuss the impact of disabling HARQ feedback for IoT-NTN. The following is observed and proposed:

Observation: To minimise the risk of protocol stalling at RLC when using higher data rates, initial RLC transmission could be sent using HARQ processes configured with HARQ Mode B to improve throughput, and RLC retransmissions and RLC STATUS PDUs could be sent using HARQ processes configured with HARQ mode A to ensure reliable delivery.

Proposal 1: At least for eMTC: LCP restriction based on uplinkHARQ-Mode can be configured for different RLC PDU types, such that RLC initial transmissions, retransmission, and STATUS PDUs can be restricted to a particular HARQ Mode.

Proposal 2: Assuming the RAN1 working assumption is confirmed, then:

Proposal 2a: Specify in MAC that a HARQ process can have HARQ feedback enabled or disabled, based on RRC configuration or DCI indication.

Proposal 2b: Study whether any further changes are needed to support DCI based HARQ feedback disabling (e.g. timer handling).

Proposal 3: RAN2 to consider whether uplink HARQ mode based on DCI should be supported.

If so, then:

Proposal 3b: Send an LS to RAN1 indicating RAN2 conclusion.

Proposal 3c: Specify in MAC that a HARQ process use Uplink HARQ mode A or B, based on RRC configuration or DCI indication.

Proposal 3d: Study whether any further changes are needed to support DCI based HARQ mode setting (E.g. timer handling, LCP restriction handling)

# References

- RP-223519,  “Revised WID on IoT NTN enhancements”

- R2-2208187, “Disabling HARQ feedback for IoT-NTN”, Interdigital, Inc.