3GPP TSG RAN WG2 #120	                                    			R2-2300582

Athens, Greece, 27th February – 3rd March, 2023

Agenda Item:	8.6.4

Source:	InterDigital, Inc.

Title:	IoT-NTN discontinuous coverage enhancements.

Document for:	Discussion

# Introduction

The WI on IoT NTN enhancements [1] includes the following objectives for discontinuous coverage enhancement:

4.1.3 Further enhancement to discontinuous coverage - Study and specify, if needed, mobility management enhancements and power saving enhancements for discontinuous coverage, taking into account the conclusions from the SA2 study FS_5GSAT_Ph2. [RAN2, RAN3].

The relevant conclusions from the SA2 study [2] are as follows:

## 8.1 Conclusion on general mobility management and/or power saving The conclusions for solutions to the requirements R1, R2 for KI#1 and R5 and R6 for KI#2 are the following: Principles of system behaviour: - NTN MEO/LEO satellite or satellite constellation that provides discontinuous coverage is considered as the satellite access in 5GS and EPS for the power saving enhancement and UE unreachability period can be provided to both the UE and AMF/MME. - UE unreachability period provided to the UE and/or AMF/MME shall include either timing information when UE moves out/in of NTN coverage or information on satellite coverage at current and potential future locations of the UE. - Both network centric and UE centric procedures are used to determine and coordinate the UE unreachability period. These two approaches are not mutually exclusive, they serve different use cases and they can co-exist in the same network. In Network centric procedure, the AMF/MME determines the PSM parameters taking into account the UE unreachability period. In UE centric procedure, the UE informs the network about UE determined UE unreachability period and/or an indication when leaving/entering coverage. AMF/MME information retrieval: - AMF/MME shall not determine UE unreachability period based on satellite orbit data by itself. The AMF/MME receives UE unreachability period to be used for setting up the power saving parameters or handling mobility management of NTN UE. - AMF/MME obtains UE unreachability period information (timing information when UE moves out/in of NTN coverage) from an AF via the NEF/SCEF, from the UE or from OAM. The AMF/MME also takes into account UE location/mobility/trajectory information. The role of AMF/MME: - The AMF/MME configures the UEs mobility management and Power saving mode (PSM) parameters and sends the MICO/PSM parameters to UE before the satellite discontinuous coverage starts. - Takes into account the UE unreachability period in order to make sure the UE is in power saving mode if it is leaving network coverage, to avoid attempting to page the UE during this time and that the UE need not initiate NAS procedures excessively frequently compared to the Expected UE behavior, if known by the AMF/MME. - The mobility management and MICO/PSM parameters include: - Periodic registration update timer / Periodic TAU Timer, - MICO mode with optional Active Time and PS, - MICO mode with Extended Connected time (only for AMF case), - eDRX parameters. - AMF/MME takes the Tracking Area reported by RAN before AN release as the paging area. - The AMF/MME sets an implicit detach timer based on the UE unreachability period information to avoid de-registering or detaching the UE when it is in discontinuous coverage. NOTE: Whether the UE can be in CM-connected with RRC inactive and the AN release procedure is needed when there is no satellite coverage can be discussed during the normative phase. - Optionally, AMF can provide a backoff timer to prevent the UE from sending MO data/signalling while out of coverage. - Request the UE to inform AMF/MME about when UE unreachability periods begin and end. Regarding which information UE has and how UE retrieves it: - If possible, accurate UE coverage information is shared with UE, i.e. UE coverage information received from the network with enhanced solution compared to Rel-17. - Satellite coverage data may be transferred to a UE from an external server or an AF. - Following Rel-17, the UE has satellite assistance information for predication of discontinuous coverage from RAN broadcasts. Editor's note: LS need to have feedback from CT1 on whether UP or NAS via an AMF/MME (CP) will be used for the transfer depends on the discussion. Regarding the role of the UE: - The capability for the UE to be able to determine requested NAS timer values and MICO/PSM parameters based on its predicted UE unreachability periods needs to be specified. A UE that uses power saving initiates a NAS procedure, with sufficient time to complete the NAS procedure before the start of the unreachability period, and negotiates timer values and power saving parameters with AMF/MME as per existing specifications.

In this contribution we discuss the potential impact.

# Background

## 2.1	Discontinuous coverage enhancement in  Rel-17 NB-IoT

In Rel-17, due to lack of time, only the information broadcast in SIB32/SIB32-NB is specified, and it’s up to UE implementation what to do during discontinuous coverage in RRC_IDLE and how to handle running timers and so on. The UE may choose not to perform idle mode tasks while in discontinuous coverage. Since most of the behavior is left to UE implementation the benefit of the information broadcast in SIB32/SIB32-NB is limited.

The following parameters are provided (same IE is used in both SIB32/SIB32-NB)

SystemInformationBlockType32 field descriptions

elevationAngleLeft, elevationAngleRight Leftmost and rightmost (with reference to the satellite direction) elevation angle. Unit in degree. Step of 5 degree. Actual value = field value * 5. If the field elevationAngleLeft is absent, the value of field elevationAngleRight applies.

footprintInfo Satellite footprint. E-UTRAN may configure elevationAngles and/or radius for earth moving satellite. E-UTRAN may configure referencePoint and radius for quasi earth fixed satellite.

latitude Latitude of the reference point. Unit in degree. Step of 360 / 262144 degree. Actual value = field value * (360 / 262144).

longitude Longitude of the reference point. Unit in degree. Step of 360 / 262144 degree. Actual value = field value * (360 / 262144).

radius Distance between the reference point and the edge of the satellite or beam coverage. Unit in km. Step of 10 km. Actual value = field value * 10.

serviceInfo Information on when the satellite will provide coverage. E-UTRAN always configures tle-EphemerisParameters for a satellite with earth moving cell(s) and always configures t-ServiceStart for a quasi-earth fixed satellite.

tle-EphemerisParameters Mean values of the satellite orbital parameters based on the TLE set format for estimating in-coverage and out-of-coverage periods for a satellite with earth moving cell(s), see TS 36.304 [4].

t-ServiceStart Time information on when the incoming satellite is going to start serving the area for quasi-earth fixed satellite.

# Discussion

## 3.1	RRC_IDLE and RRC_INACTIVE

For NB-IoT and eMTC, UE unreachability period prediction in RRC_IDLE and RRC_INACTIVE may provide an opportunity to save power even in R17, as the UE is allowed to suspend Idle mode tasks.

UEs may choose not to perform measurements of the serving cell or the neighbour cells during a UE unreachability period. The UE may avoid going to “any cell selection” state and start performing a scan of supported carriers/RATs during a UE unreachability period. Rather, the UE may choose to suspend measurements and reselection and rather “sleep” during the UE unreachability period in order to save battery life.

The other main benefit of suspending idle mode tasks would of course be DRX/eDRX and paging monitoring. If the UE is configured with a relatively short DRX (i.e. not eDRX) then it is likely that the paging cycle is shorter than a UE unreachability period. In this case the UE should simply be allowed not to monitor the paging occasions that fall within the UE unreachability period - this would anyway be impossible and allowing the UE to save power by avoiding trying to synchronise to a cell during this time would be beneficial.

In our understanding, the behaviour captured in R17 allows UE not to perform idle mode tasks, therefore this behaviour is already allowed. However, we should consider whether to clarify more explicitly.

Proposal 1: For eMTC and NB-IoT: A UE in R17 is allowed not to perform RRC_IDLE mode tasks during a UE unreachability period. Consider whether to explicitly clarify that this means that if a UE in RRC_IDLE or RRC_INACTIVE determines it is in a UE unreachability period, the UE may choose not to perform measurements of the serving cell or neighbour cells, and may postpone moving to “any cell selection” state, and is allowed not to attempt to monitor paging occasions which occur during a UE unreachability period.

According to SA2 [2] “The AMF/MME configures the UEs mobility management and Power saving mode (PSM) parameters and sends the MICO/PSM parameters to UE before the satellite discontinuous coverage starts.” and “The mobility management and MICO/PSM parameters include eDRX parameters”

In case the UE is configured with a long DRX (i.e. eDRX) then it may be the case that the UE is configured with a paging cycle which is longer than a UE unreachability period. In some cases, the PTW may fall within the UE unreachability period, and in extreme cases e.g. when there are many relatively long UE unreachability periods this may result in the UE being unreachable for long periods of time (i.e. much longer than the configured eDRX cycle due to missed PTWs). It may be possible by configuration to avoid this some of the time, however this may not be possible in all cases.

We should consider whether the PTW should be updated for UEs which would have no usable PTW during an eDRX cycle – for example, if the PTW falls within a UE unreachability period, to monitor for paging before or after the UE unreachability period. The PTW could be extended or shifted to accommodate the gap in coverage so that the UE can still be paged in a reasonable amount of time.

Alternatively it may be possible to redefine the PTW calculation such that UEs are distributed evenly only within the in-coverage H-SFNs. For example, based on the serving cell t-ServiceStart, we can calculate which H-SFNs will be out of coverage. By performing modulo using the UE-ID on the number of in-coverage H-SFNs we can distribute the UE PHs such that even UEs with a very long eDRX will have a PH while in-coverage.

Proposal 2: For eMTC and NB-IoT: RAN2 to consider updating the PH and PO calculations to take into account UE unreachability periods.

## 3.2	RLM, RLF

For NB-IoT, there is no support of connected mode mobility. As we know, mobility between cells relies on RLF and re-establishment to the new cell. In case of a UE unreachability period, the current procedure would cause the UE to start T310 upon detection of N310 out-of-sync indications from layer 1. Then, during T310 the UE would (unsuccessfully due to UE unreachability period) attempt to obtain in-sync with the current cell. Upon T310 expiry the UE would then attempt RRC re-establishment and (unsuccessfully due to UE unreachability period) attempt a cell selection. The UE would eventually declare RRC connection failure and enter idle mode, performing any cell selection until the UE unreachability period ends. Performing this frequently would result in unnecessarily high power consumption. It would also mean the UE is never able to re-establish on the new cell in the case that the UE unreachability period is longer than the maximum T310+T311 values because timers T310 and T311 would always expire. To avoid this, it should be allowed to suspend RLF and re-establishment for the duration of a UE unreachability period.

Currently in 36.304 it is stated “It is up to UE implementation to handle running timers.” but it is not clear whether this refers to RRC_CONNECTED (RRC spec) timers. In order to improve RRC_CONNECTED mode, we should specify how to handle RLF that would normally be declared when out of sync is detected due to out of coverage, to ensure the UE can recover more quickly when back in coverage.

Proposal 3: For eMTC and NB-IoT: RLM, RLF detection, and RRC re-establishment are suspended during a UE unreachability period.

## 3.3	RRC Reconfiguration

For RRC_CONNECTED, a gap in coverage would obviously impact mobility. For eMTC, the network may prefer to perform a handover from one cell to the new cell while keeping the UE in RRC_CONNECTED rather than having to release to IDLE to connect again later, particularly if the UE unreachability period is relatively short.

However, assuming that the handover command could be provided before the current cell coverage ends, a gap in coverage would result in RLF on the target cell as the UE would not be able to synchronise.

To support this we would suggest that for NTN the RRC Reconfiguration message could be enhanced to include an “activation time” to accommodate the UE unreachability period. If the UE receives an RRC Reconfiguration providing reconfiguration to a new cell and including an activation time, the UE should be allowed to not detect/declare RLF on the source cell, and delay the reconfiguration procedure until the activation time (i.e. UE unreachability period end time).

Proposal 4: For eMTC: To avoid always sending the UE to RRC_IDLE/RRC_INACTIVE during a UE unreachability period, introduce an activation time in RRC Reconfiguration to allow handover between cells occurring before and after a UE unreachability period.

In addition, time-based and location-based triggers for CHO have been agreed to be introduced for IoT-NTN in R18. If a CHO is provided with a target cell that will come into coverage after a gap, then CHO should take this into account, for example delay attempting to complete the CHO until after the UE unreachability period. This would be similar to providing an activation time in the RRC Reconfiguration.

Proposal 5: For eMTC: Consider how discontinuous coverage impacts CHO.

## 3.4	Reporting of UE unreachability period

According to the SA2 conclusions, “Both network centric and UE centric procedures are used to determine and coordinate the UE unreachability period. These two approaches are not mutually exclusive, they serve different use cases and they can co-exist in the same network. In Network centric procedure, the AMF/MME determines the PSM parameters taking into account the UE unreachability period. In UE centric procedure, the UE informs the network about UE determined UE unreachability period and/or an indication when leaving/entering coverage.”

According to SA2 [2] the “UE includes Unreachability Period in its Registration Request. The Unreachability Period tells the AMF the foreseen unreachability period caused by discontinuous satellite coverage in the UE location.”

Based on the SA2 study, it seems that no RRC reporting of UE unreachability period information is necessary. However, this assumes that the reporting in Registration request can be kept sufficiently up to date – a registration request is not usually sent very frequently, and depending on how often a registration request is triggered, the accuracy of UE location and hence UE unreachability period estimation may be impacted. If the UE unreachability estimation can be kept up to date and accurate then no RRC procedures would be needed to perform an update. However, it is not clear that this is the case.

Proposal 6:  RAN2 to discuss whether it can be assumed that reporting of UE unreachability period in Registration Request can be kept sufficiently (i.e. to support AS based solutions) up to date and takes into consideration UE mobility, or whether this needs to be confirmed with SA2.

# Conclusion

In this contribution we discuss the potential discontinuous coverage enhancements for R18 IoT-NTN and have the following proposals:

Proposal 1: For eMTC and NB-IoT: A UE in R17 is allowed not to perform RRC_IDLE mode tasks during a UE unreachability period. Consider whether to explicitly clarify that this means that if a UE in RRC_IDLE or RRC_INACTIVE determines it is in a UE unreachability period, the UE may choose not to perform measurements of the serving cell or neighbour cells, and may postpone moving to “any cell selection” state, and is allowed not to attempt to monitor paging occasions which occur during a UE unreachability period.

Proposal 2: For eMTC and NB-IoT: RAN2 to consider updating the PH and PO calculations to take into account UE unreachability periods.

Proposal 3: For eMTC and NB-IoT: RLM, RLF detection, and RRC re-establishment are suspended during a UE unreachability period.

Proposal 4: For eMTC: To avoid always sending the UE to RRC_IDLE/RRC_INACTIVE during a UE unreachability period, introduce an activation time in RRC Reconfiguration to allow handover between cells occurring before and after a UE unreachability period.

Proposal 5: For eMTC: Consider how discontinuous coverage impacts CHO.

Proposal 6:  RAN2 to discuss whether it can be assumed that reporting of UE unreachability period in Registration Request can be kept sufficiently (i.e. to support AS based solutions) up to date and takes into consideration UE mobility, or whether this needs to be confirmed with SA2.

# References

- RP-223519,  “Revised WID on IoT NTN enhancements”

- 23.700-28-i00, “Study on Integration of satellite components in the 5G architecture; Phase 2”