3GPP TSG-RAN WG2 Meeting #121	                                                                 R2-2300589

Athens, Greece, February-March, 2023

Agenda Item	:	8.5.2.3

Title	:	Discussion on PDU Discard

Source	:	Google Inc.

Document for	:	Discussion and Decision

# 1		Introduction

In response to the inquiry from RAN2, SA2 referred to their conclusion in the FS_XRM study (TR 23.700-60 [1]) and stated that they have reached agreement on the definition of new QoS parameters for the PDU-Set concept, including PDU-Set Error Rate (PSER), PDU-Set Delay Budget (PSDB) and the PDU-Set Integrated Handling Indication (PSIHI). Furthermore, SA2 stated that they have agreed the inclusion of a PDU-Set importance indicator which can be conveyed on a per-PDU-Set basis. It was emphasized that while all PDU-Sets within a single QoS flow should apply the same PSER, PSDB and PSIHI, the importance of individual PDU-Sets within the QoS flow may vary.

# 2		PDU Discard

According to SA4 (S4-220505) and in some implementations, not all PDUs in a PDU-set are of similar importance. A first PDU in a PDU-set can have higher importance than the last PDU in a PDU-set. The first PDU can be carrying source information and the last PDU could be carrying repair/redundant information.

At least RAN and the UE application could be made aware of the used video codec implementation, and the two implementations could be supported in the design and configurable by the network:

- Option 1: the remainder of the PDUs in the PDU Set should be discarded in case a PDU meets the discard criteria

- Option 2: the remainder of the PDUs in the PDU Set are still delivered in case a PDU meets the discard criteria.

Proposal 1: PDCP discard of PDU-Sets could be configurable per DRB.

SA2 also referred to their conclusion in the FS_XRM study that the PDU Set information ‘PDU Set importance’ may be used by NG-RAN for PDU Set level packet discarding in presence of congestion.

Also, according to the TR 23.700-60 [1], and in the definition below given to the PDU Set Importance, it seems this information has to be used by RAN in presence of congestion to carry discarding:

- This parameter is used to identify the importance of a PDU Set within a QoS flow. RAN may use it for PDU Set level packet discarding in presence of congestion.

Proposal 2: PDU Set importance indication is used by RAN and UE for discarding in addition to the PDCP discard timer. PDCP discarding by the UE based on PDU Set importance could be configurable to the UE.

Proposal 3: PDCP discard based on the PDU Set importance could be configurable per DRB.

In UL, once the UE decides to discard a PDU Set following the expiry of the discard timer or based on the PDU Set importance, the UE PDCP or RLC should inform the UE MAC/PHY in timely manner to send the cancellation indication of the CG-PUSCH occasions no longer needed to the gNB on the UCI (CG-UCI or other UCI). Then, the gNB can recycle the cancelled CG PUSCH occasions to schedule other UEs and hence improve the system capacity by not wasting UL resources. This will also allow the UE to save power.

Proposal 4: If the UE decides to discard a PDU Set, the UE transmits to the gNB a UCI cancellation indication to cancel the remaining CG-PUSCH occasions allocated for the PDU Set.

If the UE decides to discard a PDU-Set but it receives a gNB DCI scheduling retransmissions of some previously transmitted PDUs of the same PDU-Set that it had already decided to discard, the UE can ignore the scheduling DCI and the UE can use the allocated resources to send a discard indication to terminate the associated HARQ process to avoid additional retransmission scheduling.

Proposal 5: The UE ignores a DCI scheduling a retransmission of a PDU belonging to the discarded PDU-Set and requests the network to terminate the associated HARQ process.

The existing LCP mechanism for XR traffic can be improved by considering the remaining delay budget of the PDU-Sets to prevent discarding lower priority PDU-Sets. Higher importance PDU-Sets that arrive later should not take precedence over lower importance PDU-Sets close to expiry. The priority of the LCH could be adjusted based on the remaining delay budget of the PDU-Set. For instance, low importance PDU-Sets should receive more priority as their remaining delay budget decreases.

Proposal 6: Enhance existing LCP mechanisms by considering the remaining delay budget of the PDU-Sets to increase the priority of low importance PDU-Sets close to expiry.

In DL, the UE will receive PDUs of a PDU-Set and will buffer them to be decoded and processed. The UE can receive further information from gNB about the PDUs to prioritize the processing, like PDU-Set importance, PDU-Set remaining delay budget, etc… The UE can make use of this information for prioritization and discarding. However, this comes with extra signalling overhead and also giving this flexibility to the UE leads to reduced control at the gNB side.

Proposal 7: For the DL traffic, two options to be studied for the dynamic signalling of QoS parameters (e.g., importance) to the UE receiver:

- UE can be signalled with QoS parameters of the DL traffic to be used for prioritization and discarding of DL PDUs

- UE is not signalled with QoS parameters of the DL traffic and is not allowed to prioritize and discard DL PDUs.

When the UE discards a PDU-Set, it is useful for the gNB to know about the new UE buffer size.  Therefore, the UE can send a BSR report after each PDU-Set discarding to inform the gNB about the change in the buffer status

Proposal 8: PDU-Set discarding at the UE triggers a new BSR report.

# 3		Conclusions

In conclusion, we have the following proposals:

Proposal 1: PDCP discard of PDU-Sets could be configurable per DRB.

Proposal 2: PDU Set importance indication is used by RAN and UE for discarding in addition to the PDCP discard timer. PDCP discarding by the UE based on PDU Set importance could be configurable to the UE.

Proposal 3: PDCP discard based on the PDU Set importance could be configurable per DRB.

Proposal 4: If the UE decides to discard a PDU Set, the UE transmits to the gNB a UCI cancellation indication to cancel the remaining CG-PUSCH occasions allocated for the PDU Set.

Proposal 5: The UE ignores a DCI scheduling a retransmission of a PDU belonging to the discarded PDU-Set and requests the network to terminate the associated HARQ process.

Proposal 6: Enhance existing LCP mechanisms by considering the remaining delay budget of the PDU-Sets to increase the priority of low importance PDU-Sets close to expiry.

Proposal 7: In DL, two options to be studied for the signalling of QoS parameters (e.g. importance) to the UE per PDU:

- UE can be signalled with QoS parameters of the DL traffic to be used for prioritization and discarding of DL PDUs

- UE is not signalled with QoS parameters of the DL traffic and is not allowed to prioritize and discard DL PDUs.

Proposal 8: PDU-Set discarding at the UE triggers a new BSR report.

# 4		References

- TR 23.700-60 v18.0.0, Study on XR (Extended Reality) and media services (Rel-18).

- S4aV220921 ‘Reply LS on further details on XR traffic’ Reply LS on further details on XR traffic, e-meeting, 11th October 2022.