3GPP TSG-RAN WG2 Meeting #121                                                                        R2-2300591

Athens, Greece, February-March, 2023

Agenda Item	:	8.5.3

Title	:	XR-specific power saving enhancement

Source	:	Google Inc.

Document for	:	Discussion and Decision

# Introduction

In this Tdoc, we discuss the DRX support of XR frame rates corresponding to non-integer periodicities through at least semi-static mechanisms like RRC signalling

# C-DRX Enhancement

DRX enhancement to support the XR periodicities has been extensively discussed in the Rel-18 SI both in RAN1 and RAN2.

Dynamic adjustment is useful if we are addressing the C-DRX periodicity alignment simultaneously with the jitter handling. This requires however the network to have instantaneous or a priori knowledge about the jitter of the traffic burst arriving in the next C-DRX occasion which is not possible according to SA4 feedback [12]. Hence during the WI, we prefer to focus on the semi-static solutions.

Dynamic solutions can be explored later if more feedback from SA regarding the jitter is received.

- RAN2 does not assume instantaneous jitter is available or predictable unless further inputs from SA.

- Prioritize semi-static solutions for the periodicity alignment between C-DRX and XR traffic.

We prefer to keep the legacy approach by re-using the 38.321 equation with some modification. The equation should remain backward compatible for example by allowing the network to set some parameters to zero or skip the configuration of some parameters to fall-back to the legacy equation. This gives better flexibility at the network side and minimize the implementation changes at both the UE and the network sides. Also, this allows to adopt a single a solution for all sort of traffics.

- Re-use the 38.321 equation with some modification for the start of On-Duration timer .

In the MAC specification [TS 38.321], the formulas to start On-Duration timer are:

- For long DRX cycle: [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset , where SFN is the current System Frame Number, and the subframe number is the current subframe number.

- For short DRX cycle : [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle) .

We propose two possible solutions to update the above formulas:

- Solution 1:

In this solution, we adjust the drift every fixed number of C-DRX cycles and we adjust with a fixed offset. For example for a Frame Rate = 30 fps (33.33 ms periodicity) and drx-Cycle = 32, we will adjust every 3 cycles with an offset equal to 4.

We propose to introduce three new parameters:

- drx-CyclesAdjust: number of cycles to apply a correction shift

- drx-CyclesOffset: the value of the offset to apply every drx-CyclesAdjust cycles

- drx-CorrectionCounter: A counter to apply the offset

And apply the following algorithm:

  - Increment the counter:

drx-CorrectionCounter = 0;

if (([(SFN × 10) + subframe number] modulo [(drx-CorrectionCounter + 1) × drx-CyclesAdjust × drx-Cycle] = drx-CorrectionCounter × drx-CyclesOffset ) && [(SFN × 10) + subframe number - drx-StartOffset > 0 ]  )

drx-CorrectionCounter = drx-CorrectionCounter + 1;

  - Start of On-Duration:

[(SFN × 10) + subframe number] modulo (drx-Cycle) = (drx-StartOffset + [drx-CorrectionCounter × drx-CyclesOffset]) modulo (drx-Cycle), where SFN is the current System Frame Number, and the subframe number is the current subframe number.

The formulas are evaluated in Appendix case-1 and case-2.

Introducing a drx-Cycle = 33 can further reduce the residual error as shown in in Appendix case-3.

gNB can use the frame rate irreducible fraction to determine drx-CyclesAdjust and drx-CyclesOffset.

e.g. if Frame Rate = 30 fps and drx-Cycle = 32:

- 30 fps= 301000=3100

- drx-CyclesAdjust = 3

- drx-CyclesOffset = 100 - drx-CyclesAdjust × drx-Cycle = 4

The advantage of Solution 1 is that it allows to correct the drift by adding a fixed offset every fixed number of cycles. The solution is simple, predictable and requires only an RRC signalling to the UE of two extra parameters drx-CyclesAdjust and drx-CyclesOffset.

- Solution 2:

In solution 2, we take the C-DRX On-Duration time into consideration. Basically, if the traffic arrival time is within the On-Duration there is no need to correct the drift between the XR traffic arrival and the C-DRX cycles. The formula can be written as:

[(SFN × 10) + subframe number] modulo (drx-Cycle) = (drx-StartOffset + floor(m × d)) modulo (drx-Cycle)

The m is increased by 1 every n drx-Cycles and d = n × (xr_periodicity - drx-Cycle). The n can be given by gNB or a pre-defined value.

The formula is evaluated in Appendix case-4.

The advantage of this solution is that we correct the drift only when needed and there is no need to correct as long as the expected arrival time is still within the On-Duration.

# Conclusion

In this contribution, we made the following proposals

- RAN2 does not assume instantaneous jitter is available or predictable unless further inputs from SA.

- Prioritize semi-static solutions for the periodicity alignment between C-DRX and XR traffic.

- Re-use the 38.321 equation with some modification for the start of On-Duration timer .

# References

- S4aV220921 “Reply LS on further details on XR traffic” 3GPP TSG-WG SA4 VIDEO SWG call, E-meeting October 2022.

# Appendix

- Solution 1:

- Case 1:

  - Frame Rate = 30 fps

  - drx-Cycle = 32

  - drx-CyclesAdjust = 3

  - drx-CyclesOffset = 4

| Traffic Arrival | start On-Duration from the proposed formula | Residual Error |
| --- | --- | --- |
| 33.3333 | 32 | 1.33333 |
| 66.6667 | 64 | 2.66667 |
| 100 | 100 | 0 |
| 133.333 | 132 | 1.33333 |
| 166.667 | 164 | 2.66667 |
| 200 | 200 | 0 |
| 233.333 | 232 | 1.33333 |
| 266.667 | 264 | 2.66667 |
| 300 | 300 | 0 |
| 333.333 | 332 | 1.33333 |
| 366.667 | 364 | 2.66667 |
| 400 | 400 | 0 |
| 433.333 | 432 | 1.33333 |
| 466.667 | 464 | 2.66667 |
| 500 | 500 | 0 |
| 533.333 | 532 | 1.33333 |
| 566.667 | 564 | 2.66667 |
| 600 | 600 | 0 |
| 633.333 | 632 | 1.33333 |
| 666.667 | 664 | 2.66667 |
| 700 | 700 | 0 |
| 733.333 | 732 | 1.33333 |
| 766.667 | 764 | 2.66667 |
| 800 | 800 | 0 |
| 833.333 | 832 | 1.33333 |
| 866.667 | 864 | 2.66667 |
| 900 | 900 | 0 |
| 933.333 | 932 | 1.33333 |
| 966.667 | 964 | 2.66667 |
| 1000 | 1000 | 0 |
| 1033.33 | 1032 | 1.33333 |

- Case 2:

  - Frame Rate = 60 fps

  - drx-Cycle = 16

  - drx-CyclesAdjust = 3

  - drx-CyclesOffset = 2

| Traffic Arrival | start On-Duration from the proposed formula | Residual Error |
| --- | --- | --- |
| 16.6667 | 16 | 0.66667 |
| 33.3333 | 32 | 1.33333 |
| 50 | 50 | 0 |
| 66.6667 | 66 | 0.66667 |
| 83.3333 | 82 | 1.33333 |
| 100 | 100 | 0 |
| 116.667 | 116 | 0.66667 |
| 133.333 | 132 | 1.33333 |
| 150 | 150 | 0 |
| 166.667 | 166 | 0.66667 |
| 183.333 | 182 | 1.33333 |
| 200 | 200 | 0 |
| 216.667 | 216 | 0.66667 |
| 233.333 | 232 | 1.33333 |
| 250 | 250 | 0 |
| 266.667 | 266 | 0.66667 |
| 283.333 | 282 | 1.33333 |
| 300 | 300 | 0 |
| 316.667 | 316 | 0.66667 |
| 333.333 | 332 | 1.33333 |
| 350 | 350 | 0 |
| 366.667 | 366 | 0.66667 |
| 383.333 | 382 | 1.33333 |
| 400 | 400 | 0 |
| 416.667 | 416 | 0.66667 |
| 433.333 | 432 | 1.33333 |
| 450 | 450 | 0 |
| 466.667 | 466 | 0.66667 |
| 483.333 | 482 | 1.33333 |
| 500 | 500 | 0 |
| 516.667 | 516 | 0.66667 |

- Case 3:

  - Frame Rate = 30 fps

  - drx-Cycle = 33

  - drx-CyclesAdjust = 3

  - drx-CyclesOffset = 4

| Traffic Arrival | start On-Duration from the proposed formula | Residual Error |
| --- | --- | --- |
| 33.3333 | 33 | 0.33333 |
| 66.6667 | 66 | 0.66667 |
| 100 | 100 | 0 |
| 133.333 | 133 | 0.33333 |
| 166.667 | 166 | 0.66667 |
| 200 | 200 | 0 |
| 233.333 | 233 | 0.33333 |
| 266.667 | 266 | 0.66667 |
| 300 | 300 | 0 |
| 333.333 | 333 | 0.33333 |
| 366.667 | 366 | 0.66667 |
| 400 | 400 | 0 |
| 433.333 | 433 | 0.33333 |
| 466.667 | 466 | 0.66667 |
| 500 | 500 | 0 |
| 533.333 | 533 | 0.33333 |
| 566.667 | 566 | 0.66667 |
| 600 | 600 | 0 |
| 633.333 | 633 | 0.33333 |
| 666.667 | 666 | 0.66667 |
| 700 | 700 | 0 |
| 733.333 | 733 | 0.33333 |
| 766.667 | 766 | 0.66667 |
| 800 | 800 | 0 |
| 833.333 | 833 | 0.33333 |
| 866.667 | 866 | 0.66667 |
| 900 | 900 | 0 |
| 933.333 | 933 | 0.33333 |
| 966.667 | 966 | 0.66667 |
| 1000 | 1000 | 0 |
| 1033.33 | 1033 | 0.33333 |

- Solution 2:

  - Case 4:

    - Frame Rate = 30 fps

    - drx-Cycle = 32

    - On-Duration = 11ms

| Traffic Arrival | start On-Duration from the proposed formula | Residual Error |
| --- | --- | --- |
| 33.3333 | 32 | 1.33333333 |
| 66.6667 | 64 | 2.66666667 |
| 100 | 96 | 4 |
| 133.333 | 128 | 5.33333333 |
| 166.667 | 160 | 6.66666667 |
| 200 | 192 | 8 |
| 233.333 | 224 | 9.33333333 |
| 266.667 | 256 | 10.6666667 |
| 300 | 300 | 0 |
| 333.333 | 332 | 1.33333333 |
| 366.667 | 364 | 2.66666667 |
| 400 | 396 | 4 |
| 433.333 | 428 | 5.33333333 |
| 466.667 | 460 | 6.66666667 |
| 500 | 492 | 8 |
| 533.333 | 524 | 9.33333333 |
| 566.667 | 556 | 10.6666667 |
| 600 | 600 | 0 |
| 633.333 | 632 | 1.33333333 |
| 666.667 | 664 | 2.66666667 |
| 700 | 696 | 4 |
| 733.333 | 728 | 5.33333333 |
| 766.667 | 760 | 6.66666667 |
| 800 | 792 | 8 |
| 833.333 | 824 | 9.33333333 |
| 866.667 | 856 | 10.6666667 |
| 900 | 900 | 0 |
| 933.333 | 932 | 1.33333333 |
| 966.667 | 964 | 2.66666667 |
| 1000 | 996 | 4 |
| 1033.33 | 1028 | 5.33333333 |