3GPP TSG RAN WG2 Meeting #121	R2- 2300593

Athens, Greece Feb 27 – Mar 3, 2023

Agenda Item:	8.2.2 - Sidelink positioning

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	UE Positioning using Sidelink

Document for:	Discussion & Decision

# Introduction

The WID [1] for study on expanded and improved NR positioning enhancements in Rel. 18 lists among others, the following as RAN2 objectives:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2]

In this contribution, we provide our views on sidelink positioning.

# Coverage scenarios

For sidelink positioning, in certain deployment scenarios the roadside units may be the UEs that serve as anchor UEs for positioning. In these scenarios, the target UEs benefit from being able to measure the RSTDs without having to necessarily establish sessions with each of the anchors. The configuration of the SL-RS to be transmitted by the target UE to the anchor UEs or the other way around could be provided as broadcast or as preconfigured data.

Proposal 1: For the fixed infrastructure anchors, at least sessionless operation shall be supported.

One key argument in favour of session-based operation is protecting the privacy of measurement data between anchor and target UE, as this may have privacy concerns if broadcasted. However, the anchors can provide the measurement to the LMF directly in case of LMF-based and in case of target-UE based, a report free ranging  (please see Section 4) can be considered.

Proposal 2: Anchor UEs shall not broadcast the measurements to other UEs. The measurement shall only be transmitted in unicast or report free ranging shall be supported.

# Preconfigured AD with validity area for sidelink

In Rel. 17, RAN2 agreed on providing multiple instances of preconfigured AD to a UE, where each instance of AD is applicable to an area in the network. In case of positioning for V2X scenarios based on Uu interface, then the resulting scenario could be simplified as shown in Figure 1.

Figure 1: Example of preconfigured assistance data using RSU using Rel. 17 Uu-based positioning methods.

If self-localization of a UE is targeted, as depicted above and the TRPs were to be replaced by anchor UEs to enable sidelink positioning, the UE would be able to receive the sidelink reference signal emitted by the anchor UEs and compute its position. The configuration of sidelink anchor UEs (road-side units) could be provided as preconfigured assistance data. In a hybrid scenario, this would look like Figure 3.

Figure 2: Example of application of preconfigured assistance data to a hybrid modus with RSU consisting of TRPs and anchor UEs (Sidelink).

If a UE is preconfigured with AD instances applicable to Area 1 and Area 2, where the UE either computes its own location or provides the measurements to the network via LPP, then active sessions between the sidelink UEs are not necessary. The location of sidelink UEs may be stationary (e.g. fixed RSUs) or semi-stationary (e.g. stationary devices deployed as anchors). Such assistance data can be preconfigured in the same manner as the assistance data for UL/DL positioning.

Observation 1: Sidelink positioning can be supported in sessionless mode when preconfigured assistance data is provided to the target UE, as long as anchors are at least semi-stationary.

Proposal 3: Providing sidelink reference signal configuration using  preconfigured assistance data mechanism shall be supported.

In Rel. 17, we discussed validity criteria for preconfigured assistance data as a means for reducing latency. For Uu-interface, the TRPs which served as positioning anchors were fixed in space. The anchors in sidelink scenario may be static, semi-static or mobile. Accordingly, the assistance data shall reflect that some components of assistance data shall have a time-validity.

Proposal 4: Elements of preconfigured assistance data may be configured with time validity in addition to area validity.

# Report-free ranging operation for sidelink

Figure 3: Depiction of report free sidelink positioning in target-UE-based mode.

For sidelink positioning, if the ranging, using round trip time, is done the classical way where each side target UE and anchor UE report the RxTxDiff to the LMF (in UE assisted network based mode) or where the anchor UEs report the RxTxDiff to the target UE (in target UE based, anchor UE assisted mode), this leads to high number of packets that need to be exchanged over the wireless interface. This can be reduced by enabling report-free ranging operation where a UE initiates a transmission of sidelink operation and the second UE responds to the UE maintaining a fixed time between reception and transmission. Now the initiating UE computes the RxTxTimeDiff and subtracts the fixed value (corresponding to the fixed timing maintained by its counterpart) from the computed value.

The advantages of report-free ranging operation are as follows:

- Reduction in congestion (no packets to be transmitted/retransmitted), which arises from the fact that positioning reference signals can be received and measured at much lower values compared to the SNRs needed for successful packet decoding.

- Reduction in latency, which arises from not having to establish separate connections with sidelink UEs to exchange measurements.

- Privacy protection, because the range cannot be determined if the responding UE does not respond. This may not be an issue for infrastructure UEs (such as roadside units) but could be privacy issue for UEs attached to people or objects.

One of the ways a fixed response time can be maintained is by cyclically shifting the transmitted sequence taking into account the Rx-time and the true Tx-time.

Observation 2: Signalling overhead is significantly reduced by using report-free RTT. No additional messages for providing measurements are needed for this approach, and ProvideLocationInformation is one of the most frequently exchanged message in a positioning session.

Observation 3: A UE initiating sidelink positioning reference signal for ranging does not need to receive RxTxDiff from another UE, as long as its counterpart transmits a signal maintaining (at least virtually) a fixed value of transmit-receive time difference.

Proposal 5: Determination of ranging between two UEs in sidelink without having to report the RxTxTimeDiff shall be supported.

# Session-based operation for sidelink

A sessionless approach, where the target UE obtained its position on a best effort basis, relied on assuming that a third UE would be able to perform measurements, transmit reference signals, receive reference signals, provide measurements and so on. This approach would rely on the best-effort cooperation by the third UE and the target UE cannot rely on cooperation from a third UE without a commitment. For example, a UE whose location may be critical for providing a required DOP may fail to provide measurement, possibly due to its own traffic.

Figure 4: Anchor participating in a session depending on UE location

A session-based operation may attempt to provide a more reliable positioning operation, by searching and commiting UEs to cooperate in a positioning session. Providing a session may be beneficial for tracking application, which may consist of one or more deferred MT-LRs or deferred MO-LRs.

A session-based operation shall

- Actively attempt to achieve the QoS criteria requested by the location client

- Establish dedicated signalling connection with the anchor UEs andcommit them to  perform certain measurements or transmitting reference signal with certain configuration

- Select and reselect the anchors suitable for positioning

- Monitor the anchors – remove existing anchors or add new anchors in the session as needed. Allocate resources or reconfigure, such that the positioning operation can be carried out without hindering the UE operation

- Attempt to optimize resource use.

Proposal 6: A session may be formed by combining of one or more deferred location requests. A session may have more than one request and more than one LCS response.

Proposal 7: Session-based operation shall rely on monitoring the parameters needed for ensuring quality of service. Parameters such as line-of-sight conditions, DOP, multipath scenario at the receiver may be monitored.

The optimal set of constellation depends on the coarse location of UE itself. Hence, initial parameters shall be selected from measurements done on a best effort basis. For example, a LOS condition can be determined without having to use a 100 MHz signal (which may be needed later to ensure accuracy).

Proposal 8: Initial parameters for selecting the UEs for a session shall rely on measurements done in a sessionless operation.

A UE managing the session could support ranging by hiding the identities of two UEs ranging among themselves from each other acting as an intermediary. From the perspective of target and anchors, they are transmitting reference signals and reporting measurement, without knowing who they are ranging with.

# Conclusions

In this contribution, we have made the following observations:

Observation 1: Sidelink positioning can be supported in sessionless mode when preconfigured assistance data is provided to the target UE, as long as anchors are at least semi-static.

Observation 2: Signalling overhead is significantly reduced by using report-free RTT. No additional messages for providing measurements is needed for this approach, and ProvideLocationInformation is one of the most frequently exchanged message in a positioning session.

Observation 3: A UE initiating sidelink positioning reference signal for ranging does not need to receive RxTxDiff from another UE, as long as its counterpart transmits a signal maintaining (at least virtually) a fixed value of transmit-receive time difference.

Based on the above observations, we make the following proposals:

Proposal 1: For the fixed infrastructure anchors, at least sessionless operation shall be supported.

Proposal 2: Anchor UEs shall not broadcast the measurements to other UEs. The measurement shall only be transmitted in unicast or report free ranging shall be supported.

Proposal 3: Providing sidelink reference signal configuration using  preconfigured assistance data mechanism shall be supported

Proposal 4: Elements of preconfigured assistance data may configured with time validity in addition to area validity.

Proposal 5: Determination of ranging between two UEs in sidelink without having to report the RxTxTimeDiff shall be supported.

Proposal 6: A session may be formed by combining of one or more deferred location requests. A session may have more than one request and more than one LCS response.

Proposal 7: Session-based operation shall rely on monitoring the parameters needed for ensuring quality of service. Parameters such as line-of-sight conditions, DOP, multipath scenario at the receiver may be monitored.

Proposal 8: Initial parameters for selecting the UEs for a session shall rely on measurements done in a sessionless operation.

# References

| [1] | RP-223549, "New WID on Expanded and Improved NR Positioning," 3GPP, Online, 2022-12. |
| --- | --- |