3GPP TSG-RAN WG2 Meeting #121	R2-2300598

Athens, Greece, 27th February – 3rd March, 2023

Agenda item:	8.5.2.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on PDU set discarding for XR traffic

Document for:	Discussion and Decision

# Introduction

UL PDU set discarding has been agreed to be supported at PDCP in RAN2#119 bis.

1. For UE transmitter, the PDCP discard should be performed per PDU set basis.

2. For UE transmitter, The PDCP discard is managed per SDU for PDU set, the PDCP entity discards all PDCP SDUs associated with the PDU set.

RAN2#120 further agreed that the UL PDU set discarding contains the timer-based discarding (at PDCP) and other potential aspects.

For Uplink

1. Handling of discard FFS.

2. RAN2 to support timer-based discarding of UL transmit side of PDCP PDU/SDUs of a PDU set. FFS how this is modelled in PDCP specification, can be discussed in WI phase.

In this document, we will discuss whether the current PDCP SDU/PDU discarding mechanism is sufficient for the PDU set discarding and what enhancements are potentially needed.

# Discussion

SA2 defined a new QoS parameter “PDU Set Integrated Handling Indication” (PSIHI) to indicate if all PDUs of the PDU set are needed for the application layer to be able to use it, see the following text quoted from S2-2301472 agreed in SA2#154 [1]. If the PDU Set Integrated Handling Indication indicates that not all PDUs are needed, then even if one or more PDUs have been lost, the remaining PDUs of the same PDU set still need to be transmitted. Hence, PDU set level discarding should be configurable by the network based on the received PSIHI information. The network would normally configure it only for the DRBs containing QoS flows configured with the “PDU Set Integrated Handling Indication”, and the UE will apply PDU set level discarding only to such DRBs while the legacy PDU discarding will be applied otherwise.

The PDU Set Integrated Handling Indication (PSIHI) indicates that whether all PDUs are needed for the usage of the PDU Set by the application layer in the receiver side.

- PDU set level discarding should be configurable per DRB.

A PDU set will not be valid forever and its validity is related to PSDB, see the following text quoted from TR 23.700-60 [2]. From the definition of PSDB we observe that jitter is not considered when defining the upper bound of the time delay for a PDU set. Therefore, we propose that when designing the timer based PDU set discarding mechanism, UL jitter is not considered either.

PDU Set Delay Budget: The PSDB defines an upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, i.e. time between reception of the first PDU and the successful delivery of the last arrived PDU of a PDU Set. PSDB applies to the DL PDU Set received by the UPF over the N6 interface, and to the UL PDU Set sent by the UE. For a certain 5QI the value of the PSDB is the same in UL and DL.

- Jitter is not considered in the definition of PSDB.

- UL jitter is not considered in the design of the timer based PDU set discarding mechanism.

Figure 1 illustrates the meaning of PSDB. Assuming that the CN-PDB of each PDU within a PDU set is the same, then the AN-PDBs of these PDUs are different, the former PDU has a looser delay requirement than the later one. The range of such difference depends on the arrival time of the first and the last PDU.

Figure 1: illustration of PSDB

- The AN-PDB of a PDU within a PDU set may differ and the difference range depends on the duration of the inter arrival time between the first and the last PDU.

It should be noted that according to SA2 specifications (see [1]), the inter-arrival time is subject to SLA, so it is known to the network in advance. On the other hand, there is no mention of the possible inter-arrival time values, which can depend on a particular application:

NOTE 1: To enable support for PSDB, it is assumed that there is a maximum duration threshold for inter arrival time between the first received PDU and the last received PDUs constituting a the PDU Set as per SLA or pre-configuration.

The duration of the inter-arrival time between the first and the last PDU of the PDU set may have an impact on the PDCP discarding mechanism, which is elaborated below.

If the duration of the inter arrival time between the first and the last PDU is small, e.g., less than 1 ms, then all PDUs of a PDU set arrive at almost the same time. Legacy PDCP discard timers for each of these PDUs would be started almost at the same time and consequently, if they expire, they would also expire almost simultaneously.

- If the duration of the inter arrival time between the first and last PDUs is small, the current PDCP discardTimer is sufficient for the timer-based PDU set discarding.

If the duration of the inter arrival time between the first and the last PDU is large, the current PDCP discardTimer mechanism is not sufficient anymore. As Figure 2 explains, the length of the PDCP discard timer is configured per RB today, as long as the PDUs of a PDU set are mapped to a single DRB, the PDCP entity will use the same length of PDCP discard timer for these PDUs. Then a problem happens as when the previous PDUs have been discarded due to PDCP discardTimers expiry, the later PDUs are also expected to be discarded but may still be in the UE. In order to overcome this problem, 3 possible options are explored:

Figure 2: PDUs of a PDU set are discarded at different time

Option 1: Introduce a new PDU set level discard timer, with enhancements to the SDU discard procedure.

The new PDU set discard timer is maintained per PDU set. A PDU set discard timer is started when the first PDU of a PDU set arrives, and the expiry of a PDU set discard timer triggers the PDCP entity to discard all SDUs/PDUs belong to the PDU set.

One question which needs to be answered is whether the legacy PDCP discard timer should still be used. We can specify that the new PDU set level discard timer and the legacy PDU level discard timer do not work simultaneously, alternatively we can continually use the legacy PDCP discard timer but set the time length to be “infinity” or other large value, once the PDU set level discard timer expires, all of the associated PDU level discard timers are regarded as expired.

Option 2: No new timer is introduced, but the length of PDCP discard timers vary for different PDUs.

For example, the 1st PDU arrives at time 0 and the associated PDCP discard timer is set to be 10 ms, 3ms later the 2nd PDU arrives, then its associated PDCP discard timer will be 7ms (i.e., the current value of the previous PDU’s PDCP discard timer), another 2ms later the 3rd PDU arrives and its associated PDCP discard timer is set to be 5ms, and so on. By doing so, all of the PDCP discard timers associated to the PDUs of the same PDU set will expire at the same time.

Option 3: No new timer is introduced, but expiry of the first PDCP discard timer triggers the discard of the whole PDU set.

More specifically, once the PDCP discard timer of the 1st PDU expires, all the PDUs within the same PDU set shall be discarded. In other words, once the 1st PDU’s discard timer expires, the discard timers of the other PDUs belong to the same PDU set are regarded as expired.

All the above options achieve the same goal, but among them, option 3 reuses the current PDCP procedure to the largest extent.

- Existing PDCP discard timer can be reused for PDU set level discarding, i.e. the expiry of the 1st PDU’s discard timer triggers discard of the whole PDU set.

In addition to the timer based PDU set discarding discussed above, PDU loss can also trigger the PDU set discarding, see the following text quoted from TR 38.835 [3]. As soon as the sender knows a PDU set has no chance of being decoded because too many PDUs were lost already, the sender can discard the remaining PDUs of the PDU set to save the radio resources.

When a certain number of PDUs of a PDU set are known to be required by the application layer to use the corresponding unit of information (for instance due to the absence or limitations of error concealment techniques, see TR 26.926 [6]), as soon as the number of PDUs known to be lost exceeds this number, the remaining PDUs of that PDU set are no longer needed by the application and may be subject to discard operation (see subclause 5.3.2).

The question that needs to be answered is how the sender can know when to discard a PDU set due to PDU loss? There are two possible ways:

- Sender controlled way, as shown in Figure 3, where the sender maintains the PDU set & member PDUs relationship. Based on the receiver's status report, the sender knows whether the PDU set has no chance to be decoded because too many PDUs were lost. If so, the sender can skip the current PDU set's transmission and continue to process the next PDU set;

- Receiver controlled way, as shown in Figure 4, where the receiver is aware of the PDU set & member PDUs relationship and monitors its reception status. When too many PDUs of the PDU set have been lost, the receiver indicates the sender to stop transmitting the current PDU set. Note that in-band signaling has been excluded due to a lot of standard changes and a waste of radio resource in RAN2#120 [4], therefore the receiver can’t get the PDU set & member PDUs relationship from the sender via UP, it can get the information from the sender via CP signaling or be notified by CN signaling. However, either way would lead to the problem of signaling blocking the data transmission as the PDU cannot be transmitted until the signaling carrying PDU set & member PDUs relationship is successfully processed. Moreover, additional CP signaling from the sender will consume the radio resources which is against the goal of capacity improvement.

Figure 3: Sender-controlled way

Figure 4: Receiver-controlled way

As can be seen, compared to the receiver controlled way, the sender controlled way is much simpler with significantly less impact to the current protocol design and operation. Therefore, some kind of status report about the lost PDUs can be used to allow the sender to discard the remaining PDUs of the PDU set when the PDU set cannot be decoded already, e.g. SR at the PDCP or at RLC layer.

- RLC or PDCP status report can be used to inform the transmitter that one or more PDUs of the PDU set have been lost, allowing the sender to discard the remaining PDUs of the same PDU set.

# Conclusion

In this contribution, we have discussed the PDU set discarding for XR traffic and made the following observations and proposals:

- Jitter is not considered in the definition of PSDB.

- The AN-PDB of a PDU within a PDU set may differ and the difference range depends on the duration of the inter arrival time between the first and the last PDU

- If the duration of the inter arrival time between the first and last PDUs is small, the current PDCP discardTimer is sufficient for the timer-based PDU set discarding.

- PDU set level discarding should be configurable per DRB.

- UL jitter is not considered in the design of the timer based PDU set discarding mechanism.

- Existing PDCP discard timer can be reused for PDU set level discarding, i.e. the expiry of the 1st PDU’s discard timer triggers discard of the whole PDU set.

- RLC or PDCP status report can be used to inform the transmitter that one or more PDUs of the PDU set have been lost, allowing the sender to discard the remaining PDUs of the same PDU set.

# Reference

- S2-2301472, SA2#154, Support of PDU Set based handling.

- 3GPP TR 23.700-60 V18.0.0, Study on XR (Extended Reality) and media services (Release 18).

- 3GPP TR 38.835 V1.0.0, Study on XR enhancements for NR (Release 18).

- Chairs note of RAN2#120, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_120/Inbox/Chairs_Notes/RAN2-120%20LTE%20DCCA%20MUSIM%20Slicing%2071%20GHz%20QoE%20XR%20(Tero)_2022-11-18-1432.docx