3GPP TSG RAN2 #121										R2-2300603

Athens, Greece, 27th February – 3rd March, 2023

Agenda Item:		8.14.5	Other topics

Source:			Huawei, HiSilicon

Title:		QoE continuity between LTE-5GC and NR

Document for:		Discussion and Decision

# 1 Introduction

In the WID [1], one objective concerns QoE continuity between LTE/5GC and NR.

- Support the continuity of legacy QoE measurement job for streaming and MTSI service during intra-5GC inter-RAT handover process [RAN2, RAN3].

In this paper, we would like to discuss the possible solutions in order to meet the objective.

# 2 Discussion

## 2.1  Discussion on the requirements

LTE QoE feature was introduced in Rel-15, and the feature summary can be found in [2], while NR QoE feature was introduced in Rel-17 with its summary in [3].

LTE QoE supports the following functionalities:

- The supported services are: streaming, MTSI

- Both signalling based and management based cases are supported

- Both configuration and reporting are encapsulated in containers in RRC signalling

- Only one QoE measurement collection session is supported

Rel-17 NR QoE supports more functionalities than LTE QoE, e.g.:

- It can support more than one QoE measurement collection session, e.g. enabled by measConfigAppLayerId

- RAN visible QoE

- Pause/resume due to RAN overload

- RRC segmentation for QoE reporting

- Other functionalities

Before discussing the issue of QoE continuity and potential solutions, it is important to understand the requirements. We think there can be two interpretations for the QoE continuity requirement:

Understanding 1: The UE continues QoE measurements in the new RAT, but suspends the QoE reporting in the target RAT

For example, the UE is in RAT1 with an ongoing QoE measurement QoE1. When it moves to RAT2, QoE1 reporting is suspended and when the UE comes back to RAT1, QoE1 reporting can continue.

Figure 1: scenarios for understanding 1

For understanding 1, there may be the following use cases:

- Use case 1: NR -> LTE/5GC -> NR

- Use case 2: LTE/5GC -> NR -> LTE/5GC

Understanding 2: The UE continues QoE measurements and QoE reporting in the target RAT

For example, UE is in RAT1 with an ongoing QoE measurement QoE1. When it moves to RAT2, QoE1 measurements and reporting is continued and QoE reports are sent to RAT2.

Figure 2: scenarios for understanding 2

For understanding 2, there may be the following use cases:

- Use case 3: NR -> LTE/5GC

- Use case 4: LTE/5GC -> NR

Table below describes the main principles that could be applied to different use cases mentioned above while section 2.2 provides some technical analysis for those.

| Understanding | Use case | Principle |
| --- | --- | --- |
| 1 (The UE continues QoE measurements in the new RAT, but suspends the QoE reporting in the target RAT) | 1 (NR -> LTE/5GC -> NR) | During the inter-RAT HO and when the UE stays in LTE/5GC, the UE can pause all QoE measurements based on network indications. When the UE goes back to NR, the previous QoE measurements can be continued. |
| 1 | 2 (LTE/5GC -> NR -> LTE/5GC) | During the inter-RAT HO and when the UE stays in NR, the UE can pause its QoE measurements based on network indications. When the UE goes back to LTE/5GC, the previous QoE measurements can be continued. |
| 2 (The UE continues QoE measurements and QoE reporting in the target RAT) | 3 (NR -> LTE/5GC) | When the UE goes into LTE/5GC, one NR QoE measurements can be continued, and the UE can report the QoE measurements to LTE/5GC. |
| 2 | 4 (LTE/5GC -> NR) | When the UE goes into NR, LTE QoE measurements can be continued, and the UE can report the QoE measurements to NR. |

## 2.2  Solutions and standard impacts

For use cases listed in section 2.1, solutions and standard impacts are provided as below.

### 2.2.1  Analysis on use case 1

Principle (NR->LTE/5GC->NR):

Source NR node sends RRC message to UE to pause the NR QoE. Later, when the UE goes back to NR, the NR can indicate the UE to continue the previous NR QoE reporting.

The solution for use case 1 is:

- When the UE stays in NR:

  - NR configures QoE measurements to the UE, and then sends the pause indication to the UE. The indication can be included in inter-RAT HO command.

- When the UE goes to LTE/5GC:

  - The UE pauses all of NR QoE measurements (following the definitions in TS 38.331). The UE needs to store the QoE configurations which are intended to continue in the target RAT even though its whole NR RRC configuration is released.

  - The source RAT needs to provide target with information about the QoE configurations stored and performed at the UE.

- When the UE goes back to NR:

  - The source RAT provides the information about the QoE configurations stored/ongoing at the UE, so that the target RAT can now fetch the reports.

  - NR sends the resume indication to the UE to continue QoE reporting.

Possible specification impacts:

For TS 36.331, there are no impacts.

For TS 38.331, there may be some impacts due to inter-RAT HO command and QoE configurations storing.

TS 38.300 may need some updates due to this new feature.

For TS 38.423, whether existing QoE config in HANDOVER REQUEST procedure can cover LTE QoE config or not can be checked.

### 2.2.2  Analysis on use case 2

Principle (LTE/5GC->NR->LTE/5GC):

Source LTE/5GC node sends RRC message to UE to pause the LTE QoE, which has some impacts to TS 36.331. Later, when the UE goes back to LTE/5GC, the LTE/5GC can indicate the UE to continue the previous LTE QoE reporting.

The solution for use case 2 is:

- When the UE stays in LTE/5GC:

  - LTE/5GC configures QoE measurements to the UE, and then sends the pause indication to the UE. The indication can be included in inter-RAT HO command.

- When the UE goes to NR:

  - The UE pauses the LTE QoE measurements and stores the QoE configuration.

  - The source RAT needs to provide target with information about the QoE configuration stored and performed at the UE.

- When the UE goes back to LTE/5GC:

  - The source RAT provides the information about the QoE configuration stored/ongoing at the UE, so that the target RAT can now fetch the reports.

  - LTE/5GC sends the resume indication to the UE to continue QoE reporting.

Possible specification impacts:

For TS 36.331, pause/resume mechanism needs to be introduced.

For TS 38.331, there are no impacts.

TS 38.300 may need some updates due to this new feature.

For TS 38.423, whether existing QoE config in HANDOVER REQUEST procedure can cover LTE QoE config or not can be checked.

### 2.2.3  Analysis on use case 3

Principle (NR->LTE/5GC):

When the UE goes to LTE/5GC from NR, one previous QoE measurement can be activated in LTE/5GC (while others can be paused or released), and the QoE measurement and reporting will continue in LTE/5GC.

The solution for use case 3 is:

- When the UE stays in NR:

  - NR configures QoE measurements to the UE. For example, app meas id 1 (streaming), app meas id 2 (streaming), app meas id (MTSI), app meas id (VR).

- When the UE goes to LTE/5GC:

  - For the HANDOVER REQUEST procedure, NR can send indications to LTE/5GC, and then LTE/5GC can decide which of QoE measurements can be continued (only one QoE measurement) and LTE/5GC can inform NR. For example, app meas id 1 can be continued after inter-RAT HO, and other app meas ids are paused. As an alternative, NR can also make a decision, e.g. based on QoE priorities, and then NR can indicate which QoE configuration should be kept by LTE/5GC.

  - For the inter-RAT HO command, the network can indicate the UE to continue one QoE measurement (e.g. app meas id 1).

  - It is noted that the app meas id will be maintained in both UE and LTE/5GC sides, but it seems no need to add the id into LTE RRC signallings.

Possible specification impacts:

For TS 36.331, there will be some impacts related to configuration of the QoE measurements from NR.

For TS 38.331, there may be some impacts due to inter-RAT HO command.

TS 38.300 may need some updates due to this new feature.

For TS 38.423, NR or LTE/5GC can decide which of QoE measurements can be continued and inform NR, which has impacts to Xn. In addition, whether existing QoE config in HANDOVER REQUEST procedure can cover LTE QoE config or not can be checked.

### 2.2.4  Analysis on use case 4

Principle (LTE/5GC->NR):

When the UE goes to NR from LTE/5GC, the previous QoE measurement can be continued in NR. During the inter-RAT HO, the target RAT can also configure other QoE measurements to the UE.

The solution for use case 4 is:

- When the UE stays in LTE/5GC:

  - LTE/5GC configures QoE measurements to the UE.

- When the UE goes to NR, and an inter-RAT HO happens:

  - For the HANDOVER REQUEST procedure, the source LTE/5GC needs to send the LTE QoE configuration to the target NR. In addition, the measConfigAppLayerId IE may need to be added as part of LTE QoE configurations, so that the target NR can manage this QoE measurement. For the HANDOVER REQUEST procedure, currently it has been defined that when there is a HO from LTE/5GC to gNB, the HandoverPreparationInformation message defined in TS 38.331 is used. Inside the message, the QMC Configuration Information rrcReconfiguration IE can include the previous NR QoE configuration (introduced for the purpose of NR QoE feature). It needs to be discussed whether the existing IE can cover LTE QoE configuration or not, and if not, what enhancements will be needed.

  - For the inter-RAT HO command, the network can indicate the QoE configurations to the UE, e.g. the network can just indicate the previous LTE QoE measurements (including the measConfigAppLayerId IE), and optionally new QoE measurements. For other functionalities like pause/resume, transmission of session start/stop, RAN visible QoE reporting, we think there is no need to introduce them into TS 36.331 due to extra complexities.

Possible specification impacts:

For TS 36.331, there may be some impacts due to inter-RAT HO command.

For TS 38.331, there may be some impacts related to ensuring continuation of QoE measurement from LTE when the UE moves to NR.

TS 38.300 may need some updates due to this new feature.

For TS 38.423, whether existing QoE config in HANDOVER REQUEST procedure can cover LTE QoE config or not can be checked.

### 2.2.5  Summary

The following table summarizes solutions for use case 1/2/3/4:

| Use case | Solution principles | RAN2 impacts | RAN3 impacts |
| --- | --- | --- | --- |
| 1 (NR -> LTE/5GC -> NR) | Source NR node sends RRC message to UE to pause the NR QoE. Later, when the UE goes back to NR, the NR can indicate the UE to continue the previous NR QoE reporting. | TS 36.331: no impacts TS 38.331: impacts to inter-RAT HO command and QoE configurations storing TS 38.300: stage-2 description | TS 38.423: check whether existing QoE config in HANDOVER REQUEST procedure can cover LTE QoE config or not |
| 2 (LTE/5GC -> NR -> LTE/5GC) | Source LTE/5GC node sends RRC message to UE to pause the LTE QoE, which has some impacts to TS 36.331. Later, when the UE goes back to LTE/5GC, the LTE/5GC can indicate the UE to continue the previous LTE QoE reporting. | TS 36.331: impacts due to the introduction of pause/resume mechanism TS 38.331: no impacts TS 38.300: stage-2 description | The same as for use case 1 |
| 3 (NR -> LTE/5GC) | When the UE goes to LTE/5GC from NR, one previous QoE measurement can be activated in LTE/5GC (while others can be paused or released), and the QoE measurement and reporting will continue in LTE/5GC. | TS 36.331: impacts related to configuration of the QoE measurements from NR TS 38.331: impacts to inter-RAT HO command TS 38.300: stage-2 description | TS 38.423: NR or LTE/5GC can decide which of QoE measurements can be continued and inform NR, which has impacts to Xn. In addition, whether existing QoE config in HANDOVER REQUEST procedure can cover LTE QoE config or not can be checked. |
| 4 (LTE/5GC -> NR) | When the UE goes to NR from LTE/5GC, the previous QoE measurement can be continued in NR. During the inter-RAT HO, the target RAT can also configure other QoE measurements to the UE. | TS 36.331: impacts to inter-RAT HO command TS 38.331: some impacts related to ensuring continuation of QoE measurement from LTE when the UE moves to NR TS 38.300: stage-2 description | The same as for use case 1 |

Based on the above analysis, we observe that solutions for use case 3 and 4 have more standard impacts than solutions for use case 1 and 2, so it is proposed to start with discussing use case 1 and 2.

Proposal 1: It is proposed RAN2 starts with discussing solutions for use case 1 and 2, considering the solution details and possible specification impacts.

## 2.3  Coordination between RAN2 and RAN3

For the objective, RAN2 is the leading WG and RAN3 is the secondary WG. Once RAN2 has made some progress, in particular on clarifying which use cases as described above need to be supported, it would be helpful for RAN2 to send a LS to RAN3 so that RAN3 can start some technical discussions, e.g. check impacts to their specifications.

In the past, when the LTE/5GC feature was introduced in Rel-15, there were some discussions on whether a legacy feature (defined in legacy LTE) can be supported in LTE/5GC or not. At that time, the common understanding was that from RAN2 specifications point of view, a legacy feature (e.g. LTE QoE) should also work in LTE/5GC. However, it was not entirely clear whether RAN3 specifications can support LTE QoE with 5GC. In general, for the objective, the relevant question is whether LTE QoE feature can be supported in LTE/5GC from RAN3 point of view, which would need to be discussed in RAN3 as well.

Proposal 2: RAN2 should make an assumption on the intended use cases to support for QoE continuity and an LS should be sent to RAN3 to ask for feasibility and their view (including whether LTE QoE feature can be supported in LTE/5GC or not).

# 3 Conclusion

In this paper, we discuss QoE continuity between LTE/5GC and NR. Firstly, we think LTE related impacts should be minimized. Secondly, for the QoE continuity requirement, we observe two understandings and there are some use cases for them. For each use case, we provide technical analysis on principle, solution details, and possible specification impacts. So we have the following proposals:

Proposal 1: It is proposed RAN2 starts with discussing solutions for use case 1 and 2, considering the solution details and possible specification impacts.

Proposal 2: RAN2 should make an assumption on the intended use cases to support for QoE continuity and an LS should be sent to RAN3 to ask for feasibility and their view (including whether LTE QoE feature can be supported in LTE/5GC or not).

# 4 References

[1] RP-223488, Enhancement on NR QoE management and optimizations for diverse services, China Unicom

[2] RP-181821, Summary for WI Quality of Experience (QoE) Measurement Collection for streaming services in E-UTRAN, China Unicom

[3] RP-220556, WI Summary for NR QoE management and optimizations for diverse services, China Unicom

# 5 Annex – Comparison between LTE QoE and NR QoE

Configurations:

TS 38.331

AppLayerMeasConfig-r17 ::=           SEQUENCE {

measConfigAppLayerToAddModList-r17   SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayer-r17     OPTIONAL, -- Need N

measConfigAppLayerToReleaseList-r17  SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayerId-r17   OPTIONAL, -- Need N

rrc-SegAllowed-r17                   ENUMERATED {enabled}                                                       OPTIONAL, -- Need R

...

}

MeasConfigAppLayer-r17 ::=           SEQUENCE {

measConfigAppLayerId-r17             MeasConfigAppLayerId-r17,

measConfigAppLayerContainer-r17      OCTET STRING (SIZE (1..8000))                                              OPTIONAL, -- Need N

serviceType-r17                      ENUMERATED {streaming, mtsi, vr, spare5, spare4, spare3, spare2, spare1}   OPTIONAL, -- Need M

pauseReporting-r17                   BOOLEAN                                                                    OPTIONAL, -- Need M

transmissionOfSessionStartStop-r17   BOOLEAN                                                                    OPTIONAL, -- Need M

ran-VisibleParameters-r17            SetupRelease {RAN-VisibleParameters-r17}                                   OPTIONAL, -- Cond ServiceType

...

}

TS 36.331

[[	measConfigAppLayer-r15		CHOICE{

release					NULL,

setup					SEQUENCE{

measConfigAppLayerContainer-r15		OCTET STRING (SIZE(1..1000)),

serviceType-r15						ENUMERATED {qoe, qoemtsi, spare6, spare5, spare4, spare3, spare2, spare1}

}

}	OPTIONAL,	-- Need ON

Reporting:

TS 38.331

–	MeasurementReportAppLayer

The MeasurementReportAppLayer message is used for sending application layer measurement report.

Signalling radio bearer: SRB4

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

MeasurementReportAppLayer message

-- ASN1START

-- TAG-MEASUREMENTREPORTAPPLAYER-START

MeasurementReportAppLayer-r17 ::=     SEQUENCE {

criticalExtensions                    CHOICE {

measurementReportAppLayer-r17     MeasurementReportAppLayer-r17-IEs,

criticalExtensionsFuture              SEQUENCE {}

}

}

MeasurementReportAppLayer-r17-IEs ::=   SEQUENCE {

measurementReportAppLayerList-r17       MeasurementReportAppLayerList-r17,

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    SEQUENCE{}                                                             OPTIONAL

}

MeasurementReportAppLayerList-r17 ::= SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasReportAppLayer-r17

MeasReportAppLayer-r17 ::=        SEQUENCE {

measConfigAppLayerId-r17              MeasConfigAppLayerId-r17,

measReportAppLayerContainer-r17       OCTET STRING                                                             OPTIONAL,

appLayerSessionStatus-r17             ENUMERATED {started, stopped}                                            OPTIONAL,

ran-VisibleMeasurements-r17           RAN-VisibleMeasurements-r17                                              OPTIONAL

}

RAN-VisibleMeasurements-r17 ::=       SEQUENCE {

appLayerBufferLevelList-r17           SEQUENCE (SIZE (1..8)) OF AppLayerBufferLevel-r17                        OPTIONAL,

playoutDelayForMediaStartup-r17       INTEGER (0..30000)                                                       OPTIONAL,

pdu-SessionIdList-r17                 SEQUENCE (SIZE (1..maxNrofPDU-Sessions-r17)) OF PDU-SessionID            OPTIONAL,

...

}

AppLayerBufferLevel-r17 ::= INTEGER (0..30000)

-- TAG-MEASUREMENTREPORTAPPLAYER-STOP

-- ASN1STOP

MeasReportAppLayer field descriptions

appLayerSessionStatus Indicates that an application layer measurement session in the application layer starts or ends.

measReportAppLayerContainer The field contains application layer measurement report, see Annex L (normative) in TS 26.247 [68], clause 16.5 in TS 26.114 [69] and TS 26.118 [70].

ran-VisibleMeasurements The field contains the configuration of RAN visible application layer measurement parameters.

RAN-VisibleMeasurements field descriptions

appLayerBufferLevelList The field indicates a list of application layer buffer levels, and each AppLayerBufferLevel indicates the application layer buffer level in ms. Value 0 corresponds to 0ms, value 1 corresponds to 10ms, value 2 corresponds to 20 ms and so on. If the buffer level is larger than the maximum value of 30000 (5 minutes), the UE reports 30000.

playoutDelayForMediaStartup Indicates the application layer playout delay for media start-up in ms. Value 0 corresponds to 0ms, value 1 corresponds to 1ms, value 2 corresponds to 2 ms and so on. If the playout delay for media start-up is larger than the maximum value of 30000ms, the UE reports 30000.

pdu-SessionIdList Contains the identity of the PDU session, or the identities of the PDU sessions, used for application data flows subject to the RAN visible application layer measurements.

TS 36.331

–	MeasReportAppLayer

The MeasReportAppLayer message is used for sending application layer measurement report.

Signalling radio bearer: SRB4

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

MeasReportAppLayer message

-- ASN1START

MeasReportAppLayer-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

measReportAppLayer-r15				MeasReportAppLayer-r15-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

MeasReportAppLayer-r15-IEs ::=		SEQUENCE {

measReportAppLayerContainer-r15		OCTET STRING (SIZE(1..8000))			OPTIONAL,

serviceType-r15						ENUMERATED {qoe, qoemtsi, spare6, spare5, spare4, spare3, spare2, spare1}		OPTIONAL,

nonCriticalExtension				MeasReportAppLayer-v1590-IEs				OPTIONAL

}

MeasReportAppLayer-v1590-IEs ::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

MeasReportAppLayer field descriptions

measReportAppLayerContainer The field contains container of application layer measurements, see Annex L (normative) in TS 26.247 [90] and clause 16.5 in TS 26.114 [99].

serviceType Indicates the type of application layer measurement. Value qoe indicates Quality of Experience Measurement Collection for streaming services, value qoemtsi indicates Quality of Experience Measurement Collection for MTSI.

# 6 Annex – Handover Request Messages in TS 38.423

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

#### 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 |  | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |
| DAPS Response Information | O |  | 9.2.1.34 |  | YES | reject |
| Conditional Handover Information Acknowledge | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | – |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.101 |  | – |  |
| MBS Session Information Response List | O |  | 9.2.1.38 |  | YES | ignore |