3GPP TSG-RAN WG2 Meeting #121	R2-2300609

Athens, Greece, 27 Feb – 3 Mar 2023

Agenda item:	8.3.4

Source:	Intel Corporation

Title:	Cell (re)selection for handling legacy UEs in NES

WID/SID:	Netw_Energy_NR-Core

Document for:	Discussion and Decision

# 1	Introduction

The network energy saving WI [1] includes the following objective:

Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2]

In this contribution, the above objective is further discussed.

# 2	Discussion

During the SI phase, RAN2 agreed that there is a need to prevent legacy UEs from camping on the NES cell as in the TR [2]:

For backward compatibility, there is a need to allow NES cells to prevent legacy UEs from camping. NES cells should be able to configure whether to prevent legacy UEs, while allowing NES-capable UEs to camp on. Possible solutions may include but not limited to:

-	Use IntraFreqExcludedCellList/InterFreqExcludedCellList

-	Use the cellBarred or cell reservation fields in MIB/SIB

As aforementioned, this can be done via existing mechanisms as follow:

- CellBarred in the MIB

- CellReservedForOtherUse/CellReservedForFutureUse

- InterFreq/IntraFreqExcludedCellList

For 1), if CellBarred in the MIB is used to bar legacy UE only, there is a need to introduce another bit in the MIB to allow the cell to bar the NES capable UE. As there is only 1 bit left in the MIB, this bit should be reserved for very critical issue that may arise in the future. Hence, we should avoid using the CellBarred in the MIB in a NES cell to bar legacy UEs from camping in a NES cell.

For 2), CellReservedForOtherUse is used to bar non-NPN UEs from using an NPN-only cell. CellReservedForFutureUse is introduced to bar NPN UEs in the cell which may not be suitable for legacy NPN UE.  CellReservedForOtherUse/CellReservedForFutureUse can be used to bar legacy non-NPN UE/NPN UE from accessing the NES cell. However, additional cell reservation bit(s) need to be introduced to bar NES capable non-NPN/NPN capable UEs on cells where NES capable non-NPN/NPN capable UE are not allowed.

The following table show one way on how it can work to bar legacy non-NPN UE, legacy NPN UE and NES capable UE:

| CellReservedForOtherUse (legacy non-NPN and non NES capable UE) | CellReservedForFutureUse (NPN capable but non NES capable UE) | New bit for non NPN and NES capable UE | New bit for NPN and NES capable UE | UE behaviour | Cell type |
| --- | --- | --- | --- | --- | --- |
| False | False | False | False | All UEs can use the cell | Legacy cell |
| True | False | False | False | All non-NPN legacy UEs are barred | Legacy NPN only cell |
| True | True | False | False | All non-NES capable UEs are barred NES capable UEs can use the cell | NES Cell. |
| True | True | True | False | All non-NES capable UEs and non-NPN capable but NES capable UEs are barred NES capable NPN UE can use the cell | NES NPN only cell |
| True | True | True | True | All UEs are barred. | Future cell type |

Above table shows that it is feasible to use cell reservation mechanism to bar all UEs (i.e. legacy non-NPN UE, legacy NPN UE and NES capable UE) using 2) in different cell types.

For 3), again if InterFreq/IntraFreqExcludedCellList is used by legacy UE for excluding NES cell, then there is a need to have another InterFreq/IntraFreqExcludedCellList to exclude cell for NES capable UE. Depending on how fast the NES cell changes in its NES mode, this may require inter-node coordination and thus impact RAN3 which is not in the scope for this objective.

Both 2) and 3) can be used to prevent legacy UEs from accessing NES cell while allowing NES capable UEs to camp and access NES cell. However, while 2) handles both cell selection and reselection, 3) is only useful for cell reselection.  We think that only one mechanism is needed and 2) seems sufficient mechanism for this purpose.  There is no need for further optimisation for cell reselection using excluded cell list as in 3) for legacy UE handling.

Observation#1: Using CellBarred in the MIB for preventing legacy UE accessing the NES cell may require using the last spare bit in the MIB for barring NES capable UEs. This should be avoided.

Observation#2: Extending CellReservedForOtherUse/CellReservedForFutureUse in the SIB1 for preventing legacy UEs accessing the NES cell is applicable to both cell selection and reselection

Observation#3: Extending InterFreq/IntraFreqExcludedCellList in the SIB1 for preventing legacy UE accessing NES cell is only applicable to cell reselection. It may also require inter-node coordination and thus impact RAN3 which is not in the scope for this objective.

Based on the observations, extending CellReservedForOtherUse/CellReservedForFutureUse in the SIB1 for preventing legacy UEs accessing the NES cell seems sufficient for both cell selection/reselection.

Proposal#1: Only include CellReservedForOtherUse/CellReservedForFutureUse to prevent legacy UEs camping on NES cells in the WI phase.

Proposal#2: Introduce 2 further bits in SIB1:

- 1 bit to enable NPN only NES cell to bar NES capable non-NPN UEs

- 1 bit to enable future cell type to bar NES capable NPN only UEs

# 3	Conclusion

It is requested that RAN2 agree to the proposals and observations below:

Observation#1: Using CellBarred in the MIB for preventing legacy UE accessing the NES cell may require using the last spare bit in the MIB for barring NES capable UEs. This should be avoided.

Observation#2: Extending CellReservedForOtherUse/CellReservedForFutureUse in the SIB1 for preventing legacy UEs accessing the NES cell is applicable to both cell selection and reselection

Observation#3: Extending InterFreq/IntraFreqExcludedCellList in the SIB1 for preventing legacy UE accessing NES cell is only applicable to cell reselection. It may also require inter-node coordination and thus impact RAN3 which is not in the scope for this objective.

Based on the observations, extending CellReservedForOtherUse/CellReservedForFutureUse in the SIB1 for preventing legacy UEs accessing the NES cell seems sufficient for both cell selection/reselection.

Proposal#1: Only include CellReservedForOtherUse/CellReservedForFutureUse to prevent legacy UEs camping on NES cells in the WI phase.

Proposal#2: Introduce 2 further bits in SIB1:

- 1 bit to enable NPN only NES cell to bar NES capable non-NPN UEs

- 1 bit to enable future cell type to bar NES capable NPN only UEs

# 4	References

[1] RP-223540 New WID: Network energy savings for NR

[2] RP-222821 TR 38.864 v1.0.0 Study on network energy savings for NR