3GPP TSG-RAN WG2 Meeting #121	R2-2300611

Athens, Greece, 27 Feb – 3 Mar 2023

Agenda item:	8.3.2

Source:	Intel Corporation

Title:	Considerations of Cell DTX and DRX

WID/SID:	Netw_Energy_NR-Core

Document for:	Discussion and Decision

# 1	Introduction

The network energy saving WI [1] includes the following objective:

Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3]

- Note: No change for SSB transmission due to cell DTX/DRX.

- Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

In this contribution, the procedure and signalling aspects and the UE behaviour of the Cell DTX/DRX are further discussed.

# 2	Discussion

## 2.1	Procedure and signalling aspects

As in the TR, the general understanding of the solution is that the configuration for the Cell DTX/DRX are provided by the RRC signalling and when to initiate the use of the configuration can be indicated by dynamic L1/L2 signalling and this is as described below in the TR [2]:

The Cell DTX/DRX mode can be activated/de-activated via dynamic L1/L2 signalling and UE-specific RRC signaling. Both UE specific and common L1/L2 signalling can be considered for activating/deactivating the Cell DTX/DRX mode.

The configuration for the Cell DTX/DRX contains RRC parameters that define the Cell DTX/DRX pattern. Like UE C-DRX pattern, the baseline is that “on-duration” of the Cell DTX/DRX pattern is periodic.  On the other hand, unlike UE C-DRX which is configured based on the UE QoS requirement (with some offset to spread the scheduling load) and UE power saving in mind, whether/when to turn on the periodic pattern also depends on the load of the network. The periodic pattern configured by RRC should only be started when the network needs to perform network energy saving mode (e.g. due to low load) so that network has more chance of energy saving and at some point, the network may need to turn off the network energy saving mode by stopping the application of the periodic pattern configured by the RRC (e.g. when the load becomes normal).

To achieve maximum network energy saving, there is a need for the network to indicate the start and stop applying of the Cell DTX/DRX quickly, but without releasing the Cell DTX/DRX configuration.  Hence having a L1/L2 indication to start and stop applying the Cell DRX/DRX seems to be beneficial.  As such configuration is not just for a UE but most likely to all or a group of UEs in a cell, an efficient method to signal such indication to prevent signalling overhead is needed. Therefore, a group configuration update mechanism seems useful. Without such a group configuration update mechanism, the network has to signal the change to each of the UE in RRC Connected mode via dedicated signalling (e.g. RRC signalling or UE specific L1/L2 signalling) and hence does not allow the network to bring its desired network energy efficiency in a timely manner.  This may also result in huge signalling overhead and thus does not provide an efficient operation.

Observation#1: Unlike UE C-DRX which is configured based on the UE QOS requirement (with some offset to spread the scheduling load) and UE power saving, whether/when to turn on the periodic cell DTX/DRX pattern depends also on the load of the network. The periodic pattern configured by RRC should only be started when the network needs to perform network energy saving mode (e.g. due to low load) so that network has more chance of energy saving and at some point, the network may need to turn off the network energy saving mode by stopping the application of the periodic pattern configured by the RRC.

Observation#1-1: To achieve maximum network energy saving, there is a need for the network to be able to indicate to the UE the start and stop applying of the Cell DTX/DRX quickly and frequently without releasing the Cell DTX/DRX configuration provided to the UE.

Proposal#1: RAN2 to assume a dedicated L1 signaling to indicate the start and the end of the Cell DTX/DRX pattern.

When the cell goes through DTX/DRX, it will be applicable for all NES-capable UEs in the cell in order to achieve the network energy saving.  Hence a common/Cell specific indication will be more efficient over providing UE dedicated indications for all the UEs in the cell.  This can only be achieved via a common L1 signalling.

Observation#2: As such L1/L2 indication is not just for a UE but most likely to all UEs in a cell, an efficient method is needed to prevent signalling overhead and to bring its desired network energy efficiency in a timely manner.

Proposal#2: Common L1 indication is assumed to be supported for the indication of the Cell DTX/DRX in Proposal 1 on top of the  dedicated L1 indication.

Proposal#2-1: Inform RAN1 if Proposals 1 and/or 2 are recommended from RAN2 point of view.

As on whether to support multiple configurations of Cell DTX/DRX, the UE needs to be configured with at least one Cell DTX/DRX configuration (i.e. turn on Cell DTX/DRX) when the gNB load is low in addition to the normal configuration.  This normal configuration applies when no DTX/DRX configuration is applied (i.e. turn off Cell DTX/DRX), e.g. when the gNB load is high and/or QoS requirements of the UEs are not satisfied. Typical gNB operation may serve different loading conditions, such as low, moderate, or high load and correspondingly system resource utilization would vary. Multiple configurations of Cell DTX/DRX will allow the network to select the most appropriate Cell DTX/DRX configurations for the load of a cell and for meeting the QoS requirements of the UEs in the cell.  However, at any one time, only 1 configuration should be activated for a serving cell.

Observation#3: Multiple configurations of Cell DTX/DRX will allow the network to select the most appropriate Cell DTX/DRX configurations for the load of a cell and for meeting the QoS requirements of the UEs in the cell.

These multiple configurations of Cell DTX/DRX for a serving cell should be provided to the UE in advance and will allow the network to indicate to the UE via the common L1 signalling via a configuration ID or bitmap.

Proposal#3: UE can be configured with multiple configurations of Cell DTX/DRX for a serving cell in advance and which configuration to use can be signalled in the common L1 signalling via a configuration ID or bitmap.

The procedure overview of the Cell DTX/DRX is shown below in Figure 1 for UE in RRC Connected mode:

Figure 1: High level view of the Cell DTX/DRX

Consider that UE enters connected mode indicating it supports Cell DTX/DRX

Step 1: gNB configures the UE with one or more Cell DTX/DRX configurations (e.g. periodic pattern) in advance. If multiple configurations are configured, each configuration is identified by a configuration ID or bitmap

Step 2: When the load is low and the Cell DTX/DRX configuration can meet the QoS of the UEs or group of UEs in the cell, the gNB triggers the UE(s) to start applying Cell DTX/DRX configurations (i.e. the Cell DTX.DRX pattern) using the dynamic (common/dedicated) L1/L2 indication. For the case multiple configurations are configured to the UE, the indication includes the configuration ID or bitmap.

Step 3: When the load is normal and/or the Cell DTX/DRX configuration is no longer acceptable to the QoS of the UEs or group of UEs in the cell, the gNB indicates to the UE to stop applying Cell DRX/DRX configurations (i.e. the Cell DTX/DRX pattern) using the dynamic (common/dedicated) L1/L2 indication.

## 2.2	Active and non-active period of Cell DTX/DRX

It is agreed in the SI phase that the Cell DTX/DRX pattern is periodic. That is, a cycle of a Cell DTX/DRX includes an Active period and a non-Active period with the Active period defined at least by the On-duration and the cycle defined by a periodicity as below:

For the Active period of the Cell DTX, the UE is assumed to be able to receive all DL transmissions that it is configured to receive while for Cell DRX, the UE is assumed to be able to transmit in the UL that it is configured to transmit. In the non-Active period, the following examples of gNB behaviours are included into the TR [2] as follow:

- Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX/DRX non-active periods.

- Example 2: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals)

- Example 3: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS).

- Example 4: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

The different examples allow for different modes of energy saving by the gNB.  Instead of assuming that all transmissions and receptions are turned off during the non-Active duration (as in the Example#1 above), the network can indicate to the UE which transmissions and receptions are to be turned off or continued during the non-Active duration of the Cell DTX/DRX. The following table contains some illustrations of the different possible modes of transmissions/receptions (TX/RX) during the non-Active duration (in the order of most network energy saving mode (i.e. Mode#1) to the least network energy saving mode (i.e. Mode#5) other than there is no non-Active duration.

Table 1: Different modes of signal/channel transmission/reception that can be allowed in non-active duration

| Rx/Tx occasions in non-Active duration |  | Modes of TX/RX in non-Active duration |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |
| SPS data and CG data |  |  |  |  |  |  |
| Dynamic scheduling data |  |  |  |  |  |  |
| SR and PRACH |  |  |  |  |  |  |
| UL reference signal: SRS |  |  |  |  |  |  |
| DL reference signals: SSB |  |  |  |  |  |  |
| DL reference signals: CSI-RS and/or TRS |  |  |  |  |  |  |
| DL/UL Retransmissions |  |  |  |  |  |  |

is not allowed for transmission/reception by the gNB

is allowed for transmission/reception by the gNB

One way of indicating to the UE on the mode of transmissions/receptions in non-Active period is to signal it in the dedicated RRC signalling as part of the Cell DTX/DRX configuration for a cell or a group of cells (in the CA case that applies the same Cell DTX/DRX configuration).  Once the Cell DTX/DRX configuration is enabled via RRC signalling or via L1/L2 signalling, the mode of transmission/receptions for the UEs to assume in the non-Active period will be based on the mode configured corresponding to as part of the Cell DRX/DRX configuration. In the case of multiple Cell DTX/DRX configurations, each corresponding mode is provided for each Cell DTX/DRX configuration and when UE is enabled via RRC signalling or L1/L2 signalling with the configuration index, the corresponding mode of transmissions/receptions in non-Active duration is applied.

Which modes to support

In the WI scope, it is stated that there should be no change to SSB transmission when cell DTX/DRX pattern comes into effect as follow:

- Note: No change for SSB transmission due to cell DTX/DRX.

By default, Mode 3 needs to be supported. Anyway, Mode 3 provides the maximum network energy gain since no transmission and reception are occurring during the Cell DRX/DRX non-active periods and is done without impacting Idle mode or Inactive state UEs since the SSB is still transmitted during non-Active period of Cell DTX as mentioned in the WI scope:

- Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

As indicated in Section 2.1, the Cell DTX/DRX operation also has to meet the UEs’ QoS and RRM requirement in the cell (e.g. all the UEs in the cell have delay tolerant traffic QoS (e.g. background traffic that are delay tolerant) and the UEs do not need to perform much RRM and scheduling measurement (e.g. stationary or pseudo-stationary)).  Mode 5 may be required to allow the UE to support e.g. voice traffic via SPS and CG and also retransmissions, while also allowing the UE to inform of UL data arrival.  Hence, we also think that Mode 5 should also be supported.

Proposal#6: Support at least Mode 3 (i.e. UE expects only SSB during non-Active period) and Mode 5 (i.e. UE expects SPS, CG, SR, RACH, SSB and retransmissions during non-Active period) for Cell DTX/DRX where Mode 3 provides the network to maximise on network energy saving while Mode 5 allows the network to meet the QoS and RRM requirement of the UE.

Proposal#7: UE is expected to receive mode of transmissions and receptions (i.e. what is allowed to be transmitted/received) for the non-Active period as indicated by the gNB via L1 signalling (as in Proposal 1 and 2).

Possible extension of the Active period by the network

There may be some occasions where the gNB wants to extend the Active period for the UEs in the cell due to some further downlink/uplink traffic at the end of the Active period.  One simple approach is to introduce an extended Active duration timer triggered by the dedicated or common L1/L2 signalling.  Whenever the network (cell) wants to extend the on-duration/Active period of the Cell DTX/DRX for the cell, it can send an indication in the common L1/L2 signalling to extend the active duration for the duration of the inactivity timer, in effect shorten the non-active duration of a Cell DTX/DRX cycle.  The network can include as part of the indication to indicate for how many Cell DTX/DRX cycles the inactivity timer should be performed.  This approach, however, will add additional indication in the L1 signalling as well as RRC configuration to include the timer values for the extension.

Proposal#8: RAN2 to support mechanism for gNB to extend the Active period for the UEs in the cell. FFS on the details.

## 2.3 Cell DTX interactions with UE C-DRX

As both UE DRX and Cell DRX/DTX dictates on whether or when UE receives and/or transmits, there is a need to coordinate the UE behaviour between UE DRX and Cell DRX/DTX.  It can be seen as a modelling issue whether to use UE C-DRX for Cell DTX as in Option 1 below:

Option 1: Alignment of the starting position of UE’s C-DRX among UEs and provide additional behaviour to support the different levels (slide 4) and Cell DRX operation

Option 1 may require less specification as some of the UE’s C-DRX behaviour can be reused (e.g. no PDCCH monitoring during not Active time etc.). However, it may not be sufficient to just align the UE’s C-DRX configuration with Cell DTX/DRX as the UE behaviour may not be the same between UE’s C-DRX configuration. Take for example Level#3, all the reception and transmissions except for SSB are to be stopped during the non-Active period.  But UE C-DRX will still allow for UL transmissions such as SR and RACH. In order to fully aligned with Cell DTX/DRX, the different levels of transmission and receptions in the non-Active period has to be implemented as exception in the UE DRX operation.

Alternative is Option 2: Cell DRX/DTX operation overrides the UE DRX operation if Cell DRX/DTX is configured and activated. This will allow for a whole new UE behaviour for Cell DRX/DRX, taking into account the different level of transmissions and receptions. However, this comes at the expense of more specification impact.

Proposal#9: RAN2 to discuss whether Option 1: to reuse the existing UE’s C-DRX for Cell DTX via alignment of the C-DRX among the UEs or Option 2: to define new Cell DTX/DRX behaviour that override the UE DRX operation.

# 4	Conclusion

It is requested that RAN2 agree to the proposals and observations below:

Observation#1: Unlike UE C-DRX which is configured based on the UE QOS requirement (with some offset to spread the scheduling load) and UE power saving, whether/when to turn on the periodic cell DTX/DRX pattern depends also on the load of the network. The periodic pattern configured by RRC should only be started when the network needs to perform network energy saving mode (e.g. due to low load) so that network has more chance of energy saving and at some point, the network may need to turn off the network energy saving mode by stopping the application of the periodic pattern configured by the RRC.

Observation#1-1: To achieve maximum network energy saving, there is a need for the network to be able to indicate to the UE the start and stop applying of the Cell DTX/DRX quickly and frequently without releasing the Cell DTX/DRX configuration provided to the UE.

Proposal#1: RAN2 to assume a dedicated L1 signaling to indicate the start and the end of the Cell DTX/DRX pattern.

Observation#2: As such L1/L2 indication is not just for a UE but most likely to all UEs in a cell, an efficient method is needed to prevent signalling overhead and to bring its desired network energy efficiency in a timely manner.

Proposal#2: Common L1 indication is assumed to be supported for the indication of the Cell DTX/DRX on top of the dedicated L1 indication in Proposal 1.

Proposal#2-1: Inform RAN1 if Proposals 1 and/or 2 are recommended from RAN2 point of view.

Observation#3: Multiple configurations of Cell DTX/DRX will allow the network to select the most appropriate Cell DTX/DRX configurations for the load of a cell and for meeting the QoS requirements of the UEs in the cell.

These multiple configurations of Cell DTX/DRX for a serving cell should be provided to the UE in advance and will allow the network to indicate to the UE via the common L1 signalling via a configuration ID or bitmap.

Proposal#3: UE can be configured with multiple configurations of Cell DTX/DRX for a serving cell in advance and which configuration to use can be signalled in the common L1 signalling via a configuration ID or bitmap.

Proposal#6: Support at least Mode 3 (i.e. UE expects only SSB during non-Active period) and Mode 5 (i.e. UE expects SPS, CG, SR, RACH, SSB and retransmissions during non-Active period) for Cell DTX/DRX where Mode 3 provides the network to maximise on network energy saving while Mode 5 allows the network to meet the QoS and RRM requirement of the UE.

Proposal#7: UE is expected to receive mode of transmissions and receptions (i.e. what is allowed to be transmitted/received) for the non-Active period as indicated by the gNB via L1 signalling (as in Proposal 1 and 2).

Proposal#8: RAN2 to support mechanism for gNB to extend the Active period for the UEs in the cell. FFS on the details.

Proposal#9: RAN2 to discuss whether Option 1: to reuse the existing UE’s C-DRX for Cell DTX via alignment of the C-DRX among the UEs or Option 2: to define new Cell DTX/DRX behaviour that override the UE DRX operation.

# 5	References

[1] RP-223540 New WID: Network energy savings for NR

[2] RP-222821 TR 38.864 v1.0.0 Study on network energy savings for NR