3GPP TSG-RAN WG2 Meeting #121	R2-2300613

Athens, Greece, 27 Feb – 3 Mar 2023

Agenda item:	8.21.3

Source:	Intel Corporation

Title:	RAN2 impact on Rel-18 NPN enhancement

WID/SID:	TEI

Document for:	Discussion and Decision

# 1	Introduction

RAN2 has received a LS [1] from SA2 informing RAN2 that RAN impacts are identified for the following objectives in SA2 work below:

1.	Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection, including:

2.	Support for non-3GPP access for SNPN.

In this contribution, the RAN 2 impacts are discussed.

# 2	Discussion

The RAN impacts related to the 2 SA2 objectives in Section 1 are briefly mentioned in the LS [1] below:

The NG-RAN impacts identified are impacts to NGAP to support equivalent SNPNs, and impacts on cell (re-)selection for objective 1, and NGAP impacts to support selected SNPN (NID) for objective 2.

Hence the RAN2 impact is just on the cell (re)selection related to the equivalent SNPN.  This impact is also listed in [2] as extracted below with RAN impact highlighted:

Following the similar principles used in the Equivalent PLMNs, it is concluded that the following principles for the Equivalent SNPNs are to be used as basis for normative work:

-	Since UE may have either a subscription that can be used to access source and target SNPNs or has the credentials from a Credentials Holder (CH) to access both source and target SNPNs, the lists of equivalent SNPNs are stored and used per SNPN subscription by the UE.

-	UE and AMF support of equivalent SNPN list in NAS.

-	NG-RAN and AMF support of equivalent SNPNs in NGAP.

-	UE/NG-RAN/AMF take equivalent SNPN list into consideration, for supporting relevant functions, e.g.:

-	idle/connected network selection.

-	cell (re)selection.

-	AMF to inform UE the registered SNPN ID change during mobility in Registration Accept message, when there is a change of registered SNPN.

-	NGAP impact for supporting connected mode mobility between SNPNs.

NOTE:	It is up to RAN WG3 to decide how to extend NGAP for connected mode mobility.

-	Equivalent SNPNs, with different SNPN IDs, within an RA is not supported for SNPNs.

Observation#1: From the SA2 work, the RAN2 impact is related to the equivalent SNPN on cell (re)selection.

Since it is an extension of the equivalent PLMN as applied by the following statement from the TR [2],

Following the similar principles used in the Equivalent PLMNs, it is concluded that the following principles for the Equivalent SNPNs are to be used as basis for normative work

the impact to RAN2 is mainly on the idle mode specification TS38.304 and the functions related to equivalent PLMN need to be extended.  A list of the impact based on the latest TS38.304:

| Area of impacts for equivalent SNPNs | Current texts for equivalent PLMNs |
| --- | --- |
| Equivalent SNPN list definition | Clause 3.1 Equivalent PLMN list: List of PLMNs considered as equivalent by the UE for cell selection, cell reselection, and handover according to the information provided by the NAS. |
| NAS interactions with AS for equivalent SNPN | Clause 4.1: When a UE is switched on, a public land mobile network (PLMN) or a SNPN is selected by NAS. For the selected PLMN/SNPN, associated RAT(s) may be set, as specified in TS 23.122 [9]. The NAS shall provide a list of equivalent PLMNs, if available, that the AS shall use for cell selection and cell reselection. Clause 4.2: For PLMN selection: For a UE not operating in SNPN access mode, perform the following: Maintain a list of PLMNs in priority order according to TS 23.122 [9]. Select a PLMN using automatic or manual mode as specified in TS 23.122 [9] and request AS to select a cell belonging to this PLMN. For each PLMN, associated RAT(s) may be set. Evaluate reports of available PLMNs and any associated CAG-IDs from AS for PLMN selection. Maintain a list of equivalent PLMN identities. For cell reselection: For a UE not operating in SNPN access mode, maintain a list of equivalent PLMN identities and provide the list to AS. |
| Suitable cell definition while operating in SNPN access mode | Clause 4.5: The cell is part of either the selected PLMN or the registered PLMN or PLMN of the Equivalent PLMN list, and for that PLMN |
| Suitability check | Clause 5.2.4.4 this cell belongs to a PLMN which is not indicated as being equivalent to the registered PLMN |
| Intra-Frequency Reselection Indication (IFRI) | Clause 5.3.1 If the cell operates in licensed spectrum, or if this cell belongs to a PLMN which is indicated as being equivalent to the registered PLMN or the selected PLMN of the UE, or if this cell belongs to the registered SNPN or the selected SNPN of the UE |

From the above, if SA2 work only impact the Cell (re)selection due to equivalent SNPN, the RAN2 impact is relatively small.

Observation: If SA2 work only impact the Cell (re)selection due to equivalent SNPN, the RAN2 impact is relatively small.

As on the RAN3 work, it should be left to RAN3 to decide whether a WI is needed. Based on our understanding, the RAN3 impact related to the for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection (Objective 1) and for non-3GPP access for SNPN (Objective 2) are also not significant.

Proposal: From RAN2 point of view, it is ok to work on NPN enhancements from SA2 via TEI18 or a new WI.

# 3	Conclusion

It is requested that RAN2 agree to the proposals and observations below:

Observation: If SA2 work only impact the Cell (re)selection due to equivalent SNPN, the RAN2 impact is relatively small.

Proposal: From RAN2 point of view, it is ok to work on NPN enhancements from SA2 via TEI18 or a new WI.

# 4	References

[1] S2-2301437 LS on RAN impact for NPN enhancement in Rel-18.

[2] 3GPP TR23.700-08 Study on enhanced support of Non-Public Networks Phase 2.