3GPP TSG-RAN WG2 Meeting #121	R2-2300631

Athens, Greece, 27th February – 3rd March 2023

Agenda item:	8.14.5

Source:		Lenovo

Title:	Discussion on QoE measurement during intra-5GC inter-RAT handover

Document for:	Discussion and Decision

# Introduction

This paper discusses the following objective in QoE enhancement WID [1]:

-	Support the continuity of legacy QoE measurement job for streaming and MTSI service during intra-5GC inter-RAT handover process [RAN2, RAN3].

# Discussion

- NR QoE supports multiple QoE measurement collection jobs per service type:

-	One or more QoE measurement collection jobs can be activated at a UE per service type, and each QoE measurement configuration is uniquely identified by a QoE Reference.

-	A measConfigAppLayerId conveyed in the RRC signalling is used to identify the application layer measurement configuration and report between the gNB and the UE. The RRC identifier is mapped to the QoE Reference in the gNB.

Observation 1: NR QoE supports multiple QoE measurement collection jobs per service type.

- However, E-UTRAN QoE only supports single QoE measurement job at a time and only supports two service types:

-	The supported service types are QoE Measurement Collection for streaming services and QoE Measurement Collection for MTSI services

-	The E-UTRAN QoE only supports single QoE measurement collection job at a time according to TS 36.331:

measConfigAppLayer-r15		CHOICE{

release					NULL,

setup					SEQUENCE{

measConfigAppLayerContainer-r15		OCTET STRING (SIZE(1..1000)),

serviceType-r15						ENUMERATED {qoe, qoemtsi, spare6, spare5, spare4, spare3, spare2, spare1}

}

}	OPTIONAL,	-- Need ON

measConfigAppLayerContainer The field contains configuration of application layer measurements, see Annex L (normative) in TS 26.247 [90] and clause 16.5 in TS 26.114 [99]. The maximum number of configurations of application layer measurements that a UE supports is one regardless of serviceType.

Observation 2: E-UTRAN QoE only supports single QoE measurement job at a time and only supports two service types.

- Therefore, the QoE measurement capabilities of NR and E-UTRAN are not matched. One of possible solutions is to enhance E-UTRAN air interface to introduce multiple QoE measurements per service type and more service types that are same with NR. However, enhancing E-UTRAN air interface may have much impact on LTE specification and introduce new requirements on LTE gNB and UE’s implementation. Furthermore, any impacts to LTE specifications have been removed from the latest WID [1]. Therefore, modifying LTE to support continuity of legacy QoE measurement job during intra-5GC inter-RAT handover is out of scope.

Observation 3: Modifying LTE to support continuity of legacy QoE measurement job during intra-5GC inter-RAT handover is out of scope.

- In this sense, NW based solutions are preferred in our opinion.

- Case 1: Handover from source gNB to target ng-eNB with continuity of legacy QoE measurement job.

- Issue 1: More than one QoE measurement collection jobs may have been activated for a service type (e.g. streaming or MTSI) in the source gNB, but the target ng-eNB can only support one QoE measurement collection job for the service type. How to down select one of QoE measurement collection job with QoE measurement continuity during handover from source gNB to target ng-eNB?

- Case 2: Handover from source ng-eNB to target gNB with continuity of legacy QoE measurement job.

- Issue 2: For handover from source ng-eNB to target gNB, how to configure the measConfigAppLayerId with continuity of the QoE measurement for an on-going QoE measurement?

RAN2 discusses NW based solutions for below issues:

- -	For handover from source gNB to target ng-eNB, how to down select one of QoE measurement collection job with QoE measurement continuity?

- -	For handover from source ng-eNB to target gNB, how to configure the measConfigAppLayerId with continuity of the QoE measurement for an on-going QoE measurement?

# 3	Conclusion

This paper discusses the continuity of legacy QoE measurement job for streaming and MTSI service during intra-5GC inter-RAT handover process with the following observations and proposal.

Observation 1: NR QoE supports multiple QoE measurement collection jobs per service type.

Observation 2: E-UTRAN QoE only supports single QoE measurement job at a time and only supports two service types.

Observation 3: Modifying LTE to support continuity of legacy QoE measurement job during intra-5GC inter-RAT handover is out of scope.

- RAN2 discusses NW based solutions for below issues:

- -	For handover from source gNB to target ng-eNB, how to down select one of QoE measurement collection job with QoE measurement continuity?

- -	For handover from source ng-eNB to target gNB, how to configure the measConfigAppLayerId with continuity of the QoE measurement for an on-going QoE measurement?

# 4	Reference

[1] RP-223488, WID update for Enhancement on NR QoE, China Unicom