3GPP TSG-RAN WG2 Meeting #121                                                              R2-2300638

Athens, Greece, February 27 to March 3, 2023

Agenda item: 		8.12.3

Source: 		Qualcomm Incorporated

Title: 	TAC/RANAC handling for mobile IAB

Document for:	Discussion

# 1	Introduction

For TAC and RANAC broadcast by the mobile IAB-node, RAN3#117bis-e achieved the following agreements [1]:

RAN3 to further discuss the following options for TAC/RANAC issue: - Option 1: The TAC/RANAC for the mobile IAB cell can be changed in order to reflect the physical location when the mobile IAB-node moves. - Option 2: Using static TAC/RANAC for mobile IAB when it moves. Involvement of SA2 may be needed

RAN3#118 achieved the following agreements [2]:

Dynamic TACs: Static TAC solution is not pursued. RAN3 assumes that dynamic TAC solution should be supported.

These latter agreements were also sent in an LS to RAN2 in R2-2300034.

This contribution discusses RAN2 aspects of TAC/RANAC management for mobile IAB.

# 2	Discussion

## Issue 1: TAC/RANAC selection

RAN3 agreed on option 1:

Option 1: The TAC/RANAC for the mobile IAB cell can be changed in order to reflect the physical location when the mobile IAB-node moves.

The mIAB-MT can connect to a different CU than the mIAB-DU. The physical location of the mobile IAB-node is reflected by the cell the mIAB-MT is connected to. This implies that the mIAB-DU should broadcast a TAC owned by the mIAB-MT’s CU.

Proposal 1: The mIAB-DU-cell to broadcast a TAC owned by the mIAB-MT’s CU.

When the mIAB-MT is handed over to a CU in a different tracking area, the mIAB-DU-cell’s TAC has to change. The reconfiguration of the TAC on the mIAB-DU is out-of-scope for RAN2.

Proposal 2: When the mIAB-MT is handed over to a CU in a different tracking area, the mIAB-DU-cell’s TAC has to change. The configuration of the new TAC on the mIAB-DU is out-of-scope for RAN2.

In case DU migration is not performed at the same time as MT handover, the mIAB-DU’s cell can change TAC via SI update.

Proposal 3: The mIAB-DU-cell to change TAC via SI update.

For RANAC, the same mechanisms should be applied as for TAC.

Proposal 4: For RANAC, the same mechanisms to be applied as for TAC.

## Issue 2: Signalling load due to TAC/RANAC change

When the mIAB-DU’s TAC and/or RANC is changed, all idle/inactive UEs camping on the mIAB-DU-cell (or a large fraction of those) may have to perform a registration update. This may create a signaling burst on RACH and RRC. For small vehicles with few UEs, such a signaling burst will not create a problem. RAN2 should discuss if this signaling burst may create an overload condition for a mobile IAB-node hosted by a large vehicle with many UEs (e.g., long train).

Proposal 5: RAN2 to discuss whether synchronized UE registration updates in response to TAC/RANAC change may create an unacceptable overload condition on the signalling plane for a mobile IAB-node hosted on a large vehicle (e.g., long train).

# Conclusion

This contribution discussed topic related to the handling of TAC/RANAC for mobile IAB. The following proposals have been made:

Proposal 1: The mIAB-DU-cell to broadcast a TAC owned by the mIAB-MT’s CU.

Proposal 2: When the mIAB-MT is handed over to a CU in a different tracking area, the mIAB-DU-cell’s TAC has to change. The configuration of the new TAC on the mIAB-DU is out-of-scope for RAN2.

Proposal 3: The mIAB-DU-cell to change TAC via SI update.

Proposal 4: For RANAC, the same mechanisms to be applied as for TAC.

Proposal 5: RAN2 to discuss whether synchronized UE registration updates in response to TAC/RANAC change may create an unacceptable overload condition on the signalling plane for a mobile IAB-node hosted on a large vehicle (e.g., long train).

# References

- Chairman notes, TSG-RAN WG3 Meeting #117bis-e, electronic meeting, August 2022.

- Chairman notes, TSG-RAN WG3 Meeting #118, Toulouse, France, November 2022.