3GPP TSG RAN WG2 Meeting #121		R2-2300679

Athens, Greece, 27th February- 3rd March, 2023

Source:	vivo

Title:          Further discussion on AI/ML methods

Agenda Item:	8.16.2

Document for:	Discussion and Decision

- Introduction

At the RAN2#120 meeting, RAN2 had some initial discussion on model identification, data collection and model transfer/delivery, and agreed on the following:

R2 assumes that model ID can be used to identify which AI/ML model is being used in LCM including model delivery. R2 assumes that model ID can be used to identify a model (or models) during model selection/activation/deactivation/switching (can later align with R1 if needed). For model transfer/delivery for AI/ML models (for the target use cases of this SI), RAN2 to study CP-based, UP-based solutions

At the end of the meeting, two long email discussions were introduced for RAN2#121 discussion on data collection and model transfer/delivery.

[Post120][053][AIML18] model transfer delivery (Huawei) Scope: Long email discussion for next meeting on model transfer/delivery, to collect pros/cons, Can also collect comments on different architectural assumptions. Intended outcome: Report Deadline: Long [Post120][054][AIML18] model transfer delivery (Ericsson / vivo) Scope: Long email discussion for next meeting, on data collection (focus on monitoring and training), on to what extent existing methods can be useful including also identifying these existing methods and their potential extensions Intended outcome: Report Deadline: Long

Taking the summaries of the email discussions into account, this contribution will further discuss the general aspects of the AI/ML methods from RAN2’s perspective, including:

- Data collection

- Model transfer/delivery

- Model/functionality identification

- Discussion

- 2.1	Data collection

RAN1 made the following conclusion on data collection at the RAN1#110b-e meeting.

Conclusion Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

As indicated by RAN1, data collection may be performed for different purposes in LCM to provide essential data for model training, model inference, model monitoring, model selection, model update, etc.

From RAN2’s perspective, different components have different requirements for data collection:

- Model training: model training requires large volumes of data, but no latency requirements if only offline training is supported. The data can be reported once generated or stored and exchanged periodically.

- Model inference: the content of data for model inference is the same as or a subset of the data for model training. The main difference is that model inference requires lower latency for data collection.

- Model monitoring: requires low latency for data collection to deactivate/switch model in a short time when the performance of the activated model turns unacceptable.

For the mechanism of data collection, one straightforward way is to analyze whether the current frameworks are applicable and the following metrics are recommended to be considered in the summary:

- a) the content of the data, - b) the data size, - c) latency and periodicity, - d) signalling, entities involved and configuration aspects. - FFS on how to handle security/privacy.

To our understanding, bullet a) can be revisited when RAN1 concludes on the explicit data. For bullets b) and c), the corresponding performance metric requirement for the frameworks can be the maximum data size of each report, report latency and report type. Bullet d) is related to the termination entity of the report. Besides, the RRC state of UE who performs the data collection shall also be considered.

As to the security/privacy issue, we understand it should be investigated by SA3 when necessary. RAN2 can send LS to SA3 concerning user privacy and security concerns when the explicit data to be collected for AI/ML purposes is agreed.

Proposal 1: When analyzing the existing data collection frameworks for data collection, the following performance metrics can be considered: 1) maximum data size of each report, 2) report latency, 3) report type, 4) termination entity of report, 5)RRC state.

Based on “[Post120][054][AIML18] model transfer delivery (Ericsson / vivo)” outcome, an initial analysis of performance metrics of existing data collection frameworks can be as follows:

Table 2.1-1: performance metrics of existing data collection frameworks

| Data collection framework | Max data size per report | Report latency | Report type | Termination entity | RRC state |
| --- | --- | --- | --- | --- | --- |
| Immediate MDT | 9000 bytes | High | Periodical / event-triggered | TCE | CONNECTED |
| Logged MDT | 64KB | High | Periodical / event-triggered | TCE | INACTIVE/ IDLE |
| SON | 9000 bytes | High | event-triggered | gNB | CONNECTED |
| UE assistance information | 9000 bytes | Low | One-shot (on-demand) | RAN node | CONNECTED |
| RRM measurement report | 9000 bytes | Low | Periodical / event-triggered | RAN node | CONNECTED |
| CSI reporting framework | Less than 1000 bits | Extremely Low | One-shot (on-demand) | RAN node | CONNECTED |
| LPP Provide location information | Infinity | Medium | One-shot (on-demand)/ Periodical/ event triggered | LMF | CONNECTED/ INACTIVE |

For logged MDT, UE will only perform the logging of measurement results in RRC_IDLE/RRC_INACTIVE state, the total data volume is limited, which may not be optimal for data collection. For example for the AI/ML use case of CSI compression, the current non-connected state UE will not perform CSI measurement. Besides, power saving is a key requirement for non-connected UE, so they may not be expected to perform frequent measurements.

Therefore, it is proposed to consider frameworks that can support data collection in RRC_CONNECTED first. In this sense, the UE can generate and report data directly, thus no extra UE storage is needed.

Proposal 2: Prioritize the frameworks that can support data collection in RRC_CONNECTED.

Based on the analysis of the data collection requirements of each component and the performance metrics of the existing data collection frameworks, we think the existing data collection frameworks can support the requirement and shall be reused with potential enhancement. To be specific:

- For CSI and beam management, if gNB is responsible for model training and/or model monitoring, UCI can be used for small-sized reports with extremely low latency requirements and RRC measurement reports can be reused for large-sized reports with low latency requirements. If the CN entity is responsible for model training, MDT can be reused for large-sized reports with no latency requirement.

- For positioning, if LMF is responsible for model training and/or model monitoring, the LPP message can be reused with a low latency requirement.

Thus, we propose:

Proposal 3: As baseline, reuse the existing data collection framework for AI/ML purposes, including model training/monitoring:

- If gNB is the termination entity of data collection (for CSI and Beam management) for model training/monitoring, the data can be collected via UCI (small report size) or RRC measurement report (large report size);

- If LMF is the termination entity of data collection (for positioning only) for model training/monitoring, the data can be collected via LPP message;

- If the CN (except LMF) is the termination entity of data collection for model training, the data can be collected via immediate MDT.

- 2.2 Model transfer/delivery

The following potential options of model transfer/delivery were considered in RAN2 post-meeting email discussion.

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4: Server can transfer/delivery AI/ML model(s) to UE (transparent to 3GPP).

For Solution 1a, RAN2 can independently have a further discussion as it is totally in RAN2's scope. Solution 2a/3a/1b/2b/3b may further require other WGs such as RAN3/SA2, involvement, thus RAN2 shall discuss how to progress on them, and the following options are suggested to be considered in the summary:

Proposal 7: For model transfer/delivery, RAN2 can further discuss Solution 1a. For Solution 2a/3a/1b/2b/3b, RAN2 to discuss how to progress on them (e.g. how it works, impacts to other WGs, pros/cons), and the following options can be considered: - RAN2 can send LS to other WGs for the study - RAN2 can identify requirements/impacts to other WGs, and then leave it to RAN plenary discussions - Proponents could start by triggering such discussion on other WGs first - RAN2 can study such impacts and not involve other WGs in the SI phase (can involve them in WI phase)

As RAN3/SA2 does not have TU in the SI phase, RAN2 may capture a general summary of the pros/cons for each solution in the SI phase and study how to address the cons of solutions independent of RAN3 or SA2. If SI concludes that identified solutions for normative work require the involvement of RAN3 and/or SA2, RAN3 and/or SA2 can be involved in the WI phase.

Proposal 4: RAN2 focuses on the study of the solutions of model transfer/delivery without the involvement of other WGs in the SI phase. If SI concludes that identified solutions for normative work require the involvement of RAN3 and/or SA2, they can be involved in the WI phase.

When analyzing the pros/cons of the potential solutions, the following evaluation metrics can be considered:

Capability to transfer/delivery models for the following model characteristics (RAN1/RAN2 may discuss it): AI/ML model size (e.g. individual model size, cumulative model size). It may have some categories, e.g. large size, small size - Model transmission/update frequency. It may have some categories, e.g. frequent/infrequent transmission/update - Latency. It may have some categories, e.g. low-latency/high-latency - Robustness Signalling overhead Support of delta configuration Impacts due to handover Impacts due to failures (e.g. radio link failure) Possible specification impacts (e.g. RAN2, SA2, and etc) Inter-operability impacts

Based on “[Post120][053][AIML18] model transfer delivery (Huawei)” outcome, an initial analysis on performance metrics of potential model transfer solutions can be as follows:

Table 2.2-1: performance metrics of potential model transfer solutions

|  | CP-based |  |  | UP-based |  |  | Server-based |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Solution 1a | Solution 2a | Solution 3a | Solution 1b | Solution 2b | Solution 3b | Solution 4 |
| Inter-interoperability issue | No | No | No | No | No | No | Yes |
| Latency | Low | Limited | Limited | Not guaranteed |  |  | High |
| Signaling overhead | Depends on the typical model size |  |  | No | No | No | No |
| Impacts due to handover | Depends on the model generalization performance |  |  | No | No | No | No |
| Support of Delta configuration | Yes | Yes | Yes | Up to implementation |  |  |  |
| RAN2 spec impact | Medium | No | No | High | No | No | No |
| SA2 spec impact | No | High | No | Medium | High | No | No |

Based on the above comparison, UP-based model transfer solutions can not guarantee the latency of model transfer, which is one of the key performance metrics, especially for CSI and beam management use cases. For solution 4, it is out of 3GPP scope and massive offline negotiation is needed to align the model information.It is less practical as more and more network/UE vendors would be involved due to the lack of a uniform standard. As Solution 1a and Solution 3a can basically satisfy current AI/ML use cases model transfer requirements and these two solutions are in RAN2’s, we propose:

Proposal 5: RAN2 to further investigate the following solutions for model transfer:

- Solution 1a (CP solution between UE and gNB): prioritized for CSI feedback enhancement and Beam management use cases.

- Solution 3a (CP solution between UE and LMF): prioritized for positioning accuracy enhancement use case.

Solution 3a does not have size limitation issue, but Solution 1a may be negatively impacted some issues originating from RRC signaling transmission, as follows:

- Issue 1: the existing downlink RRC transmission supports up to 5 segments, i.e. maximum of 45KB signaling transmission, and larger-sized models may require more RRC segments.

- Issue 2: using SRB to transmit the model will affect the existing control signaling.

- Issue 3: RRC signaling transmission does not support mobility continuity, i.e. the incomplete RRC signaling transmission cannot be continued during Handover.

Whether Issue 1 exists depends on the typical model size determined by RAN1. If it is only dozens or hundreds of KB, it can be solved by slightly increasing the number of segments, by RRC segmentation. If the size is too large, such as dozens or hundreds of MB, too many segments is yet not appropriate. Another approach is to introduce a unified protocol layer (e.g., AI layer) to support the AI functions. Besides, the model transfer scheme should be the same for each AI use case, e.g., the model segment scheme. In this way, we think that it is feasible and is likely to support more AI use cases in the future.

Figure 2.1-1 protocol stack for the new AI layer

With above discussion, we can summarize with two options:

- Option 1: Rely on RRC segmentation to transfer larger size model

- Option 2: Introduce a unified protocol layer (e.g., AI layer) to support the model transfer (including model segment/re-segment) for each use case.

Option 2 would introduce a unified solution to support the model transfer, with future proof. Thus, we think Option 2 could be a good way forward for model transfer.

Therefore,

Proposal 6: To support the model transfer using Solution 1a, RAN2 to further consider the following two options:

- Option 1: Rely on RRC segmentation to transfer larger size model

- Option 2: Introduce a unified protocol layer (e.g., AI layer) to support the model transfer (including model segment/re-segment) for each use case.

For Issue 2, a lower-priority SRB can be used to transmit the signaling of the model to reduce the impact on other high-priority signaling. In R17, SRB4 was introduced to transmit QoE reports, which can be reused for model transfer. Another straightforward way is to introduce a similar low-priority SRB.

Proposal 7: To reuse existing SRB4 or to define a new low-priority SRB for model transfer to reduce the impact on the transmission of other high-priority signalings.

Issue 3 is related to model generalization performance. Specifically, if the model generalization performance is poor, such as each model only applies to the serving cell, then the incomplete RRC signaling transmission does not have to continue after UE handover to another cell. In addition, to ensure the robustness of model transfer, the new model for the target cell can be transmitted by delta configuration after handover, thus quick model deployment and activation can be achieved.

Proposal 8: Delta configuration for model transfer shall be supported to ensure the robustness of model transfer during or after handover.

In the RAN2 #119bis meeting, RAN2 discussed model format and reached the following agreements:

R2 assumes that for the existing (under discussion) AI/ML use cases, proprietary models may be supported and/or open format may be supported (and maybe RAN2 doesn’t have to further elaborate on this assumption).

Besides, how to align the AI/ML framework between two sides (UE and NW) needs to be studied. Currently, there are plenty of AI/ML frameworks, such as TensorFlow, PyTorch, and Caffe. Each AI/ML framework has its model storage format and they are not mutually compatible when loading the model from other AI/ML frameworks, for example, the ‘.h5’ format is used for TensorFlow and the ‘.pth’ is used for PyTorch,which are not interchangeable.

Regarding how to align these AI/ML frameworks between UE and NW, the following options can be considered:

- Option 1: one side reports the supported AI/ML framework format(s) and the other side chooses one of them as the format for the transferred model, e.g., UE may indicate the supported model format(s) via UE capability transfer.

- Option 2: define a specific format that is recognizable by multiple parties, either reusing existing formats such as ONNX or defining a new format for model description in 3GPP. Defining a new format may be more efficient in view of overhead and may potentially be forward-compatible, while a considerable effort on standardization must be required.

Proposal 9: Study how to align the AI/ML format transferred between the network and UE, the candidate options include:

- Option 1: A group of open formats negotiated by two sides,

- Option 2: One specific format used by both sides, either reusing existing format such as ONNX or introducing a new specified format in 3GPP.

- 2.3 Model/functionality identification

At the RAN1#111 meeting, the following working assumption was made regarding the model/functionality identification.

| Terminology | Description |
| --- | --- |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |

To our understanding, the model/functionality identification is to align the information of model/functionality between the network and UE for further life cycle management. In order to identify possible issues, we categorize it by the initiating entity into UE-initiated and network-initiated identification. The difference between them is whether the entity with model information is the network or the UE at the very beginning.

For Network-initiated identification, the model information is sent from the network to UE and the model identification can be performed during model transfer, that is, model information can be transmitted together with AI model. For model identification, one or multiple models can be maintained for each functionality and each model shall associate with a unique model ID (globally unique) for life cycle management. Due to the need for flexible management (e.g., modification/deletion) of specific model(s) at the network side, functionality identification is not needed for network-initiated identification.

UE-initiated identification is to inform the network that AI functionality or AI/ML models are available at the UE. There can be two typical scenarios:

- Scenario 1: UE may store the AI/ML model transferred from the network when entering RRC_IDLE. After entering RRC_CONNECTED, UE can perform the model identification to avoid duplicated model transfer from the network. Likewise, each model shall also associate with the globally unique model ID previously allocated by the network.

- Scenario 2: UE may indicate its supported AI/ML-based functionality through UE capability. In this case, the model does not need to associate model ID since the network is able to manage the model per functionality.

Proposal 10: A globally unique model ID shall be associated with each model during model identification.

Proposal 11: UE indicates its supported AI/ML-based functionality through UE capability during functionality identification.

- Conclusion

Data collection

Proposal 1: When analyzing the existing data collection frameworks for data collection, the following performance metrics can be considered: 1) maximum data size of each report, 2) report latency, 3) report type, 4) termination entity of report, 5)RRC state.

Proposal 2: Prioritize the frameworks that can support data collection in RRC_CONNECTED.

Proposal 3: As baseline, reuse the existing data collection framework for AI/ML purposes, including model training/monitoring:

- If gNB is the termination entity of data collection (for CSI and Beam management) for model training/monitoring, the data can be collected via UCI (small report size) or RRC measurement report (large report size);

- If LMF is the termination entity of data collection (for positioning only) for model training/monitoring, the data can be collected via LPP message;

- If the CN (except LMF) is the termination entity of data collection for model training, the data can be collected via immediate MDT.

Model transfer/delivery

Proposal 4: RAN2 focuses on the study of the solutions of model transfer/delivery without the involvement of other WGs in the SI phase. If SI concludes that identified solutions for normative work require the involvement of RAN3 and/or SA2, they can be involved in the WI phase.

Proposal 5: RAN2 to further investigate the following solutions for model transfer:

- Solution 1a (CP solution between UE and gNB): prioritized for CSI feedback enhancement and Beam management use cases.

- Solution 3a (CP solution between UE and LMF): prioritized for positioning accuracy enhancement use case.

Proposal 6: To support the model transfer using Solution 1a, RAN2 to further consider the following two options:

- Option 1: Rely on RRC segmentation to transfer larger size model

- Option 2: Introduce a unified protocol layer (e.g., AI layer) to support the model transfer (including model segment/re-segment) for each use case.

Proposal 7: To reuse existing SRB4 or to define a new low-priority SRB for model transfer to reduce the impact on the transmission of other high-priority signalings.

Proposal 8: Delta configuration for model transfer shall be supported to ensure the robustness of model transfer during or after handover.

Proposal 9: Study how to align the AI/ML format transferred between the network and UE, the candidate options include:

- Option 1: A group of open formats negotiated by two sides,

- Option 2: One specific format used by both sides, either reusing existing format such as ONNX or introducing a new specified format in 3GPP.

Model/functionality Identification

Proposal 10: A globally unique model ID shall be associated with each model during model identification.

Proposal 11: UE indicates its supported AI/ML-based functionality through UE capability during functionality identification.

- Reference

[1]. RP-213599		Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface