3GPP TSG-RAN WG2 Meeting #121	R2-2300682

Athens, Greece, 27th February – 3rd March, 2023

Source: 	vivo

Title:  	Discussion on MRO for CPAC

Agenda Item:	8.13.8

Document for:	Discussion and Decision

Introduction

In RAN2#120 [1], the following agreements were achieved regarding CPAC failure:

Agreements:

1	RAN2 confirms the CPA/CPC scenarios agreed by RAN3 and discuss corresponding UE impacts.

2	 SCGFailureInformation is enhanced to support CPAC MRO (i.e, no need to introduce new reports/message).

In this contribution, we will provide our analyses and proposals on the contents of CPAC failure.

Discussion

According to the last RAN3 agreements [2], the concept of “Too Late CPC Execution”, “Too Early CPC Execution”, and “CPC/CPA Execution to wrong PSCell” is clarified by RAN3 as follows:

MRO for CPAC: Not consider too late CPA. CPA Execution to wrong PSCell will be considered, e.g. UE receives CPA configuration and CPA execution condition is satisfied, CPA execution fails or an SCG failure occurs shortly after a successful CPA execution; a suitable PSCell different with target PSCell is found based on the measurements reported from the UE. Too Late CPC Execution, Too Early CPC Execution and CPC Execution to wrong PSCell will be considered: - Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different with source PSCell is found based on the measurements reported for the UE. - Too Early CPC Execution: UE receives CPC configuration and CPC execution condition is satisfied, CPC execution fails or an SCG failure occurs shortly after a successful CPC execution; source PSCell is still the suitable PSCell based on the measurements reported from the UE. - CPC Execution to wrong PSCell: UE receives CPC configuration and CPC execution condition is satisfied, CPC execution fails or an SCG failure occurs shortly after a successful CPC execution; a suitable PSCell different with source PSCell or target PSCell is found based on the measurements reported from the UE.

If UE is configured with CPAC, based on the CPAC failure information reported from UE, the NW can figure out which type of failure really occurred, and possibly it can optimize the CPAC configurations in the future. In this section, we will discuss the contents in detail that needs to be reported for different types of CPAC failures.

Too Late CPC Execution

In CPC, the NW configures the UE with candidate target cells and the execution conditions for each candidate target cell. Then the UE evaluates the execution conditions for the candidate target cells. But SCG failure occurs while doing such an evaluation. It is necessary for the NW to know that SCG failure occurred when the UE is configured with CPC configurations, so the candidate target cells can be included in the report. Additionally, if the network knows the latest radio measurements related to the candidate target cells, the NW can properly tune the PSCell parameters for the execution conditions. It might also be beneficial if the NW obtains the latest radio measurements of the neighbour cell(s) which is not candidate cell before transmitting the CPC failure information, so that the network can add the neighbour cell(s) as candidate target cell(s). In addition, similar to the RLF report due to CHO, the time between the reception of the CPC configuration to the time it experienced the SCG failure may be also useful to the NW.

Too Early CPC Execution and CPC/CPA Execution to wrong PSCell

UE cannot distinguish between Too Early CPC Execution and CPC Execution to wrong PSCell since it is related to the subsequent configuration of the NW after the SCG failure. So from the perspective of UE, the contents related to these two scenarios are the same.

For CPAC, the UE executes the SCG reconfiguration with sync upon one of the target cells’ execution conditions being fulfilled, and then SCG reconfiguration with sync failure occurs. Intuitively, the target PSCell ID should be included in the SCG failure information and failedPSCellId can be reused. Similar to the analyses of too-late CPC Execution below, the candidate target cells, the latest radio measurements related to the candidate target cells and the latest radio measurements of neighbour cell(s) are useful for the NW to modify configuration of candidate cells. In this scenario, the time between reception of the CPAC configuration and the SCG failure can include two parts, one is the time between reception of the CPAC configuration and CPAC execution, and the other is the time between CPAC execution and SCG failure, for the time between CPAC execution and SCG failure, the existing timeSCGFailure can be reused.

Based on the analyses for the above scenarios, we propose the following:

Proposal 1: The following information should be introduced in SCGFailureInformation to support CPAC MRO:

- Candidate target cell ID;

- latest radio measurements of the candidate target cells;

- latest radio measurements of neighbour cell(s);

- the time between reception of the CPC configuration and SCG failure;

- the time between reception of CPAC configuration and CPAC execution.

During the last meeting, whether to introduce explicit indication or rely on implicit information to differentiate CAPC from conventional SCG failure is FFS. From our perspective, if candidate cell related information (e.g., candidate cell ID and measurements of the candidate cells) is agreed to be included in SCGFailureInformation for CPAC MRO, the network can deduce whether a conventional SCG failure or a CPAC is occurred based on whether the candidate cell related information is included in SCGFailureInformation. Thus, explicit indication to differentiate CAPC from conventional SCG failure is not needed.

Proposal 2: Explicit indication to differentiate CAPC from conventional SCG failure is not needed.

Furthermore, whether additional information should be introduced to help MN transfer the SCG failure information to the corresponding node should also be discussed. Since the SCGFailureInformation message is an immediate message, i.e., the UE transfers the SCGFailureInformation to MN once SCG failure has occurred, MN can implicitly infer the CPAC type (e,g., MN initiated inter-SN CPC, SN initiated inter-SN CPC or SN initiated intra-SN CPC) by the structure of the configuration signaling and the content of the SCG failure information. Therefore, explicit indication of CPAC type is not needed.

Proposal 3: Explicit indication of CPAC type (i.e, CPA, MN-initiated inter-SN CPC, SN-initiated inter-SN CPC or SN-initiated intra-SN CPC) is not needed.

Conclusions

Based on the analyses given above, we have the following proposals:

Proposal 1: The following information should be introduced in SCGFailureInformation to support CPAC MRO:

- Candidate target cell ID;

- latest radio measurements of the candidate target cells;

- latest radio measurements of neighbour cell(s);

- the time between reception of the CPC configuration and SCG failure;

- the time between reception of CPAC configuration and CPAC execution.

Proposal 2: Explicit indication to differentiate CAPC from conventional SCG failure is not needed.

Proposal 3: Explicit indication of CPAC type (i.e, CPA, MN-initiated inter-SN CPC, SN-initiated inter-SN CPC or SN-initiated intra-SN CPC) is not needed.

Reference

- Report of 3GPP TSG RAN WG2 meeting #120

- Chairman_nots of 3GPP TSG RAN WG3 meeting #117bis-e, Online