# 3GPP TSG RAN WG2 Meeting #121      	                           R2-2300704
Athens, Greece, 27th Feb. – 3rd March 2023

Agenda item:	8.3.3

Source:	Apple

Title:	Discussion on RAN2 work of inter-band SSB-less CA

WID/SID:	Netw_Energy_NR-Core– Release 18

Document for:	Discussion and Decision

# 1 Introduction

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2]

The Network Energy saving (NES) WID RP-223540 was agreed in RAN#98-e [1], the WI objective on inter-band SSB-less CA is copied below:

In this contribution, we share our view on RAN2 work of inter-band SSB-less CA for below aspects:

- RAN2 spec impacts on signaling

- L3 measurement and SCell activation procedure

# 2 Discussion

## 2.1 RAN2 spec impacts on signalling

Intra-band SSB-less CA was specified in NR Rel-15. RAN1/RAN4 first discussed and concluded its timing issue, and then RAN2 introduced below 2 specification changes:

- TS 38.331: In field description of IE absoluteFrequencySSB, capture RAN1/RAN4 conclusion that timing reference of SSB-less SCell is from the SpCell or an SCell of its serving cell group.

- TS 38.306: Specified UE capability of intra-band SSB-less CA.

Observation 1: RAN2 specification impacts on intra-band SSB-less CA include capturing timing assumption in TS 38.331 and UE capability in TS 38.306.

For inter-band SSB-less CA with restriction of FR1 and co-located cells, we think that RAN2 should first wait whether RAN4 conclude it is feasible. If RAN4 conclude it is feasible, we assume that the above 2 specification changes introduced for intra-band SSB-less CA can be reused for inter-band SSS-less CA. However, considering inter-band CA has some differences from intra-band CA, we don't think RAN2 can preclude new signaling if RAN4 conclude its necessity. For example, RAN4 may discuss below issues:

- Issue 1: whether gNB can indicate which serving cell to use as timing reference

  - For example, if PCell and SSB-less cell are inter-band CA while another SCell with SSB and SSB-less cell are intra-band CA, the UE may be confused whether PCell or SCell with SSB is its timing reference.

- Issue 2: whether TAE from gNB is needed

  - Please note that in intra-band CA, the timing alignment error (TAE) is always smaller than 0.5 CP. Thus, gNB doesn't need to indicate TAE in case of intra-band SSB-less CA. But TAE of inter-band CA may be larger than 0.5CP, even if it is co-located. Thus, further RAN4 discussion is required.

Observation 2: Considering inter-band CA has some differences from intra-band CA, RAN2 can't preclude new signaling if RAN4 conclude its necessity.

Based on above analysis, we propose that RAN2 can take signaling of intra-band SSB-less CA as baseline, but it doesn't preclude the possibility that RAN4 conclude new signaling is required.

Proposal 1: If RAN4 conclude SSB-less SCell for inter-band CA for FR1 and co-located cells is feasible, the signaling of intra-band CA (including RRC change on timing of SSB-less SCell and capability signaling) can be considered as its baseline. Whether other new signaling is required depends on RAN4 input.

## 2.2 L3 measurement and SCell activation procedure

According to WID objective, L1/L3 measurement and SCell activation procedure are in scope. Among them, L3 measurement and SCell activation procedure may have some RAN2 impacts. However, our understanding is that this part was added in WID objective because it is not clear whether the existing measurement solution of intra-band SSB-less CA can be reused. Specifically, in existing intra-band SSB-less CA, the SSB measurement in PCell is reused for SSB-less SCell. Then, RAN4 need to discuss whether SSB measurement in PCell can still be reused in inter-band SSB-less-less SCell. If not, maybe CSI-RS based measurement for SSB-less SCell needs to be introduced. In this case, the SCell activation / deactivation procedure in TS 38.321 may need some spec changes.

Observation 3: L1/L3 measurement and SCell activation were added in WID objective because it is not clear whether the existing measurement solution of intra-band SSB-less CA (i.e. the SSB measurement in PCell is reused for SSB-less SCel) can be reused.

Based on above analysis, we think whether introducing new L3 measurement and SCell activation procedure depends on RAN4 conclusion. Thus, RAN2 should not discuss this part until RAN4 provide more input.

Proposal 2: Before RAN4 provide sufficient input, RAN2 do not start discussion on enhancement of L3 measurement and SCell activation procedures.

# 3 Conclusion

In this contribution, we share our view on RAN2 work of inter-band SSB-less CA. Our observations are:

Observation 1: RAN2 specification impacts on intra-band SSB-less CA include capturing timing assumption in TS 38.331 and UE capability in TS 38.306.

Observation 2: Considering inter-band CA has some differences from intra-band CA, RAN2 can't preclude new signaling if RAN4 conclude its necessity.

Observation 3: L1/L3 measurement and SCell activation were added in WID objective because it is not clear whether the existing measurement solution of intra-band SSB-less CA (i.e. the SSB measurement in PCell is reused for SSB-less SCel) can be reused.

Based on observations, our proposals can be found below.

Proposal 1: If RAN4 conclude SSB-less SCell for inter-band CA for FR1 and co-located cells is feasible, the signaling of intra-band CA (including RRC change on timing of SSB-less SCell and capability signaling) can be considered as its baseline. Whether other new signaling is required depends on RAN4 input.

Proposal 2: Before RAN4 provide sufficient input, RAN2 do not start discussion on enhancement of L3 measurement and SCell activation procedures.

# 4 References

[1] RP-223540, New WID: Network energy savings for NR, Huawei