# 3GPP TSG RAN WG2 Meeting #121      	                           R2-2311711
Athens, Greece, 27th Feb. – 3rd March 2023

Agenda item:	8.12.2

Source:	Apple

Title:	Cell (re)selection and RNAU in mobile IAB

WID/SID:	NR_mobile_IAB-Core – Release 18

Document for:	Discussion and Decision

# 1 Introduction

WID of mobile IAB (RP-213601) was agreed in RAN#94e [1]. The related WID objectives on mobility enhancement are summarized below.

- Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2]

Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility.

- Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

The following principles should be respected:

- Mobile IAB-nodes should be able to serve legacy UEs.

- Solutions providing optimization for Mobile IAB may entail Rel-18 UE enhancements, provided that such enhancements are backwards compatible

In this contribution, we discuss the following aspects on IDLE / INACTIVE UE mobility:

- Cell reselection enhancement

- RNAU

Note that we discuss CONNECTED UE mobility in our companion contribution [5].

# 2 Discussion

## 2.1 Cell reselection enhancement

In RAN2#119b-e [2], cell reselection enhancement was discussed, and below agreements were made:

RAN2 observes that a UE could potentially consider itself on-board of a mobile-IAB cell, if the UE camps on/connects to a mobile IAB cell during a long period (i.e. the UE then need to know that this is such a cell). FFS the time. FFS if this is needed.

RAN2 assumption: For the mobile IAB cell broadcasting info:

- 1 bit mobile-IAB cell type indication is introduced, to assist mobility in Idle/Inactive mode for Rel-18 UEs (FFS if to assist UE to know it is onboard, if this need to be known)

- FFS how this is used (might be implementation specific).

In RAN2#120 [3], RAN2 tried to confirm the above WA with the compromise that the UE determines whether it is onboard by its implementation. However, it was not agreed.

Regarding the assumed mobile-IAB cell type indication, RAN2 assumes is may be specified if some related UE behaviour is specified.

First, from history of 3GPP similar discussions, it was always hard to converge to a specified speed estimation solution because 3GPP can only specify air interface related solution but UE may rely on other techniques (e.g. its sensor info), which is out of 3GPP scope.

Observation 1: From history of 3GPP similar discussions, it was always hard to converge to a specified speed estimation solution because 3GPP can only specify air interface related solution but UE may rely on other techniques (e.g. its sensor info), which is out of 3GPP scope.

Proposal 1: It is left to UE implementation how to determine it is onboard because it is out of scope

If the UE can determine it is onboard, the UE may prioritize the cell reselection to a mobile IAB cell. We think it can be captured in TS 38.304 as specified UE behaviour.

Proposal 2: In TS 38.304, capture the specified UE behavior as: "Mobile IAB capable UE prioritizes the cell reselection to a mobile IAB cell, if the UE determines itself on-board of this mobile IAB cell".

## 2.2 RNAU

In RAN3#118, RAN3 agreed the dynamic TAC solution, and send LS to RAN2 in R3-226831 [5]. We share our views on corresponding RAN2 impacts in this section.

Static TAC solution is not pursued. RAN3 assumes that dynamic TAC solution should be supported.

### 2.2.1 IDLE / INACTIVE UE handling during full migration

We think the first issue is how IDLE / INACTIVE UEs should be handled after inter-donor full migration is triggered. In RAN2#119b-e [2], dual-DU model during full migration was agreed:

RAN2 focuses on the scenario where, during full migration, the UE sees the two logical DU cells as different physical cells (e.g. with different PCI if same carrier), and where the two logical DU cells use separate physical resources (i.e., different carriers, or orthogonal time and frequency resources of the same carrier, as supported by legacy L1).

This dual-DU modelling implies that the source DU will be released by donor CU after completion of full migration. Thus, the served UEs is expected to leave the source DU after full migration is triggered. However, unlike CONNECTED UEs, IDLE / INACTIVE UEs can't be indicated by NW that full migration is triggered.

Observation 2: Dual-DU modelling implies that the source DU will be released by donor CU after full migration. Thus, the served UEs is expected to leave the source DU after full migration is triggered. However, unlike CONNECTED UEs, IDLE / INACTIVE UEs can't be indicated by NW that full migration is triggered.

Thus, the IDLE / INACTIVE UEs camping in source DU should perform cell reselection, and source DU should avoid new IDLE / INACTIVE UEs camping.

Observation 3: After full migration is triggered, the IDLE / INACTIVE UEs camping in source DU should perform cell reselection, and source DU should avoid new IDLE / INACTIVE UEs camping.

This issue is similar to the discussion on CHO enhancement. And similar solutions can be considered. To be specific, RAN2 can discuss below 3 alternatives:

- Alt-1: via setting cellBarred in MIB (applicable to legacy UE and Rel-18 UE)

- Alt-2: via decreasing transmit power of source DU (applicable to legacy UE and Rel-18 UE)

- Alt-3: via cell reselection enhancement (i.e. prioritize reselection to target DU)

Thus, we propose:

Proposal 2: RAN2 discuss how to avoid IDLE / INACTIVE UEs camping in source DU after inter-donor full migration is triggered. The candidate solutions include:

- Alt-1: via setting cellBarred in MIB (applicable to legacy UE and Rel-18 UE)

- Alt-2: via decreasing transmit power of source DU (applicable to legacy UE and Rel-18 UE)

- Alt-3: via cell reselection enhancement (i.e. prioritize reselection to target DU)

### 2.2.2 RNAU trigger after full migration

We think another issue is whether the INACTIVE UEs should trigger RNAU if these INACTIVE UEs reselect from source DU to target DU after full migration.

In legacy RNAU, the CONNECTED UE is configured with its RAN Notification Area Info (RNA info) via RRC release message. The RANA info (i.e. RAN-NotificationAreaInfo) can be a list of Cell ID or a RAN Area Code (RANAC). After the INACTIVE UE reselects to another cell, it will check whether the new cell is within the configured RNA via the NCGI or RANAC broadcast in new cell's SIB. If the new cell is of out of configured RNA, the UE will trigger RNAU so that new cell can retrieve the UE's context from last serving cell.

A key difference of RNAU during full migration is that the UE location change is caused by NW mobility rather than UE mobility. In this case, we think it is NW responsibility to handle RNAU for its served INACTIVE UEs.

Proposal 3: Unlike legacy RNAU triggered by UE mobility, it is NW mobility triggered RNAU during full migration. Thus, it should be NW responsibility to handle RNAU for its served INACTIVE UEs.

If Proposal 3 can be agreed, then when INACTIVE UEs reselect from source DU to target DU after full migration, these UEs should avoid triggering RNAU. The detailed mechanism can be further discussed.

Proposal 4: If INACTIVE UEs reselect from source DU to target DU after full migration, these UEs should avoid triggering RNAU. FFS detailed mechanism.

# 3 Conclusion

In this contribution, we discuss cell (re)selection and RNAU in mobile IAB. Our observations are:

Observation 1: From history of 3GPP similar discussions, it was always hard to converge to a specified speed estimation solution because 3GPP can only specify air interface related solution but UE may rely on other techniques (e.g. its sensor info), which is out of 3GPP scope.

Observation 2: Dual-DU modelling implies that the source DU will be released by donor CU after full migration. Thus, the served UEs is expected to leave the source DU after full migration is triggered. However, unlike CONNECTED UEs, IDLE / INACTIVE UEs can't be indicated by NW that full migration is triggered.

Observation 3: After full migration is triggered, the IDLE / INACTIVE UEs camping in source DU should perform cell reselection, and source DU should avoid new IDLE / INACTIVE UEs camping.

Based on observations, our proposals are:

Cell reselection

Proposal 1: It is left to UE implementation how to determine it is onboard because it is out of scope

Proposal 2: In TS 38.304, capture the specified UE behavior as: "Mobile IAB capable UE prioritizes the cell reselection to a mobile IAB cell, if the UE determines itself on-board of this mobile IAB cell".

RNAU

Proposal 2: RAN2 discuss how to avoid IDLE / INACTIVE UEs camping in source DU after inter-donor full migration is triggered. The candidate solutions include:

- Alt-1: via setting cellBarred in MIB (applicable to legacy UE and Rel-18 UE)

- Alt-2: via decreasing transmit power of source DU (applicable to legacy UE and Rel-18 UE)

- Alt-3: via cell reselection enhancement (i.e. prioritize reselection to target DU)

Proposal 3: Unlike legacy RNAU triggered by UE mobility, it is NW mobility triggered RNAU during full migration. Thus, it should be NW responsibility to handle RNAU for its served INACTIVE UEs.

Proposal 4: If INACTIVE UEs reselect from source DU to target DU after full migration, these UEs should avoid triggering RNAU. FFS detailed mechanism.

# 4 References

[1] RP-213601, New WID on mobile IAB.

[2] RAN2#119b-e, Chair Notes.

[3] RAN2#120, Chair Notes.

[4] R3-226831, LS on static and dynamic TAC solutions for mobile IAB node, ZTE.

[5] R2-2300710, CONNECTED mobility enhancement in mobile IAB, Apple.