3GPP TSG-RAN WG2 Meeting #121	R2-2300724

Athens, Greece, 27 February – 03 March 2023

Agenda item:	8.5.2.2

Source:	Apple

Title:	Views on XR-awareness and PDU Prioritization

WID/SID:	FS_NR_XR_enh

Document for:	Discussion / Decision

# Introduction

In continuation of the 3GPP work on XR in RAN1 and SA4 in Rel-17, RAN has approved a RAN2-led study item on XR enhancements for NR in Rel-18 [1]. According to the study item description, RAN2 should study how XR awareness can help aid XR-specific traffic handling.

The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). 1. Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling.

For Rel-18 XR, it is anticipated that the RAN and the UE are required to identify different PDU Sets and distinguish between them, including the PDUs that belong to each PDU Set. This may impact the treatment and prioritization of PDUs in L2. RAN2#121 will continue to discuss whether there is a need to treat PDU Sets of the same QoS flow differently over the air interface and whether there are any RAN2 impacts for PDU Prioritization.

This paper discusses some of our views on whether XR awareness impacts traffic prioritization of XR traffic. Furthermore, we provide our perspective on XR traffic flows not based on PDU Sets.

# Discussion

## Considerations on Traffic Prioritization

XR services can operate on a “PDU Set” which comprises multiple IP packets, and each QoS PDU can be mapped to a QoS flow. A QoS flow is identified using QoS Flow Id and each PDU Set within the QoS Flow may be identified using a PDU Set SN. Each QoS flow can be used to deliver one or more PDU Sets. The UE and the network may further identify information relating to each PDU Set, such as PDU Set importance and PDU Set dependency. The CN may also provide the information relating to PDU Sets to RAN.

New QoS parameters for PDU Set based QoS handling may be defined in the 5G system, such as:

- PDU Set Delay Budget (PSDB)

- PDU Set Error Rate (PSER)

- Whether to drop a PDU Set in case PSDB is exceeded

- Whether all PDUs are needed for the usage of PDU Set by application layer (PSIHI)

Moreover, a PDU Set may be characterized by a notion of “importance” indicating how important the PDU Set (e.g., a particular video frame) is for the application. For example, while an important PDU Set could be an I-Frame, a less important PDU Set could be a P-Frame. Figure 1 shows an example.

Figure 1: Illustration of important and non-important PDU Sets in one XR traffic stream

According to SA2 [2], the RAN can obtain the information of importance for each PDU Set (e.g., from the CN or from the application). The parameter ‘PDU Set Importance’ is used for that purpose. SA2 suggested that RAN may use the parameter for PDU Set level packet discarding in presence of congestion.

In our understanding, a PDU Set of certain importance may be scheduled with a different LCH priority compared to a PDU Set where packet discarding may happen more frequently according to the configured PDU Set Importance. This can be at the discretion of the RAN. Therefore, it seems sensible to map PDU Sets of different importance to different RLC entities and to treat them, e.g., with different LCH priority for the scheduling over the radio interface. As an optional addition (at the discretion of RAN), important PDU Sets might also be treated with better reliability over the air interface, while non-important PDU Sets could be processed in a more relaxed manner to improve user experience and/or resource efficiency.

Likewise, QoS flows with different PDU Set Integrated Handling Indication (PSIHI) may benefit from scheduling with different LCH priority. For example, consider a case where two QoS flows are mapped to the same DRB. Both QoS flows may have the same PSDB and PSER, but PSIHI may be different. In this case the QoS flow that requires all PDUs for the usage of the PDU Set by the application could be treated at slightly higher LCH priority in the RAN, that is, even if QoS flow 1 and 2 have the same priority as part of their QoS characteristics.

In light of the observations above, it may be beneficial if a transmitter can be configured with different prioritization settings for PDU Sets or QoS flows mapped to the same data radio bearer. Awareness of groups of packets is considered beneficial for power and capacity. However, existing prioritization mechanisms should be reused for as much as possible to limit RAN and UE complexity. In order to switch between LCH priorities, a DRB may be configured with at least two RLC entities, whose LCHs have different parameterizations. A PDCP entity can be configured by RRC to determine which RLC entity should be used for the packets of a PDU Set or QoS flow, when processing the PDU Set on a given DRB. With multiple RLC entities configured, the selection of RLC entity subset for submission of a PDCP PDU to RLC can be defined accordingly.

Proposal 1: RAN2 may consider the selection of multiple RLC entities for XR traffic.

## QoS Flow Awareness at MAC

For XR traffic, an application not only has multiple streams of concurrent data where QoS requirements may differ, some of the streams also have similar QoS requirements as well. As a result, in this option, multiple QoS flows or PDU Sets may be mapped to the same DRB. However, each of the traffic flows has its own traffic arrival pattern and presumably its own set of associated configured grants. Based on the current 5G NR design the MAC layer cannot identify different QoS flows within a LCH and there is no clear mapping between CGs and QoS flows / QFIs. This means MAC cannot select the most appropriate data when constructing a MAC PDU as it cannot identify SDUs based on QoS flows or QFIs within the LCH.

For example, when multiple flows are associated with a DRB, for a given logical channel it may be intended to have a 1:1 mapping between QoS flow 1 and CG1, a 1:1 mapping between QoS flow 2 and CG2, and QoS flow 3 could be mapped to CG3 and CG4. There is no clear mapping between QoS flows and radio resources (such as CGs) for a given logical channel in the current NR specifications. That is, multiple QoS flows can be mapped to the same DRB / LCH, and at the same time the network can configure multiple CGs for the same LCH, but there is no clear mapping between QoS flow and CG (or grants in general) within the same LCH.

Observation 1: In the current 5G NR design the MAC layer cannot identify different QoS flows within a LCH and there is no clear mapping between CGs and QoS flows / QFIs.

Assuming a differentiated treatment is deemed necessary at lower layers when multiple QoS flows are mapped to the same DRB, although not our first preference, a mapping between QoS flows and LCHs may need to be available to MAC. In other words, RAN2 may have to consider introducing a notion of the QFI at MAC level.

To achieve this, a LCH might be configured with a list of QFIs. So, for example, the LogicalChannelConfig IE can have an LCP restriction that is linked with a QFI/LCH, or potentially the configuredGrantConfig itself can be associated with a QFI. MAC can use this information to enhance the prioritization rules, not only to consider the LCH priority but to consider the association with a QFI as well.

However, we would prefer to rather arbitrate XR traffic in higher layers and keep lower layers close to legacy operation, depending on the overall mapping of QoS flows and DRB that RAN2 will come up with.

Proposal 2: Subject to QoS mapping decisions in RAN2, if multiple QoS flows or PDU Sets of different importance are mapped to the same DRB and differentiation of traffic is considered unavoidable in lower layers, the MAC layer needs to identify data with different severity within a LCH.

## Traffic flows not based on PDU Sets

RAN2 had a brief discussion of traffic flows not based on PDU Sets. The topic got initially discussed in email discussion [Post-119][261][XR] but could not be concluded in RAN2#119bis-e [3] and RAN2#120.

We assume XR traffic flows consist of multiple flows where some of them are based on groups of packets (i.e., PDU Sets) and some are not based on PDU Sets (such as pose or control information, or traditional audio). The traffic flows not based on PDU Sets literally consist of single packets. However, such traffic flows may need to be transmitted in a manner synchronized with other XR traffics. For example, pose control information relates to other XR traffic flows and influences the way information is represented to the user. Therefore, it needs to be synchronized within bounds just like audio has a close timing relation with video (lip-sync).

As far as power saving is concerned (which was the original discussion) we think that it is useful to provide the RAN with their periodicity, start offset and (where applicable) with a range of jitters for UE power savings.

In our understanding, traffic flows not based on PDU Sets can be handled by two ways: a) in traditional way based on single packet QoS, or b) based on PDU Set QoS. For case b), traffic flow can be considered to consist of only one packet per PDU Set. Their treatment according to a) or b) may differ depending on whether there is a close relation to other traffic flows within the multiple flows that encompass an XR service.

For example, SA2 assumes in their conclusions for Key Issue #1 that streams that are closely related and require strong application coordination are transmitted in a single PDU session by a single UE. However, those data streams that contribute to the immersive experience, but may still be valid stand-alone, may be transmitted over separate PDU sessions from multiple UEs [2].

Proposal 3: “Traffic flows not based on PDU Sets” should be characterized based on their contextual relation to other XR traffic flows and/or PDU Sets to be treated.

Proposal 4: “Traffic flows not based on PDU Sets” can be treated in two ways on a QoS flow, DRB or LCH:

- In traditional per-packet fashion (when its PDUs are independent of other XR traffic flows); or

- As PDU Set with “number of packets = 1” (when its PDUs are closely related to other XR traffic flows) e.g. to keep them in the framework for XR traffic.

# Conclusions

This contribution provides a view on XR awareness and PDU Prioritization. We have following observations and proposals:

Proposal 1: RAN2 may consider the selection of multiple RLC entities for XR traffic.

Observation 1: In the current 5G NR design the MAC layer cannot identify different QoS flows within a LCH and there is no clear mapping between CGs and QoS flows / QFIs.

Proposal 2: Subject to QoS mapping decisions in RAN2, if multiple QoS flows or PDU Sets of different importance are mapped to the same DRB and differentiation of traffic is considered unavoidable in lower layers, the MAC layer needs to identify data with different severity within a LCH.

Proposal 3: “Traffic flows not based on PDU Sets” should be characterized based on their contextual relation to other XR traffic flows and/or PDU Sets to be treated.

Proposal 4: “Traffic flows not based on PDU Sets” can be treated in two ways on a QoS flow, DRB or LCH:

- In traditional per-packet fashion (when its PDUs are independent of other XR traffic flows); or

- As PDU Set with “number of packets = 1” (when its PDUs are closely related to other XR traffic flows) e.g. to keep them in the framework for XR traffic.

# References

- RP-220285, Revised SID, Study on XR Enhancements for NR, Nokia, RAN#95e

- TR 23.700-60 v18.0.0, Study on XR (Extended Reality) and media services, Rel-18

- R2-2209455, Information to RAN for UE power savings, Qualcomm Incorporated, RAN2#119bis-e