3GPP TSG-RAN WG2 Meeting #121	R2-2300758

Athens, Greece, February 27 – March 3, 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR | 0014 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on handling of unforeseen SRAP Data PDU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-Core |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current subclause 5.4 only includes a few error cases where the UE ID or BEARER ID cannot be matched with the SRAP-config mapping table configured in either remote UE or relay UE. However, there are some additional unforeseen error cases, e.g., the remote UE and relay UE’s configurations may be out of sync when NW wants to add/modify SRAP configurations and/or PC5 RLC channels. This is because NW need to deliver those configurations in separate procedures to relay UE and remote UE respectively, which may not always happen at the same time. For example, gNB may send the RRCReconfiguration message adding a new PC5 RLC channel for a remote UE with updated SRAP configuration, but the corresponding PC5 RLC channel configuration or SRAP configuration for relay UE has not been timely delivered to the relay UE yet. In this case, the remote UE will start sending UL traffic with the new configuration, but the receiving relay UE may unable to identify the corresponding entry in its outdated SRAP mapping table because the “egress PC5 RLC channel” is not updated to include the newly added PC5 RLC channel. In this case, even the UE ID and BEARER ID in SRAP header may still match the entry in the old SRAP mapping table, the relay UE is unable to differentiate whether the traffic is for an SRB or a DRB. Relay UE will face a dilemma that it cannot determine the corresponding egress Uu RLC channel. Similarly, for a similar reason, the remote UE may also receive an unforeseen DL SRAP PDU that make it unable to determine the upper layer PDCP entity for the ingress SRAP PDU even when both UE ID and BEARER ID in SRAP header match an entry in SRAP mapping table. In principle, UE need discard those packets as same as other error cases described in 5.4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For remote UE, added “or any other unforeseen case(s) when the receiving upper layer entity cannot be uniquely identified according to 5.2.3“ as a new condition to discard SRAP PDU. - For relay UE, added “or any other unforeseen case(s) when the egress RLC channel cannot be uniquely determined according to 5.2.2.2 or 5.3.3.2“ as a new condition to discard SRAP PDU. Impact analysis Impacted functionality: Reception of SRAP PDU Inter-operability: - If an SL RX UE implements this change, but the gNB or SL TX UE has not implemented this change, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error handling for some unforseen SRAP packets will be left unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========================Begin of Change==== ========================

## 5.4	Handling of unknown, unforeseen, and erroneous protocol data

For U2N Remote UE, if sl-LocalIdentity and sl-RemoteUE-RB-Identity are both configured, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRemote is received, or any other unforeseen case(s) when the receiving upper layer entity cannot be uniquely identified according to 5.2.3, the SRAP entity shall:

-	discard the received SRAP Data PDU.

For U2N Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRelay is received except in the case where the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2IdentityRemote of the ingress link is received by U2N Relay UE, or any other unforeseen case(s) when the egress RLC channel cannot be uniquely determined according to 5.2.2.2 or 5.3.3.2, the SRAP entity shall:

-	discard the received SRAP Data PDU.

========================End of Changes============================