3GPP TSG RAN WG2#121	R2-2300764

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:		8.19.3

Source:		Apple Inc

Title:		Access and Indication topics of eRedCap

Document for:		Discussion and Decision

Work Item Code: 		NR_redcap_enh-Core

# Introduction

RAN2 will take up the eRedCap topic from RAN2-121 and in this paper we intend to bring up some basic topics starting with inter-operability and access that is needed for the operation of eRedCap devices in Rel-18 NWs.

# On the need of UE support

The WID for eRedCap provides some additional reduction in capability for the eRedCap devices. So for operation of such devices, both the indication of this extended reduced capability by the UE, and the indication by the NW that it supports such eRedCap UEs, need to be the first topic that needs resolution.

- Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified.

Observation 1: eRedCap UE needs to inform the NW about it’s further reduced capabilities when compared to R17 RedCap devices.

Observation 2: The NW also needs to know that it is dealing with a eRedCap UE, and while some might argue that an early indication might be enough for the NW, the eRedCap might be handed over to other cells, and possibly across RATs as well, and so an explicit UE capability is needed.

Proposal 1: A new UE capability field is defined for eRedCap and this is a per-UE capability.

# On the need of NW support

We can also flip the argument towards the UE perspective and can say that a eRedCap UE as part of cell selection/reselection cannot arbitrarily camp on a R17 RedCap supporting network, as this NW might not even have upgraded to Rel-18.

Observation 3 : eRedCap UEs cannot assume that a Rel-17 RedCap supporting NW can also support a eRedCap device. The NW might not have been upgraded at all and so might not honor eRedCap UE capabilities.

Proposal 2: NW broadcasts the support of eRedCap in the SIBs, and eRedCap UE only camps on the cells on which they are supported.

SIB1-v1700-IEs ::= SEQUENCE { hsdn-Cell-r17 ENUMERATED {true} OPTIONAL, -- Need R uac-BarringInfo-v1700 SEQUENCE { uac-BarringInfoSetList-v1700 UAC-BarringInfoSetList-v1700 } OPTIONAL, -- Cond MINT sdt-ConfigCommon-r17 SDT-ConfigCommonSIB-r17 OPTIONAL, -- Need R redCap-ConfigCommon-r17 RedCap-ConfigCommonSIB-r17 OPTIONAL, -- Need R ... nonCriticalExtension SIB1-v1800-IEs OPTIONAL } SIB1-v1800-IEs ::= SEQUENCE { eRedCapSupport-r18 ENUMERATED {allowed} OPTIONAL, -- Need R nonCriticalExtension SEQUENCE {} OPTIONAL }

## Cell support of eRedCap

Rel-17 RedCap support by the NW is indicated a bit indirectly in that the absence of the IE cellBarredRedCap-r17 implies that the NW does not support RedCap.

In our view, eRedCap is a subset of RedCap, and so we can assume that a NW supporting eRedCap devices is assumed to support R17 RedCap devices as well.

Observation 4: eRedCap is a subset of RedCap, and so we can assume that the a NW supporting eRedCap devices is assumed to support R17 RedCap devices as well.

Proposal 3: eRedCap supporting NW is assumed to support R17 RedCap feature.

## Cell barring aspects

In our view, before we discuss how the support/no support of eRedCap by the NW is to be implemented, we also need to discuss how the various access barring filters R17 RedCap are subjected to, are to be viewed by eRedCap.

Observation 5: R17 RedCap access to a R17 NW is filtered by the support of 1Rx/2Rx.

Observation 6: In an indirect way, the IE intraFreqReselectionRedCap-r17 also provides some level of cell access to the R17 RedCap device.

Observation 7: eRedCap is to be viewed to be a subset of RedCap UE in terms of it’s capabilities, but that subset is in the baseband processing (5MHz PD(U)SCH vs 20MHz for RedCap), and so the rest of the parameters are assumed to be not impacted.

Observation 8: It would be simpler that all the cell barring parameters and reselection parameters of R17 RedCap would be sufficient for eRedCap as well, instead of creating additional one for eRedCap.

Proposal 4: Same cell access parameters intraFreqReselectionRedCap-r17 and cellBarredRedCap-r17 used by R17 RedCap are applied to eRedCap UEs as well. No new parameters are defined and eRedCap UE uses these fields the same way as R17 RedCap UE.

Similar argument can be made of cell reselection. However, here one might argue that the R17 RedCap deployments might not have updated to eRedCap in all areas and so UE following the R17 RedCap reselection SIB broadcast info, might run into cell supporting only R17 RedCap.

Observation 9: Reusing the same RedCap specific Ncell reselection specific broadcast info for eRedCap might result in eRedCap UE finding out R17 RedCap cells that do not support eRedCap.

Observation 10: The operator might not deploy eRedCap in certain frequencies altogher, while still having R17 RedCap on these frequencies.

We think it’s better to have a SIB4 new inter-freq indication on whether this particular freq also support eRedCap.

Proposal 5: A new SIB4 inter-freq indication field is added to the support eRedCap with functionality similar to redCapAccessAllowed-r17. Absence of this new field implies the NW does not support eRedCap.

SIB1-v1700-IEs ::= SEQUENCE { hsdn-Cell-r17 ENUMERATED {true} OPTIONAL, -- Need R uac-BarringInfo-v1700 SEQUENCE { uac-BarringInfoSetList-v1700 UAC-BarringInfoSetList-v1700 } OPTIONAL, -- Cond MINT sdt-ConfigCommon-r17 SDT-ConfigCommonSIB-r17 OPTIONAL, -- Need R redCap-ConfigCommon-r17 RedCap-ConfigCommonSIB-r17 OPTIONAL, -- Need R featurePriorities-r17 SEQUENCE { redCapPriority-r17 FeaturePriority-r17 OPTIONAL, -- Need R slicingPriority-r17 FeaturePriority-r17 OPTIONAL, -- Need R msg3-Repetitions-Priority-r17 FeaturePriority-r17 OPTIONAL, -- Need R sdt-Priority-r17 FeaturePriority-r17 OPTIONAL -- Need R } OPTIONAL, -- Need R si-SchedulingInfo-v1700 SI-SchedulingInfo-v1700 OPTIONAL, -- Need R hyperSFN-r17 BIT STRING (SIZE(10)) OPTIONAL, -- Need R eDRX-AllowedIdle-r17 ENUMERATED {true} OPTIONAL, -- Need R eDRX-AllowedInactive-r17 ENUMERATED {true} OPTIONAL, -- Cond EDRX-RC intraFreqReselectionRedCap-r17 ENUMERATED {allowed, notAllowed} OPTIONAL, -- Need S cellBarredNTN-r17 ENUMERATED {barred, notBarred} OPTIONAL, -- Need S nonCriticalExtension SIB1-v1800-IEs OPTIONAL } SIB1-v1800-IEs ::= SEQUENCE { intraFreqReselectionRedCap-r18 ENUMERATED {allowed, notAllowed} OPTIONAL, -- Need S nonCriticalExtension SIB1-v1800-IEs OPTIONAL }

# Separate early indication

The discussions in RANP are expected to continue on whether early indication in MSG1/MSG3 are needed.

While the MSG1 discussion is pending in RANP, we think that RAN2 can discuss whether the MSG3 indication can have this eRedCap UE indication.

Since the UE capability transfer happens after the common/UE dedicated channels are established and used, and the NW has to be defensive till that retrieval of UE capability, to further restrict the NW with 5MHz might not be efficient.

In addition, the same argument made of R17 RedCap can be made in here: the indication using LCID does not bring in additional size increase in MSG3.

Observation 11: R17 RedCap already indicates MSG3 indication, and using another set of LC-ID values do not increase the size of MSG3.

Observation 12: It is helpful to the NW to know about eRedCap as early as possible. Also if the RANP discussion does not result in MSG1 based indication, MSG3 based would be very helpful to both the UE and the NW

Observation 13: Since the indication of a capability without protection is something that RAN2 was ok with in Rel-17 RedCap case, there should not be any additional issue with MSG3 indication for eRedCap.

Proposal 6: eRedCap UE can indicate it’s support in MSG3 using LC-ID based identification.

# Conclusions

Observation 1: eRedCap UE needs to inform the NW about it’s further reduced capabilities when compared to R17 RedCap devices.

Observation 2: The NW also needs to know that it is dealing with a eRedCap UE, and while some might argue that an early indication might be enough for the NW, the eRedCap might be handed over to other cells, and possibly across RATs as well, and so an explicit UE capability is needed.

Observation 3 : eRedCap UEs cannot assume that a Rel-17 RedCap supporting NW can also support a eRedCap device. The NW might not have been upgraded at all and so might not honor eRedCap UE capabilities.

Observation 4: eRedCap is a subset of RedCap, and so we can assume that the a NW supporting eRedCap devices is assumed to support R17 RedCap devices as well.

Observation 5: R17 RedCap access to a R17 NW is filtered by the support of 1Rx/2Rx.

Observation 6: In an indirect way, the IE intraFreqReselectionRedCap-r17 also provides some level of cell access to the R17 RedCap device.

Observation 7: eRedCap is to be viewed to be a subset of RedCap UE in terms of it’s capabilities, but that subset is in the baseband processing (5MHz PD(U)SCH vs 20MHz for RedCap), and so the rest of the parameters are assumed to be not impacted.

Observation 8: It would be simpler that all the cell barring parameters and reselection parameters of R17 RedCap would be sufficient for eRedCap as well, instead of creating additional one for eRedCap.

Observation 9: Reusing the same RedCap specific Ncell reselection specific broadcast info for eRedCap might result in eRedCap UE finding out R17 RedCap cells that do not support eRedCap.

Observation 10: The operator might not deploy eRedCap in certain frequencies altogher, while still having R17 RedCap on these frequencies.

Observation 11: R17 RedCap already indicates MSG3 indication, and using another set of LC-ID values do not increase the size of MSG3.

Observation 12: It is helpful to the NW to know about eRedCap as early as possible. Also if the RANP discussion does not result in MSG1 based indication, MSG3 based would be very helpful to both the UE and the NW

Observation 13: Since the indication of a capability without protection is something that RAN2 was ok with in Rel-17 RedCap case, there should not be any additional issue with MSG3 indication for eRedCap.

Proposal 1: A new UE capability field is defined for eRedCap and this is a per-UE capability.

Proposal 2: NW broadcasts the support of eRedCap in the SIBs, and eRedCap UE only camps on the cells on which they are supported.

Proposal 3: eRedCap supporting NW is assumed to support R17 RedCap feature.

Proposal 4: Same cell access parameters intraFreqReselectionRedCap-r17 and cellBarredRedCap-r17 used by R17 RedCap are applied to eRedCap UEs as well. No new parameters are defined and eRedCap UE uses these fields the same way as R17 RedCap UE.

Proposal 5: A new SIB4 inter-freq indication field is added to the support eRedCap with functionality similar to redCapAccessAllowed-r17. Absence of this new field implies the NW does not support eRedCap.

Proposal 6: eRedCap UE can indicate it’s support in MSG3 using LC-ID based identification.

# References