3GPP RAN2 Meeting #121		R2-2300769

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	8.21.1

Source:	Apple

Title:	Discussion on the extention of NS values

WI/SI:	NR_unlic_enh

Release:	Rel-18

Document for:	Decision

# 1	Introduction

On the topic of extending the range of NS values RAN2 has replied with some follow-up questions [6] to the RAN4 LS [5], and RAN4 has replied back [7] with the below inputs.

-	All existing NR bands (both licensed and unlicensed) have less than 7 different NS values. Thus, it is possible to label the highest value "7" as RESERVED for the purpose of introduction of the extended NS range.

-	Since the first unlicensed NR bands were introduced in Rel-16, it would be preferrable to introduce the extended NS range starting from Rel-16. Otherwise Rel-16 UEs implementing the NR-U functionality will not be able to support regulatory requirements from some countries.

-	To limit potential impact to legacy device and licensed bands, it is possible to consider a clarification that the extended NS range is applicable only to the unlicensed bands for earlier releases.

-	For now, RAN WG4 has not identified a need to have the extended NS range for the licensed bands. For the sake of future flexibility RAN WG4 would like to keep the possibility open to enable the extension IE to licensed bands in a later release.

In this discussion paper we take the input from RAN4 LS and provide proposals on implementing these requirements from RAN4.

# 2	Extended range of NS values

## 2.1	3GPP NR Release related aspects

In the original LS sent to RAN2 [5], RAN4 asked whether NS values can be extended starting from earlier releases. The main rationale for that question was that while this WI is for Rel-18, band n96 was added back in Rel-16 and the extended NS range should be ideally applied starting from the release when the corresponding band was added. Otherwise, a device implementing Rel-16 band n96 will not be able to receive and act upon extended NS values defined in later releases. The same argument applies to other unlicensed bands such as n102. RAN2  responded [6] that it is possible to make NS range extension from earlier releases. Furthermore, RAN4 reiterated [7] the point that the corresponding extension of the NS values is preferrable from Rel-16.

Observation 1: RAN4 prefers that the extension be done from Rel-16 itself

Proposal 1: RAN2 to introduce the CRs from Rel-16 at least for unlicensed bands.

## 2.3	Interoperability between releases

One of the technical issues related to enabling the NS extension in the earlier releases is how to deal with a potential scenario when there is a Rel-16 UE that may or may not implement new NS flags defined in Rel-17 and Rel-18. On the one hand, if the extended NS range is enabled starting from Rel-16, then there is nothing that prevents a Rel-16 UE to process the extended NS flags and act upon them. On the other hand, there could be Rel-16 devices implementing NR-U, which by the time when they are released to the market do not fully support all NS flags defined in e.g. Rel-18 or even later releases. The situation could be even more tangled – a Rel-16 NR-U device may support Rel-17 NS flags, but not Rel-18 ones. Table 2.2-1 below summarises which NS flags have been added in Rel-16 and Rel-17, and which new flags are being considered for Rel-18. Since the Rel-18 specification work is not over, there can be a Rel-16 NR-U devices that can support Rel-16 and Rel-17 flags, but not Rel-18 ones. Otherwise, if the network does not know which NS flags are supported by the UE, then a UE implementing e.g. Rel-17 band n102, but not the Rel-18 flags, will reject the RRC message configuring the secondary cell on the shared access spectrum band.

Observation 2: Introduction of this feature requires that the NW be aware of the UE support of these extended NS values.

Observation 3: The knowledge of the UE support of the extended values need to be based on each band the UE supports.

Proposal 2: A Per-band UE capability is needed to reflect the support of the extended NS value range.

Proposal 3: The capability is needed from Rel-16.

Table 2.2-1: Summary of the shared access spectrum bands with associated NS flags.

| Band | Rel-16 | Rel-17 | Rel-18 |
| --- | --- | --- | --- |
| n96 | NS_53, NS_54 | NS_59, NS_60, NS_61 | NS_y3, NS_y4 |
| n102 |  | NS_58 | NS_y1, NS_y2, NS_y5 |

### 2.3.1	UE capability and reuse of modifiedMPR-Behaviour

As can be seen above, we need a per-Band UE capability so that the NW is aware which NS values the UE supports for which band.

The most straightforward approach to enable the corresponding signalling is to consider modifiedMPR-Behaviour field that can indicate which NS flags are supported. In fact, some bands already use this field to indicate which NS flags are supported by the UE.  And this is already a per-Band UE capability.

Observation 4:  The existing modifiedMPR-Behaviour field already provides the UE’s NS value support upto 7 entries, and this is per-band.

Observation 5: Extending modifiedMPR-Behaviour field to allow the UE to signal the support of NS values  for greater than 7, where the RAN4 table can have entries upto 255 can solve the RAN4 requirement while still avoiding the release inter-operability.

Proposal 4: The new UE capability field covers the support of NS value entries from 8 to 255 into the RAN4 NS value table.

### 2.3.2	Using extended NS in SIB broadcast

We should also discuss the case of the gNB broadcasting the extended NS values for some bands in the SIB. The current UE behaviour is that the UE would not camp on the cell if it does not support atleast one NS value. In case the NW intends to use the extended NS value range, then the NW would have to use the value ‘7’ (as reserved) to indicate to the UE that extended NS values are being used. But legacy UEs might not understand this and might bar the cell.

So it is assumed that NWs which intend to use the extended values need to also signal the legacy NS values to allow legacy UEs to camp on the cell. But it can also be intentional for the NW to only allow UEs that support extended NS values to camp on this cell.

Observation 6: If extended NS values are to be used in cell broadcast (in SIB1/2/4), then not broadcasting any legacy NS values can result in legacy UEs not being to camp on the cell.

Observation 7: Whether to broadcast legacy NS values along with the extended NS values is upto the gNB.

Proposal 5: It is upto gNB to signal legacy NS values in broadcast when the extended NS values are being used. RAN2 assumes that legacy UE might not be able to camp on the cell if only extended NS values are broadcasted, but RAN2 understands that this is NW implementation.

## 2.4	Applicability to licensed bands

RAN4 LS reply does indicate that the Rel-16 extended NS value feature can be limited to unlicensed bands, while for Rel-17 or later, RAN4 has not precluded this for licensed bands.

While we can capture in the specification that Rel-16 extension is limited to unlicensed operation, from the perspective of UE reporting this as a per-band capability, we see no reason to capture this explicitly. In other words, as long as RAN4 does not define new NS values for licensed bands in Rel-16, the UEs would not implement these (and report the capability) and the NW would not configure these. Since the reporting is anyway per-band, not capturing anything in the specification can allow for the possibility of introducing this operation for licensed bands in Rel-16 without having to worry about backward compatibility.

Observation 8: Per-Band UE capability already provides the UE capability support in terms each band the UE supports (including licensed and unlicensed bands)

Observation 9: As long as RAN4 does not define new NS values for licensed bands in Rel-16, the UEs would not implement these (and report the capability) and the NW would not configure these.

Observation 10: Not capturing any restriction in the specification (on the applicability of extended NS values to only unlicensed bands in Rel-16) can allow for the possibility of introducing this operation for licensed bands in Rel-16 without having to worry about backward compatibility.

Proposal 6: RAN2 does not add any restriction in the field description on the applicability of extended NS values to only unlicensed bands in Rel-16. RAN2 assumes that NWs follow the UE capability and the specification and do not configure the extended NS values that are not supported by the UEs or not supported by the specification in the associated bands.

We have provided draft CRs in [1][2] capturing the above.

Proposal 7: RAN2 to agree the CRs from R2-23x, R2-23x

# 3	Conclusions

Observation 1: RAN4 prefers that the extension be done from Rel-16 itself

Observation 2: Introduction of this feature requires that the NW be aware of the UE support of these extended NS values.

Observation 3: The knowledge of the UE support of the extended values need to be based on each band the UE supports.

Observation 4:  The existing modifiedMPR-Behaviour field already provides the UE’s NS value support upto 7 entries, and this is per-band.

Observation 5: Extending modifiedMPR-Behaviour field to allow the UE to signal the support of NS values  for greater than 7, where the RAN4 table can have entries upto 255 can solve the RAN4 requirement while still avoiding the release inter-operability.

Observation 6: If extended NS values are to be used in cell broadcast (in SIB1/2/4), then not broadcasting any legacy NS values can result in legacy UEs not being to camp on the cell.

Observation 7: Whether to broadcast legacy NS values along with the extended NS values is upto the gNB.

Observation 8: Per-Band UE capability already provides the UE capability support in terms each band the UE supports (including licensed and unlicensed bands)

Observation 9: As long as RAN4 does not define new NS values for licensed bands in Rel-16, the UEs would not implement these (and report the capability) and the NW would not configure these.

Observation 10: Not capturing any restriction in the specification (on the applicability of extended NS values to only unlicensed bands in Rel-16) can allow for the possibility of introducing this operation for licensed bands in Rel-16 without having to worry about backward compatibility.

Proposal 1: RAN2 to introduce the CRs from Rel-16 at least for unlicensed bands.

Proposal 2: A Per-band UE capability is needed to reflect the support of the extended NS value range.

Proposal 3: The capability is needed from Rel-16.

Proposal 4: The new UE capability field covers the support of NS value entries from 8 to 255 into the RAN4 NS value table.

Proposal 5: It is upto gNB to signal legacy NS values in broadcast when the extended NS values are being used. RAN2 assumes that legacy UE might not be able to camp on the cell if only extended NS values are broadcasted, but RAN2 understands that this is NW implementation.

Proposal 6: RAN2 does not add any restriction in the field description on the applicability of extended NS values to only unlicensed bands in Rel-16. RAN2 assumes that NWs follow the UE capability and the specification and do not configure the extended NS values that are not supported by the UEs or not supported by the specification in the associated bands.

Proposal 7: RAN2 to agree the CRs from R2-23x, R2-23x

# 4	References

- RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

- RP-213180, "Revised WID for Introduction of operation in full unlicensed band 5925-7125MHz", Apple Inc.

- RP-221813, "New WID: Introduction of evolved shared spectrum bands", Apple Inc.

- R4-2212338, "Overview of the regulatory parameters and potential NS values", Apple Inc.

- R4-2214953, "LS on extending the maximum range for NS values from RAN WG4", RAN WG4

- R2-2211064, "Response to LS on extending the maximum range for NS values", RAN WG2

- R4-2220493, "Response LS on extending the maximum range for NS values", RAN WG4