3GPP TSG-RAN2#121	R2-2300777

Athens, 27th Feb – 3rd Mar, 2023

Agenda item:		8.18.2 (NR_MT_SDT-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on MT-SDT procedure

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 made agreements for MT-SDT as follows:

Agreements

- For RAN paging, MT-SDT indication (at least one bit) is explicitly included per UE via a paging message.  FFS if more information for MT-SDT are needed FFS what the indication will be called.  FFS signalling details

- Rel-18 MT-SDT after the MT-SDT paging trigger is detected, RA-SDT and CG SDT solutions/procedures specified in Rel-17 is re-used as a baseline.  The detailed triggers will be discussed on case by case.  FFS on resources used for access

- UE can use non-SDT random access resources for accessing the network for an MT-SDT transfer.  The UE can also use the configured grant resources and/or MO-RA resources.

- The network should be able to differentiate why the UL access was triggered, i.e. implicit or explicit indication by the UE.

- MT-SDT is data that belongs to bearers that are configured for SDT.    FFS whether the configuration is MO-SDT or MT-SDT specific.  The network can only trigger MT-SDT if the data belongs to those bearers.

- It is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT.  Subsequent UL/DL data belonging to SDT bearers while in INACTIVE is allowed like MO-SDT procedure.  FFS stage 3 details

- New Resume cause in RRC resume will be introduced, one code point MT-SDT indication

In this contribution, we present our view on MT-SDT based on the above agreements.

2.	Discussion

In the last meeting, RAN2 made some agreement for MT-SDT. According to the agreements and the current SDT procedure, Rel-18 MT-SDT procedure can be depicted as follows. Based on Figure 1, we present our understanding on MT-SDT procedure and have further discussion for each step of MT-SDT procedure.

Figure 1. Rel-18 MT-SDT procedure and Agreements in RAN2#120

In Step 1, the network provides the SDT configuration via SIB or RRCRelease message as in Rel-17 SDT.

Regarding signalling for SDT configuration, we think there may be no objection that this will be same as Rel-18 MT-SDT configuration.

In Step 2, if data belonging to RBs configured for MT-SDT is available in the network side, the network triggers MT-SDT procedure.

For the configuration for SDT RBs and MT-SDT triggering, RAN2 agreed the definition of MT-SDT data and the MT-SDT triggering condition in the network side, but whether the configuration is MO-SDT or MT-SDT specific is remained as FFS.

In our view, there is no reason to configure MO-SDT/MT-SDT specific RBs separately. If RBs are allowed to be configured as MO-SDT or MT-SDT specific, data belonging to RBs configured for MO-SDT cannot be transmitted during MT-SDT procedure. For transmission of the MO-SDT data, the UE may need to transition to RRC_CONNECTED, or the UE may need to wait for the termination of MT-SDT procedure and then initiate MO-SDT procedure. In latency and power saving point of view, this is not desirable. Therefore, data belonging to RBs configured for SDT should be able to be transmitted within SDT procedure regardless of MO-SDT or MT-SDT, i.e., RBs are not configured as MO-SDT or MT-SDT specific.

Proposal 1. RBs are not configured as MO-SDT or MT-SDT specific.

In Step 3, upon triggering MT-SDT procedure in the network side, the network sends the paging message to a UE.

Regarding the paging message, RAN2 agreed to include MT-SDT indication (at least one bit) explicitly in a paging message for MT-SDT and to discuss further whether more information for MT-SDT is needed in the paging message.

In the current specification, the paging message transmitted for a certain UE includes a UE ID for specifying the UE, so the UE ID will naturally be included. In addition to the UE ID, we discuss which information needs to be included in below.

In Rel-17 SDT, two type of resources are used for UL transmission, i.e., dynamic grant (DG) and pre-configured resource (CG). DG is a considered as a default, and CG is introduced because it is an effective method to reduce signalling overhead. With similar reason, we think that dynamic DL assignment and SPS would be used for Rel-18 MT-SDT. After initiating SDT procedure for MT-SDT, the UE basically receives DL data according to dynamic DL assignment, and if SPS is configured, the UE receives DL data using the configured SPS without L1 signalling for dynamic DL assignment. Therefore, supporting SPS along with dynamic DL assignment is beneficial in terms of signalling overhead.

Proposal 2. Both dynamic DL assignment and SPS are supported for Rel-18 MT-SDT.

Based on Proposal 2, the paging message for MT-SDT needs to include the information whether SPS resource is used for the reception of DL data. According to the current specification, upon Rel-17 SDT procedure is initiated, the UE monitors PDCCH continuously and dynamic UL grant can be always scheduled. In this principle, if MT-SDT is initiated, the UE monitors PDCCH continuously and dynamic DL assignment can be also always scheduled. However, it is not desirable for the UE to always decode the configured SPS. In the network side, there may be some case where the configured SPS is not available, and in this case, the network uses dynamic DL assignment during SDT procedure for MT-SDT. Thus, the UE does not need to decode the configured SPS, but the UE does not have knowledge whether the configured SPS is available or not. Therefore, the network needs to indicate whether the configured SPS is used for MT-SDT in the paging message and the UE decodes the configured SPS based on the paging message.

Proposal 3. The paging message includes an indication indicating whether to use SPS for MT-SDT or not.

In Step 4, upon the reception of the paging message, the UE initiates SDT procedure and selects UL resource to transmit RRCResumeRequest message in response to the paging message.

Regarding UL resource selection, RAN2 agreed that CG-SDT/RA-SDT procedure in Rel-17 is reused as a baseline and UE can use non-SDT RA resources for accessing the network for an MT-SDT transfer. Based on the agreements, the detailed procedure would be inherited from Rel-17 as follows.

- if CG-SDT criteria is met, CG-SDT resource is chosen.

- Else if RA-SDT criteria is met, RA-SDT resource is chosen.

- Otherwise, non-SDT RA resource is chosen.

One difference is that when non-SDT RA resource is chosen, RRCResume procedure is initiated for non-SDT procedure in Rel-17, but in Rel-18, RRCResume procedure is initiated for MT-SDT procedure. Thus, upon receiving RRCResumeRequest message on non-SDT RA resource, the network transitions the UE to RRC_CONNECTED in Rel-17 but transmits MT-SDT data to the UE in RRC_INACTIVE in Rel-18 if RRCResume cause is set to the new RRCResume cause discussed next.

Proposal 4. RAN2 confirm that UL resource is selected according to following procedure, and RRCResume procedure is initiated for MT-SDT procedure.

- if CG-SDT criteria is met, CG-SDT resource is chosen.

- Else if RA-SDT criteria is met, RA-SDT resource is chosen.

- Otherwise, non-SDT RA resource is chosen.

In Step 5, the UE transmits RRCResumeRequest message as an uplink response of the paging message by using the selected UL resource.

Regarding RRCResumeRequest message, RAN2 agreed that the network should be able to differentiate why the UL access was triggered and a new resume cause (e.g. mt-SDT) in RRCResumeRequest message will be introduced. The UE sets RRCResume cause and selects UL resource depending on the purpose of the transmission of RRCResumeRequest. Thus, the network can know why RRCResumeRequest is transmitted and prepare the data transmission/reception.

- For MO-SDT data, RRCResume cause = mo-Data, UL resource = CG-SDT/RA-SDT resource

- For non-SDT UL data, RRCResume cause = mo-Data, UL resource = non-SDT RA resource.

- For MT-SDT data, RRCResume cause = mt-SDT, UL resource = CG-SDT/RA-SDT/non-RA resource

- For non-SDT DL data, RRCResume cause = mt-Access, UL resource = non-RA resource

However, there may be a case where MO-SDT data or non-SDT UL data is available after initiating SDT procedure and before transmitting RRCResumeRequest message. In this case, RRCResume procedure is initiated twice, i.e. for MT-SDT and for MO-SDT/non-SDT UL. In the current specification, how to set RRCResume cause and how to select UL resource are not specified for the case where RRCResume procedure is initiated twice. Thus, RAN2 should discuss how to set RRCResume cause and how to select UL resource, and if RRCResume casue and UL resource are not enough to cover all case, RAN2 needs to consider to introduce a new indication.

Figure 2. Example of UL data occurrence during MT-SDT procedure

Proposal 5. RAN2 discuss how to set RRCResume cause and how to select UL resource, for the case where MO-SDT data or non-SDT UL data is available after initiating SDT procedure and before transmitting RRCResumeRequest message.

Before transmitting RRCResumeRequest message, the UE receives the paging message, decodes it, initiates SDT procedure, and selects UL resource and generates the uplink response. Depending on UE processing capability, the UE may transmit the uplink response a long time after receiving the paging message.

We think there are two problems due to the long time.

First problem is that the uplink response may not be transmitted within the network’s paging response monitoring window. Processing time is UE’s internal behaviour, so it is difficult to predict time between the paging message and the uplink response, in the network side. If the uplink response is transmitted after the network’s paging response monitoring window, the network may not receive the uplink response. Then, the network has to transmit the paging message to larger area, or, in the extreme case, the network cannot receive the uplink response at all. Then, MT-SDT data transmission would fail.

Figure 3. Example of failure of uplink response

Thus, it would be beneficial to transmit a positive HARQ feedback to the network upon receiving the paging message. After transmitting the positive HARQ feedback, the UE initiates SDT procedure to transmit the uplink response, selects UL resource and generate the uplink response message, and after receiving the positive HARQ feedback, the network waits the uplink response message.

Proposal 6. UE transmits a positive HARQ feedback to the network upon receiving the paging message for MT-SDT.

Second problem is that CG-SDT-TAT may be expired unexpectedly. If CG-SDT-TAT is expired while SDT procedure is ongoing, the UE considers that the ongoing CG-SDT procedure is terminated, and goes to RRC_IDLE. Once the UE moves to RRC_IDLE, the UE cannot use SDT procedure anymore until the UE goes to RRC_CONNECTED. Thus, CG-SDT-TAT should be managed during MT-SDT procedure.

Currently, TAT or CG-SDT-TAT is restarted upon receiving DL signalling, i.e., TAC MAC CE, RAR or RRCRelease, so it would be desirable to restart CG-SDT-TAT based on DL signalling in MT-SDT procedure as well, and the DL signalling can be the paging message. Upon receiving the paging message, the UE restarts CG-SDT-TAT and considers CG-SDT resource ass valid if CG-SDT-TAT is running.

Proposal 7. CG-SDT-TAT is restarted upon receiving the paging message for MT-SDT.

In Step 6, if RRCResumeRequest is successfully transmitted, the network transmits MT-SDT data to the UE. If there is MO-SDT data, the UE transmits it using CG-SDT or dynamic UL grant.

In Step 7, when the network determines there is no data to be transmitted in both the UE and the network, RRCRelease is transmitted for terminating SDT procedure.

3.	Conclusion

In this document, we present our understanding on overall MT-SDT procedure, and make proposals as follows.

Proposal 1. RBs are not configured as MO-SDT or MT-SDT specific.

Proposal 2. Both dynamic DL assignment and SPS are supported for Rel-18 MT-SDT.

Proposal 3. The paging message includes an indication indicating whether to use SPS for MT-SDT or not.

Proposal 4. RAN2 confirm that UL resource is selected according to following procedure, and RRCResume procedure is initiated for MT-SDT procedure.

- if CG-SDT criteria is met, CG-SDT resource is chosen.

- Else if RA-SDT criteria is met, RA-SDT resource is chosen.

- Otherwise, non-SDT RA resource is chosen.

Proposal 5. RAN2 discuss how to set RRCResume cause and how to select UL resource, for the case where MO-SDT data or non-SDT UL data is available after initiating SDT procedure and before transmitting RRCResumeRequest message.

Proposal 6. UE transmits a positive HARQ feedback to the network upon receiving the paging message for MT-SDT.

Proposal 7. CG-SDT-TAT is restarted upon receiving the paging message for MT-SDT.

4.	Reference

- RP-213583	New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR	ZTE Corporation