3GPP TSG-RAN WG2 Meeting #121	 R2-2300791

# Athens, Greece, 27 February - 3 March, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3837 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on TRS availability handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the TRS L1 availability indication, the corresponding validity duration is configured in SIB17, of which the time unit is one default paging cycle. Thus even though the SIB17 does not change, the validity duration would also change in case the default paging cycle changes. However, based on the current spec, it is not clear how to handle the L1 availability indication if the default paging cycle changes during the previously applied validity duration. Similar to the solution for the case of SIB17 updating, for avoiding unnecessary maintainance of old configuration and possible misalignment between the UE and the network, the UE should consider the TRS(s) as unavailable until it receives a new L1 indication(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in SIB17 that if the default paging cycle has changed, the UE considers its configured TRS(s) as unavailable until it receives the associated L1-based availability indication(s). Impact analysis Impacted 5G architecture options: NR standalone Impacted functionality: TRS for Idle and Inactive Inter-operability: If the UE is implemented according to this CR while the network is not, there is no interoperability issue. If the network is implemented according to this CR while the UE is not, the TRS(s) that the UE considers to be available may be unavailable, which may impact the UE’s operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The handling of the L1 availability indication when the default paging cycle changes is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 6.3.1	System information blocks

#### –	SIB17

SIB17 contains configurations of TRS resources for idle/inactive UEs.

SIB17 information element

-- ASN1START

-- TAG-SIB17-START

SIB17-r17 ::=               SEQUENCE {

segmentNumber-r17           INTEGER (0..63),

segmentType-r17             ENUMERATED {notLastSegment, lastSegment},

segmentContainer-r17        OCTET STRING

}

SIB17-IEs-r17 ::=           SEQUENCE {

trs-ResourceSetConfig-r17   SEQUENCE (SIZE (1..maxNrofTRS-ResourceSets-r17)) OF TRS-ResourceSet-r17,

validityDuration-r17        ENUMERATED {t1, t2, t4, t8, t16, t32, t64, t128, t256, t512, infinity, spare5, spare4, spare3, spare2,

spare1}                                                            OPTIONAL,  -- Need S

lateNonCriticalExtension    OCTET STRING                                                                   OPTIONAL,

...

}

TRS-ResourceSet-r17 ::=                SEQUENCE {

powerControlOffsetSS-r17               ENUMERATED {db-3, db0, db3, db6},

scramblingID-Info-r17                  CHOICE {

scramblingIDforCommon-r17              ScramblingId,

scramblingIDperResourceListWith2-r17   SEQUENCE (SIZE (2)) OF ScramblingId,

scramblingIDperResourceListWith4-r17   SEQUENCE (SIZE (4)) OF ScramblingId,

...

},

firstOFDMSymbolInTimeDomain-r17            INTEGER (0..9),

startingRB-r17                             INTEGER (0..maxNrofPhysicalResourceBlocks-1),

nrofRBs-r17                                INTEGER (24..maxNrofPhysicalResourceBlocksPlus1),

ssb-Index-r17                              SSB-Index,

periodicityAndOffset-r17                   CHOICE {

slots10                                    INTEGER (0..9),

slots20                                    INTEGER (0..19),

slots40                                    INTEGER (0..39),

slots80                                    INTEGER (0..79)

},

frequencyDomainAllocation-r17              BIT STRING (SIZE (4)),

indBitID-r17                               INTEGER (0..5),

nrofResources-r17                          ENUMERATED {n2, n4}

}

-- TAG-SIB17-STOP

-- ASN1STOP

SIB17 field descriptions

segmentContainer This field includes a segment of the encoded SIB17-IEs. The size of the included segment in this container should be small enough that the SIB message size is less than or equal to the maximum size of a NR SI, i.e. 2976 bits when SIB17 is broadcast.

segmentNumber This field identifies the sequence number of a segment of SIB17-IEs. A segment number of zero corresponds to the first segment, a segment number of one corresponds to the second segment, and so on.

segmentType This field indicates whether the included segment is the last segment or not.

trs-ResourceSetConfig RS configuration of TRS occasion(s) for idle/inactive UE(s), in terms of a list of N>=1 NZP TRS resource set(s). The maximum number of TRS resource sets configured by higher layer is 64. If a TRS resource is configured, the L1 based availability indication is always enabled based on that configuration. A UE which acquired SIB17 with a TRS configuration but did not yet receive an associated L1-based availability indication considers the configured TRS as unavailable. If SIB scheduling indicates that SIB17 has changed or if the default paging cycle has changed, the UE considers its configured TRS(s) as unavailable until it receives the associated L1-based availability indication(s).

validityDuration The valid time duration for L1 availability indication, time unit is one default paging cycle. When the field is absent, UE assumes a default time duration to be 2 default paging cycles. The field is only valid while the UE has a valid SIB17.

TRS-ResourceSet field descriptions

firstOFDMSymbolInTimeDomain The index of the first OFDM symbol in the PRB used for TRS in a slot. The field indicates the first symbol in a slot for the first TRS resource within the slot, and the symbol for the second TRS resource in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4.

frequencyDomainAllocation Indicates the offset of the first RE to RE#0 in a RB in row1.

indBitID The index of the associated bit in TRS availability indication field in DCI. Each TRS resource set is configured with an ID i for the association with (i+1)-th indication bit in TRS availability indication field in DCI.

nrofRBs Number of PRBs across which corresponding TRS resource spans.

nrofResources The number of TRS resources for a TRS resource set.

periodicityAndOffset The periodicity and slot offset (slot) for periodic TRS. It is used to determine the location of the first slot of TRS resource set. The periodicity value slots10 corresponds to 10 slots, value slots20 corresponds to 20 slots, and so on.

powerControlOffsetSS Power offset (dB) of NZP CSI-RS RE to SSS RE.

scramblingID-Info One or more scrambling IDs are configured for a TRS resource set. If a common scrambling ID is configured, it applies to all the TRS resources within the TRS resource set. Otherwise, each TRS resource within the TRS resource set is provided with a scrambling ID. If the number of TRS resources for the TRS resource set is 2, scramblingIDperResourceListWith2-r17 is configured, while scramblingIDperResourceListWith4-r17 is configured for the case that the number of TRS resources for the TRS resource set is 4.

ssb-Index The index of reference SSB with which quasi-collocation information is provided as specified in TS 38.214 [19] clause 5.1.5.

startingRB The PRB index where corresponding TRS resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid.

END OF CHANGE