3GPP TSG-RAN WG2 Meeting #121	R2-2300799

Athens, Greece, Febrary 27 – March 3, 2023

Agenda Item		:	8.7.4.1.2

Source		:	LG Electronics Inc.

Title		:	Discussion on NTN-NTN cell reselection enhancements

Document for		:	Discussion and Decision

1. Introduction

In RAN2#119bis-e meeting, following statements are agreed.

RAN2#119bis-e agreement: 1. For NTN-NTN cell reselection with earth moving cell, RAN2 will consider providing parameters of serving cell to UE, for UE to estimate when the serving cell stops providing coverage at the present UE location (FFS whether this will be an optional UE feature) (this does not exclude any time-based or location-based approach) (other solutions can also be considered) 2. In Earth-moving cell, the reference location and distance threshold of serving cell are provided by network for UE to estimate when the serving cell stops providing coverage at the present UE location. FFS how the reference location and/or distance threshold are provided to the UE

In this contribution, we provide our views on cell reselection enhancements for earth moving cell and quasi-earth fixed cell.

2. Discussion

2-1. Reference location of earth moving cell

During RAN2#119bis-e, companies provided their views on how to provide reference location of serving cell in earth moving scenario, however, cannot be reached to agreement. Following options are proposed to consider as a candidate solutions by rapporteur of email discussion [117] [1].

Opt-1: Only location coordinates of current cell center, and network is supposed to update the values every time when it is provided for Earth-moving cell. Opt-2: Multiple reference locations and its time information of Earth-moving cell. Opt-3: Based on the sub-satellite point derived by satellite ephemeris and the broadcasted location offset between sub-satellite point and the cell reference location Opt-4: Cell type (fixed or moving), reference location coordinates with a time stamp, and the velocity of reference location Opt-5: Cell type, reference location corresponding to the epochTime (reuse the existing epochTime).

The solutions Opt-1, 3 and 5 adopt a single reference location at a certain time point. Three options have common understanding that UE can calculate RST with reference location at the past. However, these options does not consider real-time reference location that is essential for applying distance-based mobility introduced in Rel-17. To support distance-based conditional mobility, these options force frequent SIB acquisition causing excessive energy drain to UE in RRC_IDLE/RRC_INACTIVE. Although the solutions utilize ephemeris for deriving a velocity of reference location, it is impossible to derive the velocity of reference location when the ephemeris is provided by PVT format. Therefore, the solutions Opt-1, 3, and 5 are only for calculating RST but not suitable for supporting distance-based mobility.

Observation 1	Solutions to provide a single reference location at certain time point cannot support distance-based mobility introduced in Rel-17 NTN although they are sufficient to calculate RST of serving cell.

On the other hand, by utilizing Opt-2 and 4, UE can calculate real-time reference location with trajectory of reference location. Especially, with Opt-2, UE can obtain approximated real-time reference location by interpolation of adjacent reference locations. Considering distance-based mobility in NTN, it is more beneficial to adopt Opt-2 for providing reference location of serving cell in earth moving scenario.

Additionally, distance threshold representing edge of cell coverage should be provided to calculate RST of serving cell.

Proposal 1	In earth moving scenario, serving cell provides multiple reference locations and its time information.

Proposal 2	Distance threshold representing, e.g., edge of cell coverage should be provided to calculate RST of serving cell.

2-2. Distance-based cell reselection enhancement

During the discussion in Rel-17 NTN, companies had diverse views on introducing the distance-based cell reselection and failed to reach an agreement. Since radio quality of the NTN cell has consistent value throughout the coverage, UE should consider distance to reference location of neighbor cell. The distance-based cell reselection enhancement aims to discriminate a neighbor cell that UE locates at the edge of the cell coverage. In other words, UE should treat the neighbor cell as barred if distance to reference location of neighbor cell is longer than distance threshold,. Accordingly, we propose distance-based cell reselection procedure as follows:

Step-1. UE calculates a distance to neighbor cells.

Step-2. UE treats a neighbor cell as barred if the distance to its reference location is longer than distance threshold.

Step-3. UE performs R-value-based ranking evaluation to neighbor cells.

Step-4. UE performs reselection to the highest ranked cell.

Proposal 3	Introduce distance-based cell reselection, i.e., UE should consider distance between UE location and reference location of neighbor cell to prevent UE from performing cell reselection to a neighbor cell that UE locates at the edge of the cell coverage.

Proposal 4	Neighbor cell that UE locates at the edge of the cell coverage should be excluded from R-value based ranking evaluation.
Step-1. UE calculates a distance to neighbor cells.
Step-2. UE treat a neighbor cell as barred if the distance to its reference location is longer than 
             distance threshold.
Step-3. Perform R-value-based ranking evaluation to neighbor cells.
Step-4. Perform cell reselection to the highest ranked cell.

2-3. Time-based cell reselection enhancement

When the stationary UE camps on the TN cell, the UE hardly performs cell reselection. In contrast, the stationary UE camping on the NTN cell has to perform cell reselection due to the movement of the NTN cell. Furthermore, the service time of the NTN cell varies depending on the satellite's velocity and the beam steering scheme. Such property of the NTN cell causes frequent cell reselection. If UE considers the RST to perform cell reselection, it helps to reduce the number of cell reselection. We suggest considering the RST of the candidate cell as a criterion for cell reselection.

Proposal 5	Introduce time-based cell reselection, i.e., UE should consider remaining service time of neighbor cell to prevent UE from performing cell reselection to a neighbor cell about to terminate service.

Possible solutions for time-based cell reselection enhancement are as follows:

Opt-1) Filtering neighbor cell with RST.
Step-1. UE calculates RST of neighbor cells.
Step-2. UE treats a neighbor cell as barred if RST of neighbor cell is shorter than time threshold.
Step-3. UE performs R-value-based ranking evaluation to neighbor cells.
Step-4. UE performs cell reselection to the highest ranked cell.

Opt-2) Filtering neighbor cell with R-value.
Step-1. UE performs R-value-based ranking evaluation to neighbor cells.
Step-2. UE treats a neighbor cell as barred if R-value of neighbor cell is less than threshold.
Step-3. UE performs cell reselection to neighbor cell having the longest RST.

Opt-3) Introduce RST-based R-value offset, in which neighbor cell having longer RST gets bigger value of R-value offset.
Step-1. UE calculates RST of neighbor cells.
Step-2. UE performs R-value-based ranking evaluation to neighbor cells.
Step-3. UE derives new R-value by adding RST-based R-value offset to R-value derived in Step-2.
Step-4. UE performs cell reselection to highest ranked cell.

Opt-1 prevents UE from performing cell reselection to a neighbor cell about to terminate service. On the other hand, Opt-2 focuses on minimizing cell reselection due to satellite movement. Opt-3 is hybrid form of Opt-1 and Opt-2.

Proposal 6	To adopt one of the following options for time-based reselection.

Opt-1) Filtering neighbor cell with RST.
Step-1. UE calculates RST of neighbor cells.
Step-2. UE treats a neighbor cell as barred if RST of neighbor cell is shorter than time threshold.
Step-3. UE performs R-value-based ranking evaluation to neighbor cells.
Step-4. UE performs cell reselection to the highest ranked cell.

Opt-2) Filtering neighbor cell with R-value.
Step-1. UE performs R-value-based ranking evaluation to neighbor cells.
Step-2. UE treats a neighbor cell as barred if R-value of neighbor cell is less than threshold.
Step-3. UE performs cell reselection to neighbor cell having the longest RST.

Opt-3) Introduce RST-based R-value offset, in which neighbor cell having longer RST gets bigger value of R-value offset.
Step-1. UE calculates RST of neighbor cells.
Step-2. UE performs R-value-based ranking evaluation to neighbor cells.
Step-3. UE derives new R-value by adding RST-based R-value offset to R-value derived in Step-2.
Step-4. UE performs cell reselection to highest ranked cell.

3. Conclusion

In this contribution, following statements are proposed:

Observation 1	Solutions to provide a single reference location at certain time point cannot support distance-based mobility introduced in Rel-17 NTN although they are sufficient to calculate RST of serving cell.

Proposal 1	In earth moving scenario, serving cell provides multiple reference locations and its time information.

Proposal 2	Distance threshold representing, e.g., edge of cell coverage should be provided to calculate RST of serving cell.

Proposal 3	Introduce distance-based cell reselection, i.e., UE should consider distance between UE location and reference location of neighbor cell to discriminate a neighbor cell that UE locates at the edge of the cell coverage.

Proposal 4	Distance-based cell reselection procedure should be introduced as follows in Rel-18 NTN.
Step-1. Exclude neighbor cell from candidate neighbor cell list if the distance between UE and its 
             reference location is larger than distance threshold.
Step-2. Cell ranking evaluation based on R-value criterion.
Step-3. UE reselects to the highest ranked cell.

Proposal 5	Introduce time-based cell reselection, i.e., UE should consider remaining service time of neighbor cell to prevent UE from performing cell reselection to a neighbor cell about to terminate service.

Proposal 6	To adopt one of the following options for time-based reselection.

Opt-1) Filtering neighbor cell with RST.
Step-1. UE calculates RST of neighbor cells.
Step-2. UE treats a neighbor cell as barred if RST of neighbor cell is shorter than time threshold.
Step-3. UE performs R-value-based ranking evaluation to neighbor cells.
Step-4. UE performs cell reselection to the highest ranked cell.

Opt-2) Filtering neighbor cell with R-value.
Step-1. UE performs R-value-based ranking evaluation to neighbor cells.
Step-2. UE treats a neighbor cell as barred if R-value of neighbor cell is less than threshold.
Step-3. UE performs cell reselection to neighbor cell having the longest RST.

Opt-3) Introduce RST-based R-value offset, in which neighbor cell having longer RST gets bigger value of R-value offset.
Step-1. UE calculates RST of neighbor cells.
Step-2. UE performs R-value-based ranking evaluation to neighbor cells.
Step-3. UE derives new R-value by adding RST-based R-value offset to R-value derived in Step-2.
Step-4. UE performs cell reselection to highest ranked cell.

4. References

[1] R2-2210860, Report of [offline-117] cell reselection enhancements (intel), Intel