3GPP TSG-RAN WG2 #121                                                                                        R2-2300801

Athens, Grecee, 27th Feb. – 03rd March 2023

Agenda Item:	    5.1.3.1

Source: 	    MediaTek Inc.

Title:		    Discussion on 1 second UAI resend rule after RRCReconfiguration including fullConfig

Document for:	    Discussion and decision

# 1 Introduction

This document discusses the UE UAI resending behavior upon receiving RRC Reconfiguration (handover) with full configuration.

# 2 Discussion

In 38.331 5.3.5.3, it is specified that the UE shall resend the UAI after handover if the UAI was sent 1 second before the handover command.

2>	if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup:

3>	if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, and the UE is configured to provide UE assistance information for the corresponding cell group, and the UE has initiated transmission of a UEAssistanceInformation message for the corresponding cell group since it was configured to do so in accordance with 5.7.4.2:

4>	initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information;

4>	start or restart the prohibit timer (if exists) or the leave without response timer for the MUSIM associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration;

In case the handover command includes fullConfig, it is our understanding that the UE should start from the "clean table" and configuration before fullConfig is not relevant at all. However, it is not clear that 1 second rule apply also in this case. Can the UE assume that the configuration before fullConfig is not relevant and, because of that, there is no need to resend UAI even 1 second rule would apply?

We have encountered the following scenario during IODT test for UAI overheating

- Step 1: UE sends UAI to indicate it experiences overheating

- Step 2: UE sends UAI to indicate it no longer experiences overheating

- Step 3: Handover with fullConfig occurs within 1 second of step 2. Overheating UAI is still configured.

- Step 4:

  - UE behavior 1: Due to 1 second rule, UE sends UAI which indicates the UE no longer experiences overheating.

  - UE behavior 2: UE does not send overheating UAI, because the UAI procedure starts from the scratch and the first overheating UAI can't indicate that UE is not experiencing overheating.

There are different views during the testing and it is better that 3GPP could clarify the intended UE behavior. Does UE really have to resend overheating cancellation in above case? Our view is that the resending of UAI does not make too much sense in case of full configuration. Of course that the UE may trigger the UAI again according to the new configuration but always resending is not necessary while the NW clear all previous configuration.

Proposal 1: RAN2 confirms that, the UE should not resend UAI based on 1 second resend rule in case the handover command include fullConfig.

If P1 is agreed, the next question is that whether some clarification is needed and which Release to clarity the UE behavior. As this scenario has caused some confusion in real implementation, we prefer to have some SPEC clarification. However, we think there is no need to change R15/R16 SPEC for this case. It is sufficient to clarify this from Rel-17. We provide a CR in [1]. It can be clarified by adding a new NOTE or change the procedure text.

Proposal 2: If P1 is agreed, RAN2 to discuss how to clarify the intend UE behavior based on the CR proposed in R2-2300802.

# 3 Conclusions

Based on the discussion in section 2, we propose the following:

Proposal 1: RAN2 confirms that, the UE should not resend UAI based on 1 second resend rule in case the handover command include fullConfig.

Proposal 2: If P1 is agreed, RAN2 to discuss how to clarify the intend UE behavior based on the CR proposed in R2-2300802.

# 4 References

[1] R2-2300802, “Clarification on 1 second UAI resend rule after RRCReconfiguration including fullConfig”, MediaTek.