3GPP TSG-RAN WG2 Meeting #121	R2-2300815

Athens, February., 2023

Agenda Item	: 8.9.3

Source	: LG Electronics Inc.

Title             : Consideration on service continuity enhancement for L2 U2N relay

Document for	: Discussion and Decision

1.	Introduction

In previous RAN2 meetings, when direct/indirect-to-indirect path switching is performed, RAN2 discussed which gNB decides path type and target relay UE. Before further discussion, RAN2 is waiting for RAN3's decision. RAN3 already agreed that source gNB decides path type. However, it still discusses which gNB will decide the final target relay UE in RAN3. Also, RAN2 can still discuss which gNB would be better to select the final target relay UE.

2.	Discussion

2.1 Selection of target relay UE

When direct/indirect-to-indirect path switching, the source gNB decides path type (i.e., direct path or indirect path). But it is not yet decided which gNB selects the target relay UE. There could be two options. The option 1 is that the source gNB selects the target relay UE based on the reported measurement results from the remote UE. And the option 2 is that the source gNB sends to the target gNB a list of candidate target relay UEs and relay UE related information (e.g., the SRC L2 ID and its serving cell ID for each candidate relay UE).

In option 1, to support the direct/indirect-to-indirect HO, the selected relay UE related information should be provided to the target gNB. Therefore, the source gNB needs to inform the target gNB about information on the target relay UE in XnAP HANDOVER REQUEST message (e.g, L2 U2N Relay UE's source L2 ID, serving cell ID). In addition, the source gNB may need to send the SD-RSRP between the selected relay UE and remote UE. Because the threshold for the measurement and reporting will be different depending on each gNB, the SD-RSRP to be used for selecting target relay UE in source gNB may not be applicable in the target gNB. If the SD-RSRP of the candidate relay UE selected by the source gNB side is not a proper value in the target gNB side, the target gNB can reject the HO request to the source gNB. Also, the target gNB may send the HANDOVER PREPARATION FAILURE message to the source gNB in order to reject the handover towards the selected target relay UE due to e.g., the current load of relay UE, and so on. In this case, the source gNB should reselect the target relay UE and initiate the Handover procedure towards the reselected relay UE, thus causing unnecessary Xn signalling between gNBs.

Observation 1: Option 1 causes unnecessary Xn signalling between gNBs due to the frequent handover failure.

In Rel-17 U2N service continuity, intra-gNB handover is only supported. Therefore, the gNB is able to consider the detailed information for the relay UE (e.g., current RRC state for candidate relay UE and current load of relay UE) during direct-to-indirect handover procedure. However, in Rel-18 U2N service continuity, inter-gNB handover should be supported. In Option 1, the source gNB could not be aware of the current RRC state for candidate relay UE compared to the service continuity procedure in Rel-17. If the source gNB selects the UE in RRC_IDLE or RRC_INACTIVE as a target relay UE, some additional time may be needed to transit to RRC_CONNECTED for the target relay UE.

Observation 2: In Option 1, the source gNB could not be aware of the current RRC state for candidate relay UE.

In Option 2, the source gNB sends to the target gNB a list of candidate target relay UEs and relay UE related information (e.g., the SRC L2 ID and its serving cell ID for each candidate relay UE). Then, the target gNB makes the final decision on the target relay UE in the list of candidate target relay UEs because the target gNB has more knowledge to properly select the target relay UE. In this option, the handover procedure can be successful except for the case where the target gNB rejects all the candidate relay UEs as the selected relay UE. Hence, it reduces the possibility of the handover failure compared to Option 1.

In addition, the target gNB may prefer the selection of the RRC_CONNECTED relay UE to reduce HO latency since it can avoid the state transition from RRC_IDLE/INACTIVE to RRC_CONNECTED.

Observation 3: Option 2 can reduce the possibility of the handover failure compared to Option 1.

Observation 4: In Option 2, the target gNB is able to have more knowledge to properly select the target relay UE.

However, as in [1], the detailed mechanism for Option 2 should be further discussed. For example, it is still unclear whether source gNB can choose candidate relay UEs belonging to multiple target cells or gNBs. This open issue is also related to the RAN3 working assumption “During inter-gNB path switching, source gNB can signal the serving cell of the relay UE to target gNB via existing IE Target Cell Global ID”. That is, if the Option 2 is selected, how to construct the Target Cell Global ID IE in XnAP HANDOVER REQUEST message should be also discussed.

We think that the source gNB should determine the list of candidate target relay UEs served by same target cell under the same target gNB. Then, the target gNB can select the target relay UE among the candidate relay UEs served by same target cell based on the Uu measurement result from candidate relay UE, the current RRC state and load status of the candidate relay UE, and so on. In this case, the source gNB can provide the serving cell of the candidate relay UEs via the existing Target Cell Global ID IE without any enhancements.

Another issue is how the source gNB can restrict to candidate relay UEs belonging to one target cell under same target gNB. We think that the source gNB first determines the serving cell of the relay UE with the best SD-RSRP as the target cell. Then, the list of candidate target relay UEs can be determined by including the candidate relay UE(s) fulfilling the event Y1 or Y2 (for direct-to-indirect handover case), or the event Z1 (for indirect-to-indirect handover case) under the same target cell.

Suppose that according to the measurement results, the remote UE reports the following three candidate relay UEs to the gNB1.

- Relay UE A in cell 3 (gNB2)

- Relay UE B in cell 3 (gNB2)

- Relay UE C in cell 4 (gNB3)

If the relay UE A or relay UE B has the best SD-RSRP, the source gNB selects the cell 3 as the target cell. Then, the list of the candidate relay UEs is set to {Relay A, Relay B}. Based on the additional knowledge for the relay UE (e.g., Uu measurement result from candidate relay UE, the current RRC state and load status of the candidate relay UE), the target gNB (i.e., gNB2) selects the target relay UE. If the relay UE C has the best SD-RSRP, the source gNB selects the cell 4 and Relay UE C as the target cell and candidate relay UE, respectively.

Also, our understanding is that Option 2 is not against Option 1. Option 1 can be a subset of Option 2 if the source gNB only provides one candidate relay UE to the target gNB.

Observation 5: In Option 2, the source gNB needs to determine the list of candidate target relay UEs only served by same target cell under the same target gNB.

Observation 6: Option 1 can be a subset of Option 2.

Based on the above observations, we prefer Options 2 as the compromised solution to properly select the target relay UE. Therefore, the following proposals are suggested to RAN3:

Proposal 1: The source gNB determines the list of candidate target relay UEs only served by same target cell under the same target gNB and provides this list to the target gNB.

Proposal 2: The target gNB selects the target relay UE among the candidate relay UE(s) served by the same target cell.

3.	Conclusion

Proposal 1: The source gNB determines the list of candidate target relay UEs only served by same target cell under the same target gNB and provides this list to the target gNB.

Proposal 2: The target gNB selects the target relay UE among the candidate relay UE(s) served by the same target cell.

4.	Reference

- RP-221262, “Revised WID on NR sidelink relay enhancements,” LG Electronics, June, 2022.