3GPP TSG-RAN WG2 #121                                                                                           R2-2300818

Athens, Grecee, 27th Feb. – 03rd March 2023                                             Revision of R2-2212664

Agenda Item:	    8.4.4

Source: 	    MediaTek Inc.

Title:		    Discussion on CHO with candidate SCG

Document for:	    Discussion and decision

# 1 Introduction

In this contribution, we discuss below objective #4 from mobility enhancement work item [1].

- To specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [RAN3, RAN2]

- CHO including target MCG and target SCG is used as the baseline

R2 has made the following agreements related to objective #4

In RAN2#119

- Observation: Current RAN2 Stage-3 specifications can support CHO including target MCG and target SCG in Rel-17.

- CHO configuration referring to or including CPC/CPA configuration (intended to be applicable together) can be supported.

- FFS: When triggering CHO, UE perform CPC/CPA configuration to start CPC/CPA evaluation, FFS if CHO evaluation and CPC/CPA evaluation is concurrent or sequential.

Chair: NOTE that the above agreements are NOT intended to describe the Stage3 signalling details.

In RAN2#120

- Execution order: the UE doesn’t execute CPC/CPA unless CHO condition is fulfilled (regardless parallel or sequential evaluation)

We would like to further discuss the supported scenario for objective#4.

# 2 Discussion

According to previous RAN2 discussion, Companies are thinking two different scenarios for objective#4:

- Sequential evaluation of CHO and CPC/CPA

  - Step 1: The NW configures execution condition based on candidate PCell and the associated candidate CHO configuration, where the candidate CHO configuration (i.e. RRC Reconfiguration message) includes CPA/CPC configurations.

  - Step 2: The UE evaluates the execution condition.

  - Step 3: If the execution condition is met, the UE applies the RRC Reconfiguration message for CHO and start monitoring the execution conditions as indicated in the CPA/CPC configuration.

- Simultaneous evaluation of CHO and CPC/CPA

  - Step 1: The NW configures execution condition based on candidate PCell and PSCell and the associated candidate CHO configuration, where the candidate CHO configuration (i.e. RRC Reconfiguration message) includes target SCG configurations.

  - Step 2: The UE evaluates the execution condition.

  - Step 3: If the execution condition is met, the UE applies the RRC Reconfiguration message for CHO and add/modify SCG as indicated in the target SCG configurations.

It is our understanding that the wording in objective#4 implies sequential evaluation of CHO and CPC/CPA. The WID saying that CHO with target SCG should be used as baseline for this objective. The CHO with target SCG implies that the CHO execution command includes the target SCG configuration so that SCG is added or changed after CHO execution. The CHO with candidate SCG implies that the CHO execution command includes the candidate SCG configuration (i.e. CPC/CPA) so that the UE applies the CPC/CPA configuration after CHO execution.

In Rel-17, RAN2 agreed to support CHO with target SCG, where the execution condition is based on the measurement of candidate PCell (i.e. only based on CHO execution condition, not related to candidate PSCell). For simultaneous evaluation, it is more relevant to enhance the execution condition for CHO with target SCG. The execution condition for CHO with target SCG could be modified so that the condition is based not only on candidate PCell, but also on candidate PSCell. We do not think this scenario is currently included in the WID but are open for discussion.

The SPEC impact for sequential evaluation is not so big. One of the major changes will be remove the following configuration limitation in TS 38.331.

CondReconfigToAddMod field descriptions

condRRCReconfig The RRCReconfiguration message to be applied when the condition(s) are fulfilled. The RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration or the field daps-Config. For CPA and for CPC, the RRCReconfiguration message contained in condRRCReconfig cannot contain the field scg-State.

On the other hand, the simultaneous evaluation requests more STD work than the sequential evaluation. RAN2 should clarify on how to define the execution condition based on candidate PCell and PSCell. And if only candidate PCell or PSCell fulfilled the condition, should UE perform any reconfiguration. It is more complicate than sequential evaluation.

The following table summarize the different CHO with SCG scenarios. We hope that companies in RAN2 have same understanding on different scenarios.

Table I – Different CHO with SCG scenarios

| Scenario | Execution condition | Execution Command (RRC Reconfiguration Message) |
| --- | --- | --- |
| 1 – R16 CHO | Based on Candidate PCell | Do NOT include CPC/CPA or target PSCell configuration |
| 2 – R17 CHO with target SCG | Based on Candidate PCell | Include target PSCell Configuration |
| 3 – R18 CHO with candidate SCG (Sequential evaluation) | Based on Candidate PCell | Include candidate PSCell Configuration (i.e. CPC/CPA configuration) Note: Can still reconfigure PSCell (including target PSCell) as in R17. |
| 4 – R18 CHO with candidate SCG (Parallel evaluation) | Based on Candidate PCell and Candidate PSCell | Include target PSCell Configuration Note: Can still include CPC configuration if scenario 3 is allowed. |

Based on above analysis, we suggest RAN2 to support sequential evaluation as a baseline. It can be further discussed whether the simultaneous evaluation should be included in Rel-18.

Proposal 1: RAN2 supports the following scenario for CHO with candidate SCG

- Step 1: The NW configures execution condition based on candidate PCell and the associated candidate CHO configuration, where the candidate CHO configuration (i.e. RRC Reconfiguration message) includes CPA/CPC configurations.

- Step 2: The UE evaluates the execution condition.

- Step 3: If the execution condition is met, the UE applies the RRC Reconfiguration message for CHO and start monitoring the execution conditions as indicated in the CPA/CPC configuration.

Proposal 2: RAN2 to discuss whether to enhance the execution condition for CHO with target SCG. Baseline scenario as below:

- Step 1: The NW configures execution condition based on candidate PCell and PSCell and the associated candidate CHO configuration, where the candidate CHO configuration (i.e. RRC Reconfiguration message) includes target SCG configurations.

- Step 2: The UE evaluates the execution condition.

- Step 3: If the execution condition is met, the UE applies the RRC Reconfiguration message for CHO and add/modify SCG as indicated in the target SCG configurations.

# 3 Conclusions

Based on the discussion in section 2, we propose the following:

Proposal 1: RAN2 supports the following scenario for CHO with candidate SCG

- Step 1: The NW configures execution condition based on candidate PCell and the associated candidate CHO configuration, where the candidate CHO configuration (i.e. RRC Reconfiguration message) includes CPA/CPC configurations.

- Step 2: The UE evaluates the execution condition.

- Step 3: If the execution condition is met, the UE applies the RRC Reconfiguration message for CHO and start monitoring the execution conditions as indicated in the CPA/CPC configuration.

Proposal 2: RAN2 to discuss whether to enhance the execution condition for CHO with target SCG. Baseline scenario as below:

- Step 1: The NW configures execution condition based on candidate PCell and PSCell and the associated candidate CHO configuration, where the candidate CHO configuration (i.e. RRC Reconfiguration message) includes target SCG configurations.

- Step 2: The UE evaluates the execution condition.

- Step 3: If the execution condition is met, the UE applies the RRC Reconfiguration message for CHO and add/modify SCG as indicated in the target SCG configurations.

# 4 References

[1] RP-222332, “Revised WID on Further NR mobility enhancements”, MediaTek