3GPP TSG-RAN WG2 Meeting #121	R2-2300832

# Athens, Greece, 27 February – 3 March 2023

Agenda item:		8.10.3

Source:	Intel Corporation

Title:	TDM solution for IDC

Document for:		Discussion and Decision

# Introduction

In RAN2#120 meeting, following was agreed regarding TDM solutions for IDC:

⇒ Periodic pattern is supported; FFS on the values;. ⇒ The periodic pattern reported by the UE includes cycle, start offset and active duration. FFS, whether multiple patterns are supported. FFS on per CG pattern. ⇒ NR DRX values can be treated as a starting point for assistance information reported by UE. FFS, on exact values. ⇒ Autonomous denial solution is supported in Rel-18 IDC, RAN2 will not introduce other solution on aperiodic use case (i.e. no report from UE on this aperiodic issue).

Email discussion “[Post120][651][IDC] Further details of TDM solution (vivo)” was held to progress details of TDM solutions.

In this contribution, we discuss several open issues of TDM solutions for IDC.

# Discussion

## Assistance information for BT voice use case

In email discussion “[Post120][651][IDC] Further details of TDM solution (vivo)”, most companies agree to use NR values of cycle, start offset and active duration for the periodic pattern in IDC assistance information. One open issue is how to handle BT voice use case.

Before discussing NR solution, we first review LTE IDC solution for BT voice use case. For coexistence with Bluetooth Voice (eSCO, extended Synchronous Connection Orientated), fine TDM pattern between LTE and Bluetooth is required. For Bluetooth eSCO, a clean Tx/Rx pair should be available for Bluetooth within EV3 interval, which is 3.75 ms (Annex B of TR 36.816 [1]).

Figure B.1.1-1: BT Tx/Rx pairs

In LTE, UE reports HARQ subframe pattern to eNB, and the pattern takes into account of UL synchronous HARQ.

Figure 5.2.1.2.2-1 of TR 36.816: Example of HARQ process reservation solution

After eNB receives HARQ subframe pattern, it can signal DRX configuration to the UE.

For NR, it was agreed that only periodic pattern is supported, while HARQ subframe pattern like assistance information is not supported.

During email discussion “[Post120][651][IDC] Further details of TDM solution (vivo)”, there were two approaches to solve the coexistence with BT voice case. One method is to introduce cycle length of 3.75 ms, and the other method is to report short DRX cycle in addition to the pattern parameters (cycle, start offset, and active duration).

With proper configuration of short DRX, it is possible to have a clean Tx/Rx pair available for Bluetooth within EV3 interval. For example when short DRX cycle length is 4 ms, and on duration is 2 ms, it is guaranteed that a full Tx/Rx pair can be available in NR “inactive” duration (since 3 * 0.625 ms = 1.875 ms < 2 ms). An example is shown in Figure 1 below.

Figure 1: Coexistence with Bluetooth using short DRX cycle

From above discussion, it can be seen that both methods work. One compromise is to introduce cycle length of 4 ms, so that gNB can configure short DRX cycle accordingly.

Proposal 1: For the values of periodic pattern, introduce cycle length of 4 ms for Bluetooth voice use case.

## Configuration on TDM assistance information reporting

In LTE Rel-11 IDC, there is no separate configuration of IDC FDM and TDM assistance information (i.e. idc-Indication-r11 controls both FDM and TDM assistance information reporting), while autonomous denial is separately configured.

IDC-Config-r11 ::=				SEQUENCE {

idc-Indication-r11					ENUMERATED {setup}				OPTIONAL,	-- Need OR

autonomousDenialParameters-r11		SEQUENCE {

autonomousDenialSubframes-r11			ENUMERATED {n2, n5, n10, n15,

n20, n30, spare2, spare1},

autonomousDenialValidity-r11			ENUMERATED {

sf200, sf500, sf1000, sf2000,

spare4, spare3, spare2, spare1}

}		OPTIONAL,		-- Need OR

...,

}

For Rel-18 IDC, since “The TDM solution is considered complementary to the FDM solution” according to WID RP-221281, it is proposed that whether UE can report TDM assistance information is separately configured from enhanced FDM solution reporting.

Proposal 2: Whether UE can report TDM assistance information (periodic pattern) is configured by gNB, and is separately configured from enhanced FDM reporting.

## Autonomous denial

In email discussion “[Post120][651][IDC] Further details of TDM solution (vivo)”, regarding the unit and values for autonomous denial, there are two options:

Option 1: Subframe as time unit, with LTE values reused.

Option 2: Slot as time unit.

Given that coexistence scenarios in NR and LTE are the same, it is straightforward to reuse LTE signalling since subframe length is fixed to 1 ms in NR. Option 2 requires more standardization efforts since slot length varies with respect to numerology.

Proposal 3: For autonomous denial, time unit is subframe and LTE values are reused.

# Conclusion

In this contribution, we discuss several open issues of TDM solutions for IDC, and propose the following:

Proposal 1: For the values of periodic pattern, introduce cycle length of 4 ms for Bluetooth voice use case.

Proposal 2: Whether UE can report TDM assistance information (periodic pattern) is configured by gNB, and is separately configured from enhanced FDM reporting.

Proposal 3: For autonomous denial, time unit is subframe and LTE values are reused.

# References

[1] 3GPP TR 36.816, "Study on signalling and procedure for interference avoidance for in-device coexistence"