3GPP TSG-RAN WG2 Meeting #121	R2-2300840

# Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	8.15.2

Source: 	Xiaomi

Title:  	Discussion on channel access for sidelink operation on unlicensed spectrum

Document for:	Discussion and Decision

1	Introduction

In last RAN2 meeting, RAN2 discussed the CAPC for SL-U and achieved some agreements [1]. In this contribution, we would like to discuss some remaining issues and provide the corresponding proposals.

2	Discussion

2.1 CAPC table mapping

In last RAN2 meeting we have achieved a WA on the detailed mapping between different PQI and CAPC based on PDB only. However there was some concern whether some other criterion should be considered.

Agreements on SL CAPC mapping table:

1: 	Confirm the WA “PQI is used to determine the CAPC mapping as in NR-U” as baseline.

2:	Working assumption

- Mapping PQI 90/91/92/93/21/22/23/55/56/57/58 to CAPC priority class 1. FFS on other SL CAPC mapping criterion.

- Mapping PQI 59/61 to CAPC priority class 3.

- Mapping PQI 25 to CAPC priority class 2.

- Mapping PQI 24/26/60 to CAPC priority class 1

Besides PDB, we think default priority should also be considered as one criterion when determining the mapping relationship. As indicated in the Annex, there are some services with quite restrictive PDB requirement (i.e, 5ms, 10ms, 20ms) but the corresponding default priority is the lowest (i.e., 5, 6). Therefore if only PDB is considered, then these services will be categorized as the highest CAPC priority, but actually will be pre-empted or dropped during resource selection due to the lowest default priority. So it is not reasonable to always set these services to the highest CAPC priority.

Regarding if we consider default priority, is it {PDB or default priority level} or {PDB and default priority level}? In our understanding, to consider default priority only is not comprehensive either. Similarly as the case above, there are some service with quite high priority (i.e., 1, 2) but the corresponding PDB requirement is the least restrictive (i.e, 200ms). Therefore, if only default priority is considered, then these services will be categorized as the highest CAPC priority, but actually the latency requirement is not that restrictive. So it is not reasonable to always set these services to the highest CAPC priority either.

Based on the above analysis, we think both PDB and default priority level should be taken into account when determining the PQI to CAPC mapping.

Proposal 1: RAN2 to not confirm the WA but agree both PDB and default priority should be considered together when determining the PQI to CAPC mapping.

According to the tables for PQI, if we categorize the PQI based on both PDB and priority:

- For PQI 21/55/90/91, they have quite stringent PDB requirement and quite high priority, so should be categorized as CAPC class 1;

- For PQI 56/57/92/93, they have quite stringent PDB requirement but quite low priority, so should be categorized as CAPC class 2.

- For PQI 22/23/58, they have relatively loose PDB requirement and medium priority, so should be categorized as CAPC class 1 or 2.

- For PQI 25, they have relatively loose PDB requirement but quite high priority, so should be categorized as CAPC class 2.

- For PQI 59/61, they have quite loose PDB requirement and quite low priority, so should be categorized as CAPC class 3.

- For PQI 24/26/60, even though their PDB requirement is not that restrictive, but if we follow NR-U principle, these services should be categorized as CAPC class 1.

Based on the above analysis, we propose RAN2 to adopt the following CAPC to PQI mapping table.

| CAPC | PQI |
| --- | --- |
| 1 | 21, 24, 26,55, 60, 90, 91 |
| 2 | [22, 23], 25, 56, 57, [58], 92, 93 |
| 3 | 59, 61 |
| 4 | - |
| NOTE: lower CAPC value means higher priority - |  |

Table 1

Proposal 2: RAN2 to agree to map PQI 21/24/26/55/60/90/91 to CAPC priority class 1.

Proposal 3: RAN2 to agree to map PQI 25/56/57/92/93 to CAPC priority class 2.

Proposal 4: RAN2 to agree to map PQI 59/61 to CAPC priority class 3.

Proposal 5: RAN2 to discuss whether to map PQI 22/23/58 to CAPC priority class 1 or 2.

2.2 CAPC selection and multiplexing of data

During last meeting, we have reached the following WA on how to determine the CAPC of the TB if CAPC is not indicated in DCI.

Agreement on SL CAPC rules

1: 	Working assumption: If PQI-based CAPC mapping is agreed, as in NR-U, the lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used regardless of whether the TB also contains SL MAC CEs in addition to MAC SDUs

We think the current WA is aligned with NR-U and should be confirmed. Since we have already agreed with PQI-based mapping, the “if” condition can be deleted from the proposal.

Proposal 6: RAN2 to confirm the WA “as in NR-U, the lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used regardless of whether the TB also contains SL MAC CEs in addition to MAC SDUs” as agreement.

2.3 Handling of non-standardized PQI

Regarding the handling of non-standardized PQI, we have achieved some agreements/WA during last meeting.

Agreements on SL CAPC for RRC inactive/idle/OOC UE

1: 	For an IDLE/INACTIVE/OOC UE, if the QoS flow of non-standardized PQI can be mapped to a non-default SLRB, the UE determines the CAPC of this non-standardized PQI using the CAPC of this SLRB.

2:	Working assumption: Use the CAPC of the standardized PQ I or the CAPC of non-standardized PQI configured in SIB/pre-configuration which best matches the QoS characteristics of the current non-standardized PQI based on one or more QoS characteristics

Actually there was another proposed alternative i.e., use default CAPC value which is pre-defined or configured by SIB/pre-configuration. However, as companies mentioned, non-standardized PQI is various and it is not flexible to map all the non-standardized PQI to single pre-defined default CAPC. So we propose to confirm the WA as agreement.

In addition, regarding whether to define detailed rules on how to determine which standardized PQI or non-standardized PQI configured in SIB/pre-configuration best matches the current non-standardized PQI, we think it is hard and complicated to specify all the detailed conditions especially when the PQI to CAPC mapping is based on more than one criterion, so it is better to leave this to UE implementation, i.e., UE determines the standardized PQI or non-standardized PQI configured in SIB/pre-configuration best matches the current non-standardized PQI based on one or more QoS characteristics by implementation.

Proposal 7: Confirm the WA “Use the CAPC of the standardized PQI or the CAPC of non-standardized PQI configured in SIB/pre-configuration which best matches the QoS characteristics of the current non-standardized PQI based on one or more QoS characteristics” as agreement.

Proposal 8: UE determines the standardized PQI or non-standardized PQI configured in SIB/pre-configuration best matches the current non-standardized PQI based on one or more QoS characteristics by implementation.

2.4 CAPC table on UE side

Based on the above WA: “Use the CAPC of the standardized PQI or the CAPC of non-standardized PQI configured in SIB/pre-configuration which best matches the QoS characteristics of the current non-standardized PQI based on one or more QoS characteristics”, it requires the UE to check the table to determine the CAPC for the non-standardized PQI. Therefore the CAPC table also applies to the UE side.

Proposal 9: In SL-U, the CAPC mapping table also applies to the UE side.

Regarding the CAPC for a certain DRB, for UEs operating in RRC connected mode, the network is informed of the PQI or QoS parameters for a QoS flow based on the reported SUI, then the network can take the QoS of all the flows associated with the same DRB into account to configure a proper CAPC for the DRB. While for UEs operating in RRC IDLE/INACTIVE mode or OOC, there is no such assistance information and if a DRB includes QoS flows that have not been reported to the network before, the configured CAPC in SIB/pre-configuration may not match the DRB well. In this case, a more suitable solution is to rely on the UE itself to check the CAPC table and determines the CAPC for the DRB when the UE is in RRC IDLE/INACTIE or OOC. Therefore, if the network has a good knowledge of the associated QoS flows multiplexed into the same DRB, then the gNB can include the CAPC in the DRB configuration and UE should follow the configured CAPC. However if the CAPC is not included in the DRB configuration, then the UE itself needs to check the table and determines the CAPC for the DRB.

Proposal 10: In SL-U, the RRC IDLE/INACTIVE or OOC UE should apply the configured CAPC for the DRB if CAPC is configured in SIB12 or pre-configuration.

Proposal 11: In SL-U, the RRC IDLE/INACTIVE or OOC UE checks the mapping table and determines the CAPC itself for the DRB if CAPC is not configured in SIB12 or pre-configuration.

2.5 LCP impact from COT sharing

For COT sharing in SL-U, there was some progress achieved in last RAN1 meeting:

For UE-to-UE COT sharing, - When performing S-SSB transmission(s), a responding UE can utilize a COT shared by a COT initiating UE (using type 1 channel access) when the responding UE is intended to transmit S-SSB within RB set(s) corresponding to the shared COT. - When performing PSFCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE. - FFS: whether a responding UE can transmit PSFCH(s) to UE(s) other than the initiator - When performing PSSCH/PSCCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when the responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to the shared COT is intended for the COT initiating UE - FFS whether to support the case if a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information. - FFS: how to determine / what are the restrictions to the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) to utilize the COT shared by the initiating UE. - FFS whether the responding UE can utilize the COT when at least the responding UE’s PSCCH transmission in the reserved resources within the shared COT or MCSt is intended for the COT initiating UE and what are the restrictions (e.g., priority, etc.) and indication to the responding UE. - FFS: UE forwarding/relaying information about a COT initiated by another UE.

During last RAN2 meeting, there was some discussion on COT sharing and RAN2 agreed to study if there is any impact on LCP and DRX operation from COT sharing.

Agreements on SL COT sharing

1: 	RAN2 will study whether/how LCP is impacted from COT sharing.

2: 	RAN2 will consider interaction between DRX operation and shared COT.

Regarding the LCP impact, RAN1 is still discussing the restrictions on the DST ID of the responding UE to utilize the shared COT as highlighted in green, which means whether only the initiating UE is the targeted receiver of the transmission using the shared COT or other UEs are also allowed still remains unclear. In addition, even RAN1 agreed that the responding UE can only use the shared COT for its transmission which has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information, there may be some PHY solutions, i.e., CAPC information is exchanged in SCI and initiating UE only shares COT to the UEs with smaller CAPC, which can avoid the impact on LCP. Therefore, it is too early to conclude LCP is impacted from COT sharing.

Observation 1: RAN1 is still discussing the restrictions on the DST ID of the responding UE to utilize the shared COT, it is too early for RAN2 to conclude LCP is impacted from COT sharing.

Actually in NR-U, it is up to NW to configure the CAPC in DCI for a certain packet based on the reported BSR. The logic is that with the buffer status, NW implementation is able to derive the LCHs pending for transmission and further derive the generated packet based on LCP and the allocated grant which is also under centralized control by NW. In this case, if we follow NR-U principle, it is also workable if some BSR information exchange is supported between initiating UE and responding UE, with which the initiating UE can derive the CAPC of the packet pending for transmission and share COT to responding UE with smaller CAPC.

Proposal 12: RAN2 to discuss to exchange BSR information between initiating UE and responding UE for COT sharing.

Another issue mentioned during online discussion is how to handle the case when LCP is done before COT information arrives. For this case, we think it is not efficient and will introduce additional complexity on UE implementation if UE performs another LCP to generate a packet in case the current packet does not satisfy the shared COT. Therefore, if the generated packet is not allowed to use the shared COT to transmit, the responding UE shall use type 1 LBT. Regarding whether the shared COT should be allowed for other UEs to use or not, it depends on RAN1.

Proposal 13: Responding UE performs type 1 LBT if LCP is done before COT information arrives and the generated packet cannot satisfy the shared COT.

Proposal 14: Whether the shared COT should be allowed for other UEs to use or not depends on RAN1.

2.6 DRX impact from COT sharing

Regarding DRX impact, in last meeting, there was some proposal to require the initiating UE to consider the shared COT duration it shares as its own DRX active time if DRX is supported based on the assumption that the responding UE will transmit SL data within the shared COT duration to the initiating UE. However, as highlighted above, RAN1 is still discussing the restrictions on the DST ID of the responding UE to utilize the shared COT, if responding UE is also allowed to transmit to other UEs using the shared COT, then the initiating UE may not need to keep active during the shared COT duration.

Observation 2: Initiating UE may not need to keep active during the shared COT duration if responding UE is allowed to transmit to other UEs using the shared COT which is still under discussion in RAN1.

In addition, the details on how to support COT sharing in groupcast and broadcast are still under discussion in RAN1. However, different from unicast, if the shared COT is utilized for gourpcast/broadcast transmission, all UEs within the group or all UEs interested for this L2 ID should keep active during the shared COT duration.

Proposal 15: If the shared COT is utilized for gourpcast/broadcast transmission, all UEs within the group or all UEs interested for this L2 ID should keep active during the shared COT duration.

2.7 Resource allocation impact from COT sharing

With the introduction of IUC, UE shall randomly select the resources within the intersection of the received preferred resource set and the resources indicated by the physical layer when preferred resource set is received and UE has its own sensing result. Similarly, even the detailed procedure of resource allocation is still under discussion in RAN1, from MAC perspective, some mechanism that impact the selection procedure in higher layer can be considered by RAN2. Since less effort on LBT is required for resources within the shared COT, when the UE is the responding UE and operates in mode 2, it is beneficial to firstly select resources from resources indicated by the physical layer and within the shared COT upon resource (re-)selection. If there is no remaining resource within the shared COT that can be selected, then the UE randomly select resources from the resources indicated by the physical layer as in legacy.

Proposal 16: When the UE is the responding UE and operates in mode 2, the UE firstly selects resources from resources indicated by the physical layer and within the shared COT upon resource (re-)selection.

2.8 COT sharing for mode 1

According to RAN1, COT sharing can be supported for both mode 1 and mode 2. However, different from NR-U, where both COT sharing and resource allocation are all centralized controlled by the network, in SL-U, COT sharing is performed by the UE while the transmission resource is allocated by the network when operating in mode 1. Therefore, without any information of the shared COT, the network may not be able to schedule the resource accordingly. In order to assist the network to allocate corresponding resource within the shared COT, initiating UE or responding UE or any UE detecting the shared COT shall report the related information to the network. Detailed information may include the start/end of the shared COT, the remaining COT duration, the L2 ID of the initiating UE/responding UE, CAPC etc., which can be further discussed by RAN2 and UE can reuse the existing SUI or UAI to report this assistance information to the network.

Proposal 17: RAN2 to discuss how to support UE to report the shared COT information to the network for mode 1 scheduling.

3	Conclusion

In this contribution, we discussed about CAPC for sidelink operation on unlicensed spectrum and have the corresponding observations and proposals:

Proposal 1: RAN2 to not confirm the WA but agree both PDB and default priority should be considered together when determining the PQI to CAPC mapping.

Proposal 2: RAN2 to agree to map PQI 21/24/26/55/60/90/91 to CAPC priority class 1.

Proposal 3: RAN2 to agree to map PQI 25/56/57/92/93 to CAPC priority class 2.

Proposal 4: RAN2 to agree to map PQI 59/61 to CAPC priority class 3.

Proposal 5: RAN2 to discuss whether to map PQI 22/23/58 to CAPC priority class 1 or 2.

Proposal 6: RAN2 to confirm the WA “as in NR-U, the lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used regardless of whether the TB also contains SL MAC CEs in addition to MAC SDUs” as agreement.

Proposal 7: Confirm the WA “Use the CAPC of the standardized PQI or the CAPC of non-standardized PQI configured in SIB/pre-configuration which best matches the QoS characteristics of the current non-standardized PQI based on one or more QoS characteristics” as agreement.

Proposal 8: UE determines the standardized PQI or non-standardized PQI configured in SIB/pre-configuration best matches the current non-standardized PQI based on one or more QoS characteristics by implementation.

Proposal 9: In SL-U, the CAPC mapping table also applies to the UE side.

Proposal 10: In SL-U, the RRC IDLE/INACTIVE or OOC UE should apply the configured CAPC for the DRB if CAPC is configured in SIB12 or pre-configuration.

Proposal 11: In SL-U, the RRC IDLE/INACTIVE or OOC UE checks the mapping table and determines the CAPC itself for the DRB if CAPC is not configured in SIB12 or pre-configuration.

Observation 1: RAN1 is still discussing the restrictions on the DST ID of the responding UE to utilize the shared COT, it is too early for RAN2 to conclude LCP is impacted from COT sharing.

Proposal 12: RAN2 to discuss to exchange BSR information between initiating UE and responding UE for COT sharing.

Proposal 13: Responding UE performs type 1 LBT if LCP is done before COT information arrives and the generated packet cannot satisfy the shared COT.

Proposal 14: Whether the shared COT should be allowed for other UEs to use or not depends on RAN1.

Observation 2: Initiating UE may not need to keep active during the shared COT duration if responding UE is allowed to transmit to other UEs using the shared COT which is still under discussion in RAN1.

Proposal 15: If the shared COT is utilized for gourpcast/broadcast transmission, all UEs within the group or all UEs interested for this L2 ID should keep active during the shared COT duration.

Proposal 16: When the UE is the responding UE and operates in mode 2, the UE firstly selects resources from resources indicated by the physical layer and within the shared COT upon resource (re-)selection.

Proposal 17: RAN2 to discuss how to support UE to report the shared COT information to the network for mode 1 scheduling.

4	Reference

[1] RAN2#120 meeting minutes.

5 Annex: Standardized PQI defined by TS23.287/TS23.304

Table 5.4.4-1: Standardized PQI to QoS characteristics mapping

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 21 | GBR | 3 | 20 ms | 10-4 | N/A | 2000 ms | Platooning between UEs – Higher degree of automation; Platooning between UE and RSU – Higher degree of automation |
| 22 | (NOTE 1) | 4 | 50 ms | 10-2 | N/A | 2000 ms | Sensor sharing – higher degree of automation |
| 23 |  | 3 | 100 ms | 10-4 | N/A | 2000 ms | Information sharing for automated driving – between UEs or UE and RSU - higher degree of automation |
| 55 | Non-GBR | 3 | 10 ms | 10-4 | N/A | N/A | Cooperative lane change – higher degree of automation |
| 56 |  | 6 | 20 ms | 10-1 | N/A | N/A | Platooning informative exchange – low degree of automation; Platooning – information sharing with RSU |
| 57 |  | 5 | 25 ms | 10-1 | N/A | N/A | Cooperative lane change – lower degree of automation |
| 58 |  | 4 | 100 ms | 10-2 | N/A | N/A | Sensor information sharing – lower degree of automation |
| 59 |  | 6 | 500 ms | 10-1 | N/A | N/A | Platooning – reporting to an RSU |
| 90 | Delay Critical GBR | 3 | 10 ms | 10-4 | 2000 bytes | 2000 ms | Cooperative collision avoidance; Sensor sharing – Higher degree of automation; Video sharing – higher degree of automation |
| 91 | (NOTE 1) | 2 | 3 ms | 10-5 | 2000 bytes | 2000 ms | Emergency trajectory alignment; Sensor sharing – Higher degree of automation |
| NOTE 1: GBR and Delay Critical GBR PQIs can only be used for unicast PC5 communications. |  |  |  |  |  |  |  |

NOTE 1:	For Standardized PQI to QoS characteristics mapping, the table will be extended/updated to support service requirements for other identified V2X services.

NOTE 2:	The PQIs may be used for other services than V2X.

NOTE 3:	A PQI may be used together with an application indicated priority, which overrides the Default Priority Level of the PQI.

Table 5.6.1-1: Standardized PQI values that are additionally defined to QoS characteristics mapping

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | GBR (NOTE 1) | 1 | 150 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 25 |  | 2 | 200 ms | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| 26 |  | 2 | 200 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 60 | Non-GBR | 1 | 120 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| 61 |  | 6 | 400 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9 as specified in TS 23.501 [4]) |
| 92 | Delay Critical GBR (NOTE 1) | 5 | 5ms | 10-4 | 20000 bytes | 2000 ms | Interactive service - consume VR content with high compression rate via tethered VR headset (See TS 22.261 [6]) |
| 93 |  | 6 | 10ms | 10-4 | 20000 bytes | 2000 ms | interactive service - consume VR content with low compression rate via tethered VR headset; Gaming or Interactive Data Exchanging (See TS 22.261 [6]) |
| NOTE 1: GBR and Delay Critical GBR PQIs can only be used for unicast PC5 communications. |  |  |  |  |  |  |  |