3GPP TSG-RAN WG2 #121 meeting	R2- 2300843

Athens Greece, 27th Feb –3rd Mar 2023

Source:		NEC

Agenda item:		8.5.3	XR-specific power saving

Title:		Discussion on C-DRX enhancement for XR

Document for:		Discussion and decision

# 1. Introduction

In RAN#98 meeting, work items [1] related to XR-specific power saving have been agreed as below:

Specify the enhancements related to power saving: - DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2).

In RAN2#120 meeting, agreements [1] on DRX enhancement were made as below:

RAN2 aims to allow XR frame rates that correspond to non-integer periodicities in at least semi-static manner (e.g. RRC). Details can be left to WI phase.

In this contribution, we will provide our views on DRX enhancement for XR.

# 2. Discussion

2.1 Non integer DRX periodicity

For XR traffic, typical video frame rates are {30, 60, 90, 120} frames per second (fps), where the corresponding frame periodicities (1000/fps) are {33.33, 16.67, 11.11, 8.33} ms. Legacy (up to Rel-17) CDRX cycle only supports integer number of milliseconds, that is, long DRX cycle can be configured as a value between 10 ms to 10240 ms, and short DRX cycle can be configured as a value between 2 ms to 640 ms. Therefore, the integer value of the DRX cycle cannot be perfectly aligned with the non-integer periodicity of XR traffic.

To address the issue caused by the mismatch between XR traffic periodicity and DRX cycle, there are several possible solutions:

- Solution1: Introduce non-integer periodicity. There might be different ways to realize it, one way is to add the DRX cycle values which match exactly to the XR traffic periodicity e.g.,50/3ms, consequently it is necessary to change the C-DRX formula where any arbitrary DRX cycle can fit into the formula, the example of the new formula can be found in section 2.2.

- Solution 2: Configure DRX cycle pattern like {17, 17, 16} ms.

- Solution 3: Ask UE automatically to apply an adjustment every certain amount of DRX cycles e.g., 1ms after every 3 DRX cycles.

However, all these solutions are effectively the same, but introducing non-integer periodicity would be the most simple and straightforward solution.

Proposal 1: DRX cycle with non-integer values can be introduced to match the XR traffic periodicity.

We will discuss in another section that L1/L2 DRX (re)configuration solution also can solve this issue to some extent.

2.2 New C-DRX formulas to avoid SFN wraparound issue

Since the range of SFN is 0~1023 and the range of subframe number is 0~9, SFN wraps around every 10,240 ms. If the value of DRX cycle cannot be divisible by 10240 ms, it will lead to misalignment between XR traffic and DRX cycle when SFN wraparound happens. For example, XR traffic with 60 fps has a DRX cycle of 50/3 ms, and there are 6 ms remaining at the end of a hyper frame (assuming drx-StartOffset = 0).

In [3], the following formulas have been proposed to address the non-integer XR periodicity and SFN wraparound issues.

floor [n – [drx-LongCycle × floor (n / drx-LongCycle) ] ] = drx-StartOffset		(Eq1)

where n = [(H-SFN-M × 10240) + (SFN × 10) + subframe number]

floor [n – [drx-ShortCycle × floor (n / drx-ShortCycle) ] ] =

floor [drx-StartOffset – [drx-ShortCycle × floor (drx-StartOffset / drx-ShortCycle) ] ]	(Eq2)

where n = [(H-SFN-M × 10240) + (SFN × 10) + subframe number]

Figure 1 shows an example of DRX cycle pattern for XR traffic with 60 fps. Based on the formulas, the pattern of the DRX cycles is {17, 17, 16} ms, which will realign with XR traffic every 50ms.

Figure 1: Alignment between XR traffic and DRX cycle by the new formulas

For the formulas, we propose to introduce a new parameter H-SFN-M that is maintained by the MAC layer. H-SFN-M indicates hyper frame which increments by one when the SFN wraps around (from SFN 1023 to SFN 0). H-SFN-M won’t wrap around and will keep increasing unless DRX configuration is re-configured or re-initialized. Here, a reference H-SFN-M, i.e., H-SFN-M = 0, will be determined as below.

- If DRX configuration is (re-)configured by RRC signaling,

  - A new parameter drx-timeReferenceSFN ∈ {0,512} is configured by RRC signalling. drx-timeReferenceSFN indicates SFN used for determination of the reference hyper frame with H-SFN-M = 0 in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the DRX configuration.

  - Based on drx-timeReferenceSFN, UE can determine which hyper frame the reference SFN is in, and consider the hyper frame corresponds to H-SFN-M = 0. Figure 2 shows an example for determining the reference hyper frame with H-SFN-M = 0, where drx-timeReferenceSFN (SFNref) is set to 512 in this example.

Figure 2: Determination of hyper frame with H-SFN-M = 0 (drx-timeReferenceSFN = 512)

- Else if DRX configuration is (re-)initialized by DCI or MAC CE, H-SFN-M is set 0 for the first DRX cycle where the DRX configuration was (re-)initialized.

According to the above discussion, to address the non-integer XR periodicity and SFN wraparound issues, the network only needs to configure the non-integer DRX cycle and drx-timeReferenceSFN. Additionally, legacy DRX operation can be well supported by the new C-DRX formulas as well.

Proposal 2: New C-DRX formulas and H-SFN-M can be introduced to address the non-integer XR periodicity and SFN wraparound issues.

We will discuss in another section that L1/L2 DRX (re)configuration solution also can solve this issue to some extent.

2.3 L1/L2 DRX adjustment

With current DRX mechanism, gNB can configure a very precise start of a DRX cycle i.e., the position of on duration. Even though drx-StartOffset is also in ms granularity, but a drx-SlotOffset can be configured in addition to delay the start of the on-duration window for multiple times of 1/32 ms.

However, mismatch of the packet arrival time and on duration window still can happen often and cause extra delay. This issue exists already as of today. RRC can at any time reconfigure DRX parameters when mismatch happens. But this might become problematic for XR type of service which has tight PDB requirement. Same principle as for CG, L1/L2 signaling could be used to configure/adjust some of DRX parameters in addition to RRC signaling, to make the DRX is more adaptive.

Because DRX is pre-configured by gNB via RRC signalling based on the nominal periodicity and packet arrival timing. When the XR traffic arrivals, the frame arrival timing may not exactly fall in on-duration window. gNB lower layer can quickly advance the on duration based on the buffer time of the first data burst, see following figure:

Figure 3: on-duration adjustment via L1/L2 signaling (general)

Another reason of timing mismatch is the SFN wraparound. When SFN wraparound happens, it may lead to a shorter/longer gap between the two on durations, while the arriving time between two packets are still the same. If the packet arrival timing matches the timing of on-duration before SFN wraparound, it will become mismatch after SFN wraparound happens. this is predictable, a L1/L2 signaling can send to adjust the offset value before or immediately after SFN wraparound.

For non-integer periodicity XR traffic configured with an integer periodicity, the data arrival timing will slowly draft away from the on-duration window, as shown in following figure, after several cycles, gNB should adjust the on-duration to compensate the drafting, with L1/L2 signaling.

Figure 4: on-duration adjustment via L1/L2 signaling (non-integer periodicity data case)

Proposal3:  L1/L2 DRX adjustment solution can be considered to make DRX configuration more adaptive to the data arrival timing and to solve the DRX mismatch issue caused by SFN wrapping around and non-integer periodical data flow.

2.4 Multiple DRX configurations

Another requirement from XR on C-DRX is to support multiple XR traffic flows. From TR38.838, especially in dual eye buffer model, the left and right eye frames arrive separately with a time offset but the same periodicity. Moreover in certain two stream model, e.g. video + audio/data, the video and audio streams may have different periodicity (e.g., 16.6 and 10ms), different PDB (e.g., 10ms vs 30ms) and different packet size.

With current C-DRX mechanism, for all serving cells of a MAC entity, they may be configured by RRC in two DRX groups, both DRX group have common short and long DRX cycle, same on-duration window starting point but can have different length of on-duration timer and DRX inactivity timer, these could enable serving cells in one DRX groups switch from on to off earlier.  As we only support one DRX cycle and one on-duration in a cycle. Current mechanism does not solve the multiple flow issue.

To support multiple flow well, different DRX configurations (with different DRX periodicity and offset) should be configured to match the traffic characteristics of different flows.

Proposal 4 multiple DRX configurations (with different DRX periodicity) could be considered to support multiple XR traffic flows.

# Summary

This contribution provided our analysis on candidate solutions of C-DRX enhancement, and we propose to list all these possible solutions and the issues each solution can solve:

Proposal 1: DRX cycle with non-integer values can be introduced to match the XR traffic periodicity.

Proposal 2: New C-DRX formulas and H-SFN-M can be introduced to address the non-integer XR periodicity and SFN wraparound issues.

Proposal3:  L1/L2 DRX adjustment solution can be considered to make DRX configuration more adaptive to the data arrival timing and to solve the DRX mismatch issue caused by SFN wrapping around and non-integer periodical data flow.

Proposal 4 multiple DRX configurations (with different DRX periodicity) could be considered to support multiple XR traffic flows.

# References

[1] RP-223502, New WID on XR Enhancements for NR

[2] Report of 3GPP TSG RAN WG2 meeting #120

[3] R2-2207999, C-DRX enhancements for XR, MediaTek Inc.

[4] TS 38.321, Medium Access Control (MAC) protocol specification, V17.3.0