3GPP TSG-RAN2 Meeting #121	R2-2300845

# Athens, Greece, 27 Feb – 03 Mar, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4903 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.331 on NPUSCH-ConfigDedicated-NB-v1700 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh4_LTE_eMTC6 |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In v17.0.0, NPUSCH-ConfigDedicated-NB-v1700 is introduced, which contains npusch-16QAM-Config-r17 and npusch-TxDurationConfig-r17, and is referenced by PhysicalConfigDedicated-NB. (Cited from Draft_36331-h00_v4.docx, which is the last draft version after RAN 95) NPUSCH-ConfigDedicated-NB-v1700 ::= SEQUENCE { npusch-16QAM-Config-r17 ENUMERATED {true} OPTIONAL, -- Need OR npusch-TxDurationConfig-r17 SetupRelease {NPUSCH-TxDurationConfig-NB-r17} OPTIONAL -- Need ON } PhysicalConfigDedicated-NB-r13 ::= SEQUENCE { <Unrelated parts omitted> ]], [[ npusch-ConfigDedicated-v1700 NPUSCH-ConfigDedicated-NB-v1700 OPTIONAL, -- Need ON npdsch-ConfigDedicated-v1700 SetupRelease {NPDSCH-16QAM-Config-NB-r17} OPTIONAL, -- Need ON uplinkPowerControlDedicated-v1700 UplinkPowerControlDedicated-NB-v1700 OPTIONAL -- Cond npusch-16QAM ]] } Then in RAN2 #118-e, it is proposed during ASN.1 review and agreed by RAN2 to move the npusch-TxDurationConfig-r17 (considering this is an NTN-specific parameter) to a new field ntn-ConfigDedicated-r17 which is located in PhysicalConfigDedicated-NB. As a result, NPUSCH-ConfigDedicated-NB-v1700 is no longer referenced by PhysicalConfigDedicated-NB (in fact, it is not referenced by any IE or message), which also means, npusch-16QAM-Config-r17 is referenced nowhere. (Cited from Draft_36331-h10_v3.docx, which is the last draft version after RAN 96) NPUSCH-ConfigDedicated-NB-v1700 ::= SEQUENCE { npusch-16QAM-Config-r17 ENUMERATED {true} OPTIONAL -- Need OR } PhysicalConfigDedicated-NB-r13 ::= SEQUENCE { <Unrelated parts omitted> [[ ntn-ConfigDedicated-r17 SEQUENCE { npusch-TxDuration-r17 SetupRelease {NPUSCH-TxDuration-NB-r17} } OPTIONAL, -- Cond NTN npdsch-ConfigDedicated-v1700 NPDSCH-ConfigDedicated-NB-v1710 OPTIONAL, -- Need ON uplinkPowerControlDedicated-v1700 UplinkPowerControlDedicated-NB-v1700 OPTIONAL -- Cond npusch-16QAM ]] } From our perspective, npusch-16QAM-Config-r17 should be moved back to PhysicalConfigDedicated-NB (by moving back NPUSCH-ConfigDedicated-NB-v1700) otherwise 16QAM cannot be activated for UL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add NPUSCH-ConfigDedicated-NB-v1700 to PhysicalConfigDedicated-NB. Impact analysis Impacted functionality: 16 QAM for UL Inter-operability issues: None |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 16QAM cannot be activated for UL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

6.7.3.2	NB-IoT Radio resource control information elements

[Unchanged parts omitted]

#### PhysicalConfigDedicated-NB

The IE PhysicalConfigDedicated-NB is used to specify the UE specific physical channel configuration.

PhysicalConfigDedicated-NB information element

-- ASN1START

PhysicalConfigDedicated-NB-r13 ::=	SEQUENCE {

carrierConfigDedicated-r13			CarrierConfigDedicated-NB-r13		OPTIONAL,	-- Need ON

npdcch-ConfigDedicated-r13			NPDCCH-ConfigDedicated-NB-r13		OPTIONAL,	-- Need ON

npusch-ConfigDedicated-r13			NPUSCH-ConfigDedicated-NB-r13		OPTIONAL,	-- Need ON

uplinkPowerControlDedicated-r13		UplinkPowerControlDedicated-NB-r13	OPTIONAL,	-- Need ON

...,

[[	twoHARQ-ProcessesConfig-r14		ENUMERATED {true}	OPTIONAL	-- Need OR

]],

[[	interferenceRandomisationConfig-r14	ENUMERATED {true}	OPTIONAL	-- Need OR

]],

[[	npdcch-ConfigDedicated-v1530	NPDCCH-ConfigDedicated-NB-v1530		OPTIONAL	-- Cond TDD

]],

[[	additionalTxSIB1-Config-v1540	ENUMERATED {true}	OPTIONAL	-- Cond additionalSIB1

]],

[[	npusch-ConfigDedicated-v1610		NPUSCH-ConfigDedicated-NB-v1610

OPTIONAL,	-- Cond twoHARQ

npdsch-ConfigDedicated-r16			NPDSCH-ConfigDedicated-NB-r16

OPTIONAL,	-- Need ON

resourceReservationConfigDL-r16		SetupRelease {ResourceReservationConfig-NB-r16}

OPTIONAL,	-- Cond dl-NonAnchor

resourceReservationConfigUL-r16		SetupRelease {ResourceReservationConfig-NB-r16}

OPTIONAL	-- Cond ul-NonAnchor

]],

[[	ntn-ConfigDedicated-r17				SEQUENCE {

npusch-TxDuration-r17					SetupRelease {NPUSCH-TxDuration-NB-r17}

} OPTIONAL,	-- Cond NTN

npdsch-ConfigDedicated-v1700	NPDSCH-ConfigDedicated-NB-v1710	OPTIONAL,	-- Need ON

uplinkPowerControlDedicated-v1700	UplinkPowerControlDedicated-NB-v1700		OPTIONAL -- Cond npusch-16QAM

]],

[[ npusch-ConfigDedicated-v17xy		NPUSCH-ConfigDedicated-NB-v1700	OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

<End of Change>