3GPP TSG-RAN WG2 Meeting #121                                                        R2-2300863

Athens, Greece, 27th Feb - 3rd Mar, 2023

Source:              CATT

Title:	            Issues on L2 ID(s) for D2I path switching of L2 U2N relay

Agenda Item:	6.5.2

Document for:	Discussion and Decision

Introduction

In this contribution, the analysis on the association between the L2 IDs used for discovery and communication of L2 U2N Relay will be discussed.

Discussion

Issue description

According to [1], the procedure of the D2I path switching for L2 U2N Relay scenario is listed below:

Figure-1 Procedure for L2 U2N Remote UE switching to indirect path via a L2 U2N Relay UE in RRC_CONNECTED

In step-1, Remote UE reports one or multiple candidate L2 U2N Relay UE(s) and Uu measurements. The candidate L2 U2N Relay UE(s) are detected through discovery procedure.

In step-2, gNB decided the target Relay UE, and then, the gNB sends an RRCReconfiguration message to the target L2 U2N Relay UE, which includes at least the L2 U2N Remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration. Considering PC5 unicast connection is triggered after step 3 and the L2 ID used for sidelink communication will be sent by upper layer upon PC5 unicast connection is triggered, hence, in our understanding, the L2 ID of the Remote UE contained in the RRC reconfiguration message of step 2 should be the sidelink discovery L2 ID.

Proposal 1: Suggest RAN2 to confirm that for D2I path switching, the L2 ID of the Remote UE contained in the RRC reconfiguration message in step 2 should be the sidelink discovery L2 ID.

In step-4, the Remote UE will initiate the PC5 unicast connection setup procedure. According to the following description in [2], it is obvious that Remote UE will send DCR to Relay UE using a new sidelink communication L2 ID which is different from the sidelink discovery L2 ID used in sidelink relay discovery.

- TS24.554 - 8.2.11 UE-to-network relay unicast direct communication over PC5 interface The direct communication between 5G ProSe remote UE and 5G ProSe UE-to-network relay UE reuses the procedures for unicast mode 5G ProSe direct communication over PC5 described in clause 7.2. NOTE: Any modifications needed to the procedures defined for unicast mode 5G ProSe direct communication over PC5 to support the direct communication between 5G ProSe remote UE and 5G ProSe UE-to-network relay UE are described within clause 7.2.

- TS24.554 - 7.2 Unicast mode 5G ProSe direct communication over PC5 …… - 7.2.2.2 5G ProSe direct link establishment procedure initiation by initiating UE The initiating UE shall meet the following pre-conditions before initiating this procedure: a) a request from upper layers to transmit the packet for ProSe application over PC5 or a request from lower layers to trigger ProSe direct link establishment; b) the communication mode is unicast mode (e.g., pre-configured as specified in clause 5.2.4 or indicated by upper layers); c) the link layer identifier for the initiating UE (i.e., layer-2 ID used for unicast communication) is available (e.g., pre-configured or self-assigned) and is not being used by other existing 5G ProSe direct links within the initiating UE;

Proposal 2: Suggest RAN2 to confirm that for D2I path switching, the L2 ID of Remote UE used in step 4 is a sidelink communication L2 ID, and it is different from the L2 ID of Remote UE contained in the RRC reconfiguration message of step 2.

If both Proposal 1 and Proposal 2 are confirmed, one issue raised. In step 2, the Relay UE can acquire the association between the sidelink discovery L2 ID of the Remote UE and the related SRAP configuration. But in step 4, only the sidelink communication L2 ID of the Remote UE is received by Relay UE. It cannot link the sidelink communication L2 ID and the SRAP configuration which is configured in step 2 based on the sidelink discovery L2 ID of the Remote UE.

Proposal 3: Suggest RAN2 to confirm that Relay UE cannot determine the mapping between the sidelink communication L2 ID and the corresponding SRAP configuration since the SRAP configuration sent by gNB in step 2 is based on the sidelink discovery L2 ID.

Solutions

There are two possible solutions:

- Option 1: Remote UE sends the linkage between the sidelink discovery L2 ID and the sidelink communication L2 ID to Relay UE using PC5-RRC signaling after PC5-S connection has been setup in step 4.

- Option 2: Relay UE can aware the linkage between the Remote UE’s sidelink discovery L2 ID and sidelink communication L2 ID through upper layer procedure.

Option 1 needs NBC CR. Whether Option 2 is feasible should be checked with SA2 and CT1. Considering in this stage, NBC CR should be avoided as much as possible. Hence, Option 2 is slightly preferred.

Proposal 4: Send LS to SA2/CT1 to check:

- Q1: Whether the upper layer of Relay UE can aware the linkage between the Remote UE’s sidelink discovery L2 ID and the Remote UE’s sidelink communication L2 ID?

- Q2: If the answer of Q1 is Yes, whether and when the upper layer can inform this linkage to AS layer?

Conclusion

According to the analysis in section 2, it is observed that:

Proposal 1: Suggest RAN2 to confirm that for D2I path switching, the L2 ID of the Remote UE contained in the RRC reconfiguration message in step 2 should be the sidelink discovery L2 ID.

Proposal 2: Suggest RAN2 to confirm that for D2I path switching, the L2 ID of Remote UE used in step 4 is a sidelink communication L2 ID, and it is different from the L2 ID of Remote UE contained in the RRC reconfiguration message of step 2.

Proposal 3: Suggest RAN2 to confirm that Relay UE cannot determine the mapping between the sidelink communication L2 ID and the corresponding SRAP configuration since the SRAP configuration sent by gNB in step 2 is based on the sidelink discovery L2 ID.

Proposal 4: Send LS to SA2/CT1 to check:

- Q1: Whether the upper layer of Relay UE can aware the linkage between the Remote UE’s sidelink discovery L2 ID and the Remote UE’s sidelink communication L2 ID?

- Q2: If the answer of Q1 is Yes, whether and when the upper layer can inform this linkage to AS layer?

Based on Proposal 4, the corresponding LS out is in [3].

Reference

- TS 38.300  NR; NR and NG-RAN Overall Description; Stage 2 (Release 17)

- TS 24.554  Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3 (Release 17)

- R2-2300864  LS on L2 ID issue for D2I path switching of U2N Relay  CATT