3GPP TSG-RAN WG2 Meeting #121	R2-2300871

Athens, Greece, 27 February – 03 March 2023

Agenda item:	8.3.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Reselection and Access control for NES

WID/SID:	Netw_Energy_NR-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN2 is starting WI on NW energy saving (RP-223540) with one of the goals:

- Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2]

The TR 38.864 v18.0.0 captures this on the objective:

### 6.5.1	Cell selection/reselection

For backward compatibility, there is a need to allow NES cells to prevent legacy UEs from camping. NES cells should be able to configure whether to prevent legacy UEs, while allowing NES-capable UEs to camp on. Possible solutions may include but not limited to:

-	Use IntraFreqExcludedCellList/InterFreqExcludedCellList

-	Use the cellBarred or cell reservation fields in MIB/SIB

The definition of NES cell will be discussed in the WI phase.

The NW should be able to configure NES-capable UEs to prioritize/down-prioritize a specific NES cell or NES cells on a specific frequency. It is left to the WI phase whether the existing mechanism for cell (re)selection is sufficient according to the NES techniques specified.

From RAN2 perspective, legacy UEs and NES-capable UEs can be handled via cell selection/reselection techniques in the presence of NES cells.

In this paper we discuss the objective.

# 2 	Discussion

Whenever the gNB (or one of the network entities it controls, such as a cell, a DU, a CU) operates in a network energy saving (NES) mode such as a dormant state, this entails that some of its hardware components are switched off or kept in a sleep mode to obtain network energy reduction. As a result, the capability of the gNB to provide services to the end users may be temporarily reduced until the energy saving mode is exited. Accordingly, there can be a trade-off between the energy to be saved and performance to be provided. For example, the provided performance during the NES mode (e.g.  in terms of bit rate, latency, number of served UEs, amount of data served, etc.) can be limited or degraded, and data transfer or initial access can be inhibited altogether for some UEs -  especially legacy UEs. To enable increased performance or revert to the normal performance level, the network may want to exit the NES mode, however this comes at the price of an increased energy consumption.

It also seems that the NES mode of gNB/network could change frequently (e.g. with PHY/MAC signalling) without upper layer signaling (e.g. RRC signalling). This may cause issues how the upper layer signaling can control UEs – especially UEs in IDLE/INACTIVE.

Network could choose to control for example that some UEs will not camp at all on NES cells or then they could allow camping but limit the accesses to the system while operating in NES mode.

## 2.1 	Mobility

When a cell is applying NES mode with some of the solutions under discussion, e.g. SIB-less or adaptive SSB/SIB etc, the legacy UEs might not be able to camp on/access the NES cell, while the UEs supporting NES can. Additionally it may be desired by NW that even if legacy UEs could camp they would prefer camping on cells not utilizing NES functionalities.

Quite often it happens that whole frequency applies NES method or even is turned off completely.It seems also very likely that number of frequency layers applying NES will change in time. As well as number of cells applying NES will change in time in a one specific frequency layer. If we consider tools for  handling reselection priorities currently found in the specs:

1.	Priorities in SIB – these are common for all UEs (NES, non-NES UEs.)

2.	Dedicated priorities provided in RRCRelease.

Without doing anything in the specification dedicated priorities can handle NES scenario but if NES frequency layers are changing frequently then moving UEs back to connected to change the priorities may not be convenient but on the other hand target of the WI is the low load scenario so maybe that is not really showstopper. We could consider having some NES specific reselection priorities in SIBs as well and changing those SIB reselection priorities would require SI update procedure could be problematic if needed to be done frequently. We just need to remember that this is not really in the WI objectives where the focus is to prevent legacy UE camping on cells where it cannot operate. Thus it might be challenging to do anything here in the WI due to plenary decisions and having some tools already available.

But it seems quite reasonable to assume that NES does not need to be utilized on whole frequency but just on subset of cells – e.g. in locations with more UE density it might be better to keep cells on a frequency not using NES. Thus we think RAN2 should be considering solutions for mobility that handle both intra- and inter-frequency mobility.

In the RRC_IDLE/INACTIVE mobility is controlled by reselections and mostly with broadcast signaling although it is possible to provide dedicated priorities. As the reselection priorities as such only target whole frequency they are not suitable at least as a lone solution for mixed deployment frequencies (both NES and non-NES cells on same frequency).

Observation 1: RAN2 should consider in reselection handling scenarios where whole frequency is utilizing NES (e.g. SCell frequency turned off) as well as only subset of cells on a frequency utilize NES

As we can expect that there will be UEs in the system in future which understand NES deployments and UEs which do not (legacy UE) - any solutions should be able to provide mechanisms that will handle both type of UEs. Thus it seems necessary that solutions (if any) introduced for NES reselection control should be able control legacy UE camping on a NES utilizing cells.

Observation 2: RAN2 should have a mechanism where legacy UEs could be control camping on cells/frequencies utilizing NES

So basically at least one would need to be able to control also cell selection and camping on NES cell for legacy UEs (and still allow NES UEs to camp on those). This can be easily achieved by using NES specific cell barring i.e. legacy UEs follow legacy barring but NES capable UEs would still need to check whether NES specific barring is present or not before making decision that cell is barred for NES capable UE:

Proposal 1: To be able to only prevent legacy capable UE to camp (e.g. via cell selection) on a cell we propose to introduce NES specific cell barring mechanism

Cell reselection priorities control camping on frequencies in NR and for intra-frequency mobility UEs are controlled via offsets as well as allowed/excluded cell lists. It seems easy to limit legacy UEs to camp on NES cells by listing those cells in the excluded cell lists.

Observation 3: NW could handle legacy UEs not to reselect to NES applying cells by utilizing Intra/InterFreqExcludedCellList

However the UEs supporting NES should still be able to reselect to those NES cells if intended by the NW, even if they are in the excluded cell list to block the legacy UEs. The NW could indicate those NES cells so that the NES capable UEs would consider them as allowed.

If one agrees to be able to derive different exclude cell list for NES capable UEs (compared to legacy UEs) then those cells won’t be used by UEs for cell reselection as defined by 38.304:

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

Thus cell reselection can be controlled effectively with exclude-listed cells to avoid legacy and allow NES UEs to reselect to NES cells. As the signaling of exclude-listed cells is done for both intra and inter-frequency cells it would be possible to steer UE reselection via that handling both intra and inter-frequency cases.

Observation 4: exclude-listed cells can control UE reselection to both intra- and inter-frequency cells

One could also consider using some NES specific cell reselection priorities but that won’t be able to handle intra-frequency scenarios at all

Observation 5: NES specific cell reselection priorities cannot handle intra-frequency scenarios

In RAN2#119bis it was agreed:

The network should be able to configure NES capable UEs to (de)prioritize NES cells.  mechanism such as can be considered for both frequency and cell levels cell selection/reselection (de)prioritization.  FFS on whether the existing mechanism is sufficient

As was discussed above purely exclude-listed cells can handle also this agreement without requiring additional mechanism as the exclude cell list can also indicate whole list as excluded

Observation 6: With exclude-cell list one can prevent UE utilizing all the PCIs of a frequency for reselection

This leads to us to a proposal to start exclude-cell list as baseline for reselection mechanism to prevent legacy UEs to reselect to NES cells as well as allow NES capable UE to reselect to those.

Proposal 2: To handle reselection steering to/out of NES cells: the NES capable UEs could derive different excluded list than the legacy UE does based on Intra/InterFreqExcludedCellList.

If the NW intends to use NES mode for a cell, to avoid other UEs to reselect to the cell, IntraFreqExcludedCellList/InterFreqExcludedCellList can be used. While to move the UEs already camped on the cell to reselect to another cell, the NW needs to indicate the cell as barred. As the cellBarred in MIB is used to bar legacy UEs, other mechanism needs to be introduced if the NW wants to bar also NES UEs in case it intends to shut down the cell completely.

Using a NES specific new cellBarred  in SIB1 has been proposed in the discussions. But it requires SIB update procedure which requires sending paging indicating SI update notification for a modification period then sending updated SIB for the next modification period, which is not very power efficient from NW energy saving point of view just to bar the cell for the NES UEs. It introduces delay for the NW to turn off the cell as well.

In order to speed up the cell reselection we propose to indicate e.g. in short message DCI (addressed to P-RNTI) to initiate cell reselection for all the UEs camping on the cell, after which the NW can ensure no UE camps on the cell and can turn off the cell without waiting the SI update procedure to complete.

For UEs camping on the cell, upon reception of the short message indicating NW initiated cell reselection, the UE considers the cell as barred and performs cell re-selection procedure.

Proposal 3: Introduce an indication not requiring SI update procedure to indicate the cell is being shut down to trigger NES capable UE to bar the cell and perform cell reselection

## 2.2 	Access control

As we can also have mixed deployments then it becomes more complex to control UEs (NES capable or legacy UE) to camp on some cells on frequency. Especially NES capable UEs should be allowed to camp on NES cells but this could be problematic if large of amount of NES capable cells arrive to the NES utilizing cell simultaneously. Network would need to have control on how those UEs access the cell.

Currently, the network can configure limitations to the initial access using the UAC framework. The UAC is however defined to mitigate network congestion. It seems quite likely that when network is operating in NES mode it cannot handle efficiently many UEs and thus it becomes critical that network can effectively prevent new access in the cell. The UAC information is transmitted in system information (SI), and thus requires SI modification for any changes to be made to the access barring. This implies that currently the network cannot adjust the barring information dynamically, and requires SI re-acquisition. And as the NES mode of gNB/network could change frequently (e.g. with PHY/MAC signalling) without upper layer signaling (e.g. RRC signalling), a slow update of access barring or cell barring can become difficult to manage.

Proposal 4: RAN2 should study how the network can control accesses in the NES utilizing gNB effectively.

# 3	Conclusion

Observation 1: RAN2 should consider in reselection handling scenarios where whole frequency is utilizing NES (e.g. SCell frequency turned off) as well as only subset of cells on a frequency utilize NES

Observation 2: RAN2 should have a mechanism where legacy UEs could be control camping on cells/frequencies utilizing NES

Proposal 1: To control cell camping on NES cells: Agree to introduce NES specific cell barring mechanism

Observation 3: NW could handle legacy UEs not to reselect to NES applying cells by utilizing Intra/InterFreqExcludedCellList

Observation 4: exclude-listed cells can control UE reselection to both intra- and inter-frequency cells

Observation 5: NES specific cell reselection priorities cannot handle intra-frequency scenarios

Observation 6: With exclude-cell list one can prevent UE utilizing all the PCIs of a frequency for reselection

Proposal 2: As a baseline to handle reselection steering to/out of NES cells: the NES capable UEs could derive different excluded list than the legacy UE does based on Intra/InterFreqExcludedCellList.

Proposal 3: Introduce a indication not requiring SI update procedure to indicate the cell is being shut down to trigger UE to bar the cell and perform cell reselection

Proposal 4: RAN2 should study how the network can control accesses in the NES utilizing gNB effectively.